TSG-CT WG3 Meeting #110-e	C3-203514

E-Meeting, 2nd – 11th June 2020													(Revision of C3-203387)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.486 | CR | 0010 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supported headers, Resource Data type and yaml mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2020-05-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - New tables are added to show the mapping between the APIs defined in the specification and the Yaml, according to the SBI template - New tables are added to capture supported headers for response codes 2xx and 3xx as coded in the yaml file, according to the SBI template - Capture the Data type for each Resource listed in the table allowing for a more optimized logic for the parser, according to the SBI template - Cardinality of “joinedUeIds” and “leftUeIds” need to be corrected. - Resource URI of clause 6.3.3.2.2 need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the specification as above proposals. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Quality of specification will not be improved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 6.1.3.2.2; 6.1.3.2.3.1; 6.1.3.3.2; 6.1.3.4.2; 6.1.3.4.3.1; 6.1.3.5.2; 6.2.3.2.2; 6.2.3.2.3.1; 6.2.3.3.2; 6.3.3.2.2; 6.3.3.2.3.1; 6.3.3.3.2; 6.4.3.2.2; 6.4.3.2.3.1; 6.4.3.3.2; 6.4.6.2.3; 6.5.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

…

# Proposed changes:

# *** 1st Change ***

5.1	Introduction

The table 5.1-1 shows the services provided by the VAE server and corresponding Service Operations:

Table 5.1-1 List of services provided by the VAE Server

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| VAE_MessageDelivery | Deliver_DL_Message | Request/Response | V2X application specific server |
|  | Deliver_UL_Message | Subscribe/Notify | V2X application specific server |
|  | V2X_MessageDelivery_Subscribe |  | V2X application specific server |
|  | V2X_MessageDelivery_Unsubscribe |  | V2X application specific server |
| VAE_FileDistribution | Distribute_File | Request/ Response | V2X application specific server |
| VAE_ ApplicationRequirement | Reserve_NetworkResource | Subscribe/Notify | V2X application specific server |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup | Configure_DynamicGroup | Subscribe/Notify | V2X application specific server |
| VAE_ServiceContinuity | Query_ServiceContinuity | Request/Response | VAE server |
| NOTE: A subscription applies for one UE, group of UE(s) or any UE. |  |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| VAE_MessageDelivery | 6.1 | VAE Message Delivery Service | TS29486_VAE_MessageDelivery.yaml | vae-message-delivery | A.2 |
| VAE_FileDistribution | 6.2 | VAE File Distribution Service | TS29486_VAE_FileDistribution.yaml | vae-file-distribution | A.3 |
| VAE_ ApplicationRequirement | 6.3 | VAE Application Requirement Provision Service | TS29486_VAE_ApplicationRequirement.yaml | vae-app-req | A.4 |
| VAE_DynamicGroup | 6.4 | VAE Configure Dynamic Group Information Service | TS29486_VAE_DynamicGroup.yaml | vae-dynamic-group | A.5 |
| VAE_ServiceContinuity | 6.5 | VAE Service Continuity Service | TS29486_VAE_ServiceContinuity.yaml | vae-service-continuity | A.6 |

# *** Next Change ***

6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/vae-message-delivery/{apiVersion}/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |

# *** Next Change ***

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MessageDeliverySubscriptionData | M | 1 | Parameters to create an Individual Message Delivery Subscription resources. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliverySubscriptionData | O | 0..1 | 201 Created | An Individual Message Delivery Subscription resource for the V2X UE ID or V2X group ID is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/vae-message-delivery/<apiVersion>/subscriptions/{subscriptionId} |

# *** Next Change ***

6.1.3.3.2	Resource definition

Resource URI: {apiRoot}/vae-message-delivery/{apiVersion}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| subscriptionId | string | Unique identifier of the individual Message Delivery Subscription resource for the V2X UE ID or V2X group ID. |

# *** Next Change ***

6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/vae-message-delivery/{apiVersion}/subscriptions/{subscriptionId}/message-deliveries

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| subscriptionId | string | Unique identifier of the individual Message Delivery Subscription resource for the V2X UE ID or V2X group ID. |

# *** Next Change ***

6.1.3.4.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DownlinkMessageDeliveryData | M | 1 | Parameters to create an individual Message Delivery resources. |

Table 6.1.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DownlinkMessageDeliveryData | O | 0..1 | 201 Created | An individual Message Delivery resource for the V2X UE ID or V2X group ID is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [2] shall also apply. |  |  |  |  |

Table 6.1.3.4.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/vae-message-delivery/<apiVersion>/subscriptions/{subscriptionId}/message-deliveries/{dlDeliveryId} |

# *** Next Change ***

6.1.3.5.2	Resource definition

Resource URI: {apiRoot}/vae-message-delivery/{apiVersion}/subscriptions/{subscriptionId}/message-deliveries/{dlDeliveryId}

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| subscriptionId | string | Unique identifier of the individual Message Delivery Subscription resource for the V2X UE ID or V2X group ID. |
| dlDeliveryId | string | Unique identifier of the Individual Downlink Message Delivery resource for the V2X UE ID or V2X group ID. |

# *** Next Change ***

6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/vae-file-distribution/v1/file-distributions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |

# *** Next Change ***

6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FileDistributionData | M | 1 | Parameters to create an individual File Distribution resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FileDistributionData | O | 0..1 | 201 Created | An individual File Distribution resource for the V2X group ID is created successfully. |
| NOTE : The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [2] shall also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/vae-file-distribution/<apiVersion>/file-distributions/{distributionId} |

# *** Next Change ***

6.2.3.3.2	Resource definition

Resource URI: {apiRoot}/vae-file-distribution/v1/file-distributions/{distributionId}

This resource shall support the resource URI variables defined in table 6.2.3.3-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1 |
| apiVersion | string | See clause 6.2.1 |
| distributionId | string | Unique identifier of the individual File Distribution resource for the V2X group ID. |

# *** Next Change ***

6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/ vae-app-req/{apiVersion}/application-requirements

This resource shall support the resource URI variables defined in table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| apiVersion |  | See clause 6.1.1 |

# *** Next Change ***

6.3.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ApplicationRequirementData | M | 1 | Parameters to create an individual Application Requirement resource. |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ApplicationRequirementData | O | 0..1 | 201 Created | An individual Application Requirement resource for the V2X UE ID or the V2X group ID is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [2] shall also apply. |  |  |  |  |

Table 6.3.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/vae-app-req/<apiVersion>/application-requirements/{requirementId} |

# *** Next Change ***

6.3.3.3.2	Resource definition

Resource URI: {apiRoot}/vae-app-req/{apiVersion}/application-requirements/{requirementId}

This resource shall support the resource URI variables defined in table 6.2.3.3-1.

Table 6.3.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| requirementId | string | Unique identifier of the individual Application Requirement resource for the V2X UE ID or the V2X group ID. |

# *** Next Change ***

6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/vae-dynamic-group/{apiVersion}/group-configurations

This resource shall support the resource URI variables defined in table 6.4.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |

# *** Next Change ***

6.4.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GroupConfigurationData | M | 1 | Parameters to create an individual Group Configuration resource. |

Table 6.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GroupConfigurationData | O | 0..1 | 201 Created | An individual Group Configuration resource for the V2X group ID is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [2] shall also apply. |  |  |  |  |

Table 6.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/vae-dynamic-group/<apiVersion>/group-configurations/{configId} |

# *** Next Change ***

6.4.3.3.2	Resource definition

Resource URI: {apiRoot}/vae-dynamic-group/{apiVersion}/group-configurations /{configId}

This resource shall support the resource URI variables defined in table 6.2.3.3-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| apiVersion | string | See clause 6.1.1 |
| configId | string | Unique identifier of the individual group configuration resource for the V2X group ID. |

# *** Next Change ***

6.4.6.2.3	Type: DynamicGroupNotification

Table 6.4.6.2.2-1: Definition of type DynamicGroupNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceUri | Uri | M | 1 | The resource URI of the individual Group Configuration related to the notification. |  |
| joinedUeIds | array(V2xUeId) | O | 1..N | The joined group member(s). |  |
| leftUeIds | array(V2xUeId) | O | 1..N | The left group member(s). |  |

# *** Next Change ***

6.5.3.2.2	Resource Definition

Resource URI: {apiRoot}/vae-service-continuity/{apiVersion}/geo-areas/{geoId}

This resource shall support the resource URI variables defined in table 6.5.3.2.2-1.

Table 6.5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5.1 |
| apiVersion | string | See clause 6.5.1 |
| geoId | string | Geographical area id. |

*** End of Changes ***