3GPP TSG-CT3 Meeting #125	C3-225108

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.519 | CR | 0362 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Immediate reporting for policy data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc18 |  |  |  |  |  | Date: |  |  | 2022-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UDR API for Policy Data does not support immediate reporting, although the reduction of the number of required HTTP requests can enhance overall system performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the immediate reporting option to subscriptions for UDR Policy Data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Suboptimal protocol performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.10, 5.4.2.11, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature to the OpenAPI file of the Nudr_DataRepository API for Policy Data. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.2.10	Type PolicyDataSubscription

Table 5.4.2.10-1: Definition of type PolicyDataSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | URI provided by the NF service consumer indicating where to receive the subscribed notifications from the UDR. |  |
| notifId | string | C | 0..1 | Notification Correlation ID assigned by the NF service consumer. It shall be included when the "ConditionalSubscriptionwithPartialNotification" or the "ConditionalSubscriptionWithExcludeNotification" feature is supported. | ConditionalSubscriptionwithPartialNotification ConditionalSubscriptionWithExcludeNotification |
| monitoredResourceUris | array(Uri) | M | 1..N | A set of URIs that identify the resources as defined in table 5.2.2-1 for which a modification triggers a notification. (NOTE 1) |  |
| monResItems | array(ResourceItem) | O | 1..N | When present, this IE indicates the trigger of the notification is conditioned to the data change occurs in one or more of the attributes contained in a resource fragment as defined by the ResourceItem data type, and that the triggered notification shall only include the resource fragment as defined by the ResourceItem data type. (NOTE 2) | ConditionalSubscriptionwithPartialNotification |
| excludedResItems | array(ResourceItem) | O | 1..N | When present, this IE indicates the trigger of the notification is conditioned to the data change occurs in any attribute contained in the resource(s) identified by "monitoredResourceUris", except when the changed data is in one or more of the attributes contained in a resource fragment as defined by the ResourceItem data type. (NOTE 2) (NOTE 3) (NOTE 4) | ConditionalSubscriptionWithExcludeNotification |
| immRep | boolean | O | 0..1 | If provided and set to "true", it indicates that existing entries that match this subscription shall be immediately reported within the "immReports" attribute in the response. The default value is false. The entries provided in the immediate report consider the "monitoredResourceUris" attribute and ignore any conditions potentially provided in the "monResItems" and "excludedResItems" attributes. | ImmediateReportPcc |
| immReports | array(PolicyDataChangeNotification) | O | 1..N | Contains entries stored in the UDR that match this subscription. It may be included only in the POST response body of a subscription creation or modification, and only if the request included the "immRep" attribute set to true. | ImmediateReportPcc |
| expiry | DateTime | C | 0..1 | This IE shall be included in a subscription response if, based on operator policy and taking into account the expiry time included in the request, the UDR needs to include an expiry time. This IE may be included in a subscription request. When present, this IE shall represent the time after which the subscription becomes invalid. The absence of this attribute in the subscription response means the subscription to be valid without an expiry time. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of the optional features. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| resetIds | array(string) | O | 1..N | This IE uniquely identifies a part of temporary data in UDR that contains the created resource. This attribute may be provided in the response of successful resource creation. |  |
| NOTE 1: Neither the resource URI of the IndividualPolicyDataSubscription resource nor the resource URI of the PolicyDataSubscriptions resource shall be included in the "monitoredResourceUris" attribute. NOTE 2: If both ConditionalSubscriptionwithPartialNotification and ConditionalSubscriptionWithExcludeNotification are supported, the fragments defined in "monResItems" attribute and in "excludedResItems" attribute shall refer to different resources defined in "monitoredResourceUris". NOTE 3: When "excludedResItems" attribute is included, and a change is detected in the monitored part of a resource (i.e., in those properties not included in the "excludedResItems" attribute), the triggered notification shall include the complete resource representation. NOTE 4: The "excludedResItems" attribute may only be used with the OperatorSpecificData resource. E.g. if the OperatorSpecificData resource contains a map with two elements, one with key "operatorSpecificData1" and the other one with key "operatorSpecificData2", to excude the notification of changes in the "operatorSpecificData1" element the "excludedResItems" attribute will contain the value "/operatorSpecificData1" within the "items" array. |  |  |  |  |  |

# * * * * Next change * * * *

#### 5.4.2.11	Type PolicyDataChangeNotification

Table 5.4.2.11-1: Definition of type PolicyDataChangeNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| amPolicyData | AmPolicyData | O | 0..1 | Access and Mobility Policy Data, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| uePolicySet | UePolicySet | O | 0..1 | UE Policy Set, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| plmnUePolicySet | UePolicySet | O | 0..1 | PLMN UE Policy Set, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| smPolicyData | SmPolicyData | O | 0..1 | Session Management Policy Data, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| usageMonData | UsageMonData | O | 0..1 | Usage Monitoring Data, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| sponsorConnectivityData | SponsorConnectivityData | O | 0..1 | Sponsor data connectivity profile information, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| bdtData | BdtData | O | 0..1 | Background Data Transfer Data, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| opSpecData | OperatorSpecificDataContainer | O | 0..1 | Operator Specific Data, if changed and notification was requested, or if it existed and immediate reporting was requested. It may only be used when the receiver of the notification is able to univocally identify the changed operator specific data. (NOTE 3) |  |
| opSpecDataMap | map(OperatorSpecificDataContainer) | O | 1..N | Operator Specific Data resource data, if changed and notification was requested, or if it existed and immediate reporting was requested. The key of the map is operator specific data element name and the value is the operator specific data of the UE. | OpSpecDataMapNotification |
| ueId | VarUeId | C | 0..1 | Represents the UE subscription identifier SUPI or GPSI. It shall only be present when the "amPolicyData", "uePolicySet", "smPolicyData", "opSpecData" and/or "usageMonData" attribute is present. |  |
| sponsorId | string | C | 0..1 | Represents the sponsor identity. It shall only be present when the "sponsorConnectivityData" attribute is present. |  |
| bdtRefId | BdtReferenceId | C | 0..1 | Represents the BDT reference identifier. It shall only be present when the "bdtData" attribute is present. |  |
| usageMonId | string | C | 0..1 | Represents the unique identifier of the individual SM Policy usage monitoring resource. It shall only be present when the "usageMonData" attribute is present. |  |
| plmnId | PlmnId | C | 0..1 | Represents the PLMN identifier. It shall only be present when the "plmnUePolicySet" attribute is present. |  |
| delResources | array(Uri) | O | 1..N | The resources, as defined in table 5.2.2-1, if removed from UDR and notification on resource data change was requested. Not applicable for immediate reports. (NOTE 2) | ResourceRemovalNotificationPolicyData |
| notifId | string | C | 0..1 | Notification Correlation ID assigned by the NF service consumer. It shall be included in notifications when the "ConditionalSubscriptionwithPartialNotification" feature is supported. Not applicable for immediate reports. | ConditionalSubscriptionwithPartialNotification |
| reportedFragments | array(NotificationItem) | O | 1..N | This attribute contains the resource fragments indicated in the "monResItems" attribute of the PolicyDataSubscription data type. Not applicable for immediate reports. | ConditionalSubscriptionwithPartialNotification |
| slicePolicyData | SlicePolicyData | O | 0..1 | Network slice specific policy control data for an S-NSSAI, if changed and notification was requested, or if it existed and immediate reporting was requested. |  |
| snssai | Snssai | C | 0..1 | Represents the S-NSSAI identifier, the unique identifier of the SlicePolicyControlData resource. It shall only be present when the "slicePolicyData" attribute is present. |  |
| NOTE 1: When the "ResourceRemovalNotificationPolicyData" feature and/or "ConditionalSubscriptionwithPartialNotification" feature is not supported or supported but the applicable attribute(s) are not provided, at least one of the "amPolicyData", "uePolicySet", "smPolicyData", "usageMonData", "sponsorConnectivityData", "bdtData", "opSpecData", "plmnUePolicySet" or "slicePolicyData" shall be present. NOTE 2: When the "ResourceRemovalNotificationPolicyData" feature is supported, and the "delResources" attribute is present, the attributes that correspond with the values included in the "delResources" attribute shall be omitted. E.g., when the "delResources" attribute includes the value "../policy-data/ues/{ueId}/am-data" the attribute "amPolicyData" shall be omitted. NOTE 3: This attribute should not be used if the receiver of the notification is not able to univocally identify the changed operator specific data, and "opSpecDataMap" should be used instead. |  |  |  |  |  |

# * * * * Next change * * * *

# A.2	Nudr_DataRepository API for Policy Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29519_Policy_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: Unified Data Repository Service API file for policy data

description: |

The API version is defined in 3GPP TS 29.504

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.519 V17.8.0; 5G System; Usage of the Unified Data Repository Service for Policy Data,

Application Data and Structured Data for Exposure.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.519/'

paths:

/policy-data/ues/{ueId}:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieve the policy data for a subscriber

operationId: ReadPolicyData

tags:

- PolicyDataForIndividualUe (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:read

parameters:

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: data-subset-names

in: query

style: form

explode: false

description: List of policy data subset names

required: false

schema:

type: array

items:

$ref: '#/components/schemas/PolicyDataSubset'

minItems: 2

responses:

'200':

description: Upon success, a response body containing policy data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataForIndividualUe'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/am-data:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieves the access and mobility policy data for a subscriber

operationId: ReadAccessAndMobilityPolicyData

tags:

- AccessAndMobilityPolicyData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:am-data:read

responses:

'200':

description: >

Upon success, a response body containing access and mobility policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/ue-policy-set:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieves the UE policy set data for a subscriber

operationId: ReadUEPolicySet

tags:

- UEPolicySet (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:ue-policy-set:read

parameters:

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Upon success, a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create or modify the UE policy set data for a subscriber

operationId: CreateOrReplaceUEPolicySet

tags:

- UEPolicySet (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:ue-policy-set:create

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

responses:

'201':

description: >

Successful case. The resource has been successfully created and a response body

containing a representation of the created UEPolicySet resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'200':

description: >

Successful case. The resource has been successfully created and a response body

containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'204':

description: >

Successful case. The resource has been successfully updated and no additional content

is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the UE policy set data for a subscriber

operationId: UpdateUEPolicySet

tags:

- UEPolicySet (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:ue-policy-set:modify

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/UePolicySetPatch'

responses:

'204':

description: >

Successful case. The resource has been successfully updated and no additional content is

to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/sm-data:

get:

summary: Retrieves the session management policy data for a subscriber

operationId: ReadSessionManagementPolicyData

tags:

- SessionManagementPolicyData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:sm-data:read

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: snssai

in: query

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: dnn

in: query

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Upon success, a response body containing SmPolicyData shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the session management policy data for a subscriber

operationId: UpdateSessionManagementPolicyData

tags:

- SessionManagementPolicyData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:sm-data:modify

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SmPolicyDataPatch'

responses:

'204':

description: >

Successful case. The resource has been successfully updated and no

additional content is to be sent in the response message.

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/sm-data/{usageMonId}:

get:

summary: Retrieve a usage monitoring resource

operationId: ReadUsageMonitoringInformation

tags:

- UsageMonitoringInformation (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:sm-data:read

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Successful case. The usage monitoring data is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

'204':

description: The resource was found but no usage monitoring data is available.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create a usage monitoring resource

operationId: CreateUsageMonitoringResource

tags:

- UsageMonitoringInformation (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:sm-data:create

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

responses:

'201':

description: >

Successful case. The resource has been successfully created and a response body is

returned containing a representation of the resource.

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a usage monitoring resource

operationId: DeleteUsageMonitoringInformation

tags:

- UsageMonitoringInformation (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:sm-data:modify

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/sponsor-connectivity-data/{sponsorId}:

parameters:

- name: sponsorId

in: path

required: true

schema:

type: string

get:

summary: Retrieves the sponsored connectivity information for a given sponsorId

operationId: ReadSponsorConnectivityData

tags:

- SponsorConnectivityData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:sponsor-connectivity-data:read

responses:

'200':

description: >

Upon success, a response body containing Sponsor Connectivity Data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SponsorConnectivityData'

'204':

description: The resource was found but no Sponsor Connectivity Data is available.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/bdt-data:

get:

summary: Retrieves the BDT data collection

operationId: ReadBdtData

tags:

- BdtData (Store)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:bdt-data:read

parameters:

- name: bdt-ref-ids

in: query

description: List of the BDT reference identifiers.

required: false

schema:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

minItems: 1

style: form

explode: false

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Upon success, a response body containing the BDT data shall be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/BdtData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/bdt-data/{bdtReferenceId}:

parameters:

- name: bdtReferenceId

in: path

required: true

schema:

type: string

get:

summary: Retrieves the BDT data information associated with a BDT reference Id

operationId: ReadIndividualBdtData

tags:

- IndividualBdtData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:bdt-data:read

parameters:

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Upon success, a response body containing the BDT data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Creates an BDT data resource associated with an BDT reference Id

operationId: CreateIndividualBdtData

tags:

- IndividualBdtData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:bdt-data:create

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

responses:

'201':

description: Successful case. The resource has been successfully created.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modifies an BDT data resource associated with an BDT reference Id

operationId: UpdateIndividualBdtData

tags:

- IndividualBdtData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:bdt-data:modify

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/BdtDataPatch'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

'204':

description: >

Successful case. The resource has been successfully updated and no additional content

is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an BDT data resource associated with an BDT reference Id

operationId: DeleteIndividualBdtData

tags:

- IndividualBdtData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:bdt-data:modify

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/subs-to-notify:

post:

summary: Create a subscription to receive notification of policy data changes

operationId: CreateIndividualPolicyDataSubscription

tags:

- PolicyDataSubscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data:subs-to-notify

- nudr-dr:policy-data:subs-to-notify:create

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

responses:

'201':

description: >

Upon success, a response body containing a representation of each Individual

subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

policyDataChangeNotification:

'{$request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PolicyDataChangeNotification'

minItems: 1

responses:

'204':

description: No Content, Notification was successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/subs-to-notify/{subsId}:

parameters:

- name: subsId

in: path

required: true

schema:

type: string

put:

summary: Modify a subscription to receive notification of policy data changes

operationId: ReplaceIndividualPolicyDataSubscription

tags:

- IndividualPolicyDataSubscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:subs-to-notify:modify

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

responses:

'200':

description: The individual subscription resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

'204':

description: >

The individual subscription resource was updated successfully and no

additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the individual Policy Data subscription

operationId: DeleteIndividualPolicyDataSubscription

tags:

- IndividualPolicyDataSubscription (Document)

responses:

'204':

description: Upon success, an empty response body shall be returned.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/operator-specific-data:

get:

summary: Retrieve the operator specific policy data of an UE

operationId: ReadOperatorSpecificData

tags:

- OperatorSpecificData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:operator-specific-data:read

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the operator specific policy data of a UE

operationId: UpdateOperatorSpecificData

tags:

- OperatorSpecificData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:operator-specific-data:modify

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: No content. Response to successful modification.

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create or modify the operator specific policy data of a UE

operationId: ReplaceOperatorSpecificData

tags:

- OperatorSpecificData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:operator-specific-data:create

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

required: true

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

responses:

'200':

description: OK

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

'201':

description: >

Successful case. When the feature OSDResource_Create_Delete is supported

and the resource has been successfully created, a response body containing a

representation of the created OperatorSpecificData resource shall be returned.

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: The resource has been successfully updated.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: When the feature OSDResource_Create_Delete is supported, delete OperatorSpecificData resource

operationId: DeleteOperatorSpecificData

tags:

- OperatorSpecificData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:ues:operator-specific-data:modify

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/plmns/{plmnId}/ue-policy-set:

parameters:

- name: plmnId

in: path

required: true

schema:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/VarPlmnId'

get:

summary: Retrieve the UE policy set data for an H-PLMN

operationId: ReadPlmnUePolicySet

tags:

- PlmnUePolicySet (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:plmns:ue-policy-set:read

responses:

'200':

description: Upon success, a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'412':

$ref: 'TS29571_CommonData.yaml#/components/responses/412'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/slice-control-data/{snssai}:

parameters:

- name: snssai

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

get:

summary: Retrieves a network Slice specific policy control data resource

operationId: ReadSlicePolicyControlData

tags:

- SlicePolicyControlData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:slice-control-data:read

parameters:

- name: supp-feat

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

Successful case. The network slice specific policy control data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify a network Slice specific policy control data resource

operationId: UpdateSlicePolicyControlData

tags:

- SlicePolicyControlData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:slice-control-data:modify

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SlicePolicyDataPatch'

responses:

'200':

description: >

The resource has been successfully updated and a response body containing network

slice specific policy control data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePolicyData'

'204':

description: >

The resource has been successfully updated and no additional content is

to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/mbs-session-pol-data/{polSessionId}:

parameters:

- name: polSessionId

description: >

Represents the identifier of the MBS Session Policy Control Data.

in: path

required: true

schema:

$ref: '#/components/schemas/MbsSessPolDataId'

get:

summary: Retrieve MBS Session Policy Control Data for an MBS Session.

operationId: GetMBSSessPolCtrlData

tags:

- MBSSessionPolicyControlData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:policy-data

- nudr-dr:policy-data:mbs-session-pol-data:read

responses:

'200':

description: >

OK. The requested MBS Session Policy Control Data is successfully returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsSessPolCtrlData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

PolicyDataForIndividualUe:

description: Contains policy data for a given subscriber.

type: object

properties:

uePolicyDataSet:

$ref: '#/components/schemas/UePolicySet'

smPolicyDataSet:

$ref: '#/components/schemas/SmPolicyData'

amPolicyDataSet:

$ref: '#/components/schemas/AmPolicyData'

umData:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonData'

minProperties: 1

description: >

Contains UM policies. The value of the limit identifier is used as the key of the map.

operatorSpecificDataSet:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

minProperties: 1

description: >

Contains Operator Specific Data resource data. The key of the map is operator

specific data element name and the value is the operator specific data of the UE.

AmPolicyData:

description: Contains the AM policy data for a given subscriber.

type: object

properties:

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains Presence reporting area information. The praId attribute within the

PresenceInfo data type is the key of the map.

subscCats:

type: array

items:

type: string

minItems: 1

UePolicySet:

description: Contains the UE policy data for a given subscriber.

type: object

properties:

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains Presence reporting area information. The praId attribute within the

PresenceInfo data type is the key of the map.

subscCats:

type: array

items:

type: string

minItems: 1

uePolicySections:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicySection'

minProperties: 1

description: >

Contains the UE Policy Sections. The UE Policy Section Identifier is used as

the key of the map.

upsis:

type: array

items:

type: string

minItems: 1

allowedRouteSelDescs:

type: object

additionalProperties:

$ref: '#/components/schemas/PlmnRouteSelectionDescriptor'

minProperties: 1

description: >

Contains allowed route selection descriptors per serving PLMN for a UE.

The serving PLMN identifier is the key of the map.

andspInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

osIds:

type: array

items:

$ref: '#/components/schemas/OsId'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resetIds:

type: array

items:

type: string

minItems: 1

UePolicySetPatch:

description: Contains the UE policy set for a given subscriber.

type: object

properties:

uePolicySections:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicySection'

minProperties: 1

description: >

Contains the UE Policy Sections. The UE Policy Section Identifier is used

as the key of the map.

upsis:

type: array

items:

type: string

minItems: 1

andspInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

osIds:

type: array

items:

$ref: '#/components/schemas/OsId'

minItems: 1

UePolicySection:

description: Contains the UE policy section.

type: object

properties:

uePolicySectionInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

upsi:

type: string

required:

- uePolicySectionInfo

- upsi

SmPolicyData:

description: Contains the SM policy data for a given subscriber.

type: object

properties:

smPolicySnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicySnssaiData'

minProperties: 1

description: >

Contains Session Management Policy data per S-NSSAI for all the SNSSAIs

of the subscriber. The key of the map is the S-NSSAI.

umDataLimits:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataLimit'

minProperties: 1

description: >

Contains a list of usage monitoring profiles associated with the subscriber.

The limit identifier is used as the key of the map.

umData:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonData'

minProperties: 1

description: >

Contains the remaining allowed usage data associated with the subscriber.

The limit identifier is used as the key of the map.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resetIds:

type: array

items:

type: string

minItems: 1

required:

- smPolicySnssaiData

SmPolicySnssaiData:

description: Contains the SM policy data for a given subscriber and S-NSSAI.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

smPolicyDnnData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicyDnnData'

minProperties: 1

description: >

Session Management Policy data per DNN for all the DNNs of the indicated S-NSSAI.

The key of the map is the DNN.

ueSliceMbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SliceMbr'

required:

- snssai

SmPolicyDnnData:

description: Contains the SM policy data for a given DNN (and S-NSSAI).

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

allowedServices:

type: array

items:

type: string

minItems: 1

subscCats:

type: array

items:

type: string

minItems: 1

gbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

gbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

adcSupport:

type: boolean

subscSpendingLimits:

type: boolean

ipv4Index:

$ref: '#/components/schemas/IpIndex'

ipv6Index:

$ref: '#/components/schemas/IpIndex'

offline:

type: boolean

online:

type: boolean

chfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

refUmDataLimitIds:

type: object

additionalProperties:

$ref: '#/components/schemas/LimitIdToMonitoringKey'

minProperties: 1

description: >

A reference to the UsageMonitoringDataLimit or UsageMonitoringData instances

for this DNN and SNSSAI that may also include the related monitoring key(s).

The key of the map is the limit identifier.

mpsPriority:

type: boolean

mcsPriority:

type: boolean

imsSignallingPrio:

type: boolean

mpsPriorityLevel:

type: integer

mcsPriorityLevel:

type: integer

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains Presence reporting area information. The praId attribute within the

PresenceInfo data type is the key of the map.

bdtRefIds:

type: object

additionalProperties:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceIdRm'

minProperties: 1

description: >

Identifies transfer policies of background data transfer. Any string value can

be used as a key of the map.

nullable: true

locRoutNotAllowed:

type: boolean

required:

- dnn

UsageMonDataLimit:

description: Contains usage monitoring control data for a subscriber.

type: object

properties:

limitId:

type: string

scopes:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataScope'

minProperties: 1

description: >

Identifies the SNSSAI and DNN combinations to which the usage monitoring data

limit applies. The S-NSSAI is the key of the map.

umLevel:

$ref: '#/components/schemas/UsageMonLevel'

startDate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endDate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

usageLimit:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

resetPeriod:

$ref: '#/components/schemas/TimePeriod'

required:

- limitId

UsageMonData:

description: Contains remain allowed usage data for a subscriber.

type: object

properties:

limitId:

type: string

scopes:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataScope'

minProperties: 1

description: >

Identifies the SNSSAI and DNN combinations for remain allowed usage data

for a subscriber. The S-NSSAI is the key of the map.

umLevel:

$ref: '#/components/schemas/UsageMonLevel'

allowedUsage:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

resetTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resetIds:

type: array

items:

type: string

minItems: 1

required:

- limitId

LimitIdToMonitoringKey:

description: >

Contains the limit identifier and the corresponding monitoring key for a given

S-NSSAI and DNN.

type: object

properties:

limitId:

type: string

monkey:

type: array

items:

type: string

minItems: 1

required:

- limitId

nullable: true

UsageMonDataScope:

description: >

Contains a SNSSAI and DNN combinations to which the UsageMonData instance belongs to.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

required:

- snssai

TimePeriod:

description: Contains the periodicity for the defined usage monitoring data limits.

type: object

properties:

period:

$ref: '#/components/schemas/Periodicity'

maxNumPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- period

SponsorConnectivityData:

description: >

Contains the sponsored data connectivity related information for a sponsor identifier.

type: object

properties:

aspIds:

type: array

items:

type: string

required:

- aspIds

BdtData:

description: Contains the background data transfer data.

type: object

properties:

aspId:

type: string

transPolicy:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/TransferPolicy'

bdtRefId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

nwAreaInfo:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

numOfUes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

volPerUe:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

trafficDes:

$ref: 'TS29122_ResourceManagementOfBdt.yaml#/components/schemas/TrafficDescriptor'

bdtpStatus:

$ref: '#/components/schemas/BdtPolicyStatus'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resetIds:

type: array

items:

type: string

minItems: 1

required:

- aspId

- transPolicy

PolicyDataSubscription:

description: Identifies a subscription to policy data change notification.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monResItems:

type: array

items:

$ref: '#/components/schemas/ResourceItem'

minItems: 1

excludedResItems:

type: array

items:

$ref: '#/components/schemas/ResourceItem'

minItems: 1

immRep:

type: boolean

description: >

If provided and set to true, it indicates that existing entries that

match this subscription shall be immediately reported in the response.

immReports:

type: array

items:

$ref: '#/components/schemas/PolicyDataChangeNotification'

minItems: 1

description: Immediate report with existing UDR entries.

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resetIds:

type: array

items:

type: string

minItems: 1

required:

- notificationUri

- monitoredResourceUris

PolicyDataChangeNotification:

description: Contains changed policy data for which notification was requested.

type: object

properties:

amPolicyData:

$ref: '#/components/schemas/AmPolicyData'

uePolicySet:

$ref: '#/components/schemas/UePolicySet'

plmnUePolicySet:

$ref: '#/components/schemas/UePolicySet'

smPolicyData:

$ref: '#/components/schemas/SmPolicyData'

usageMonData:

$ref: '#/components/schemas/UsageMonData'

SponsorConnectivityData:

$ref: '#/components/schemas/SponsorConnectivityData'

bdtData:

$ref: '#/components/schemas/BdtData'

opSpecData:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

opSpecDataMap:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

minProperties: 1

description: >

Operator Specific Data resource data, if changed and notification was requested.

The key of the map is operator specific data element name and the value is the

operator specific data of the UE.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

sponsorId:

type: string

bdtRefId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

usageMonId:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

delResources:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

notifId:

type: string

reportedFragments:

type: array

items:

$ref: '#/components/schemas/NotificationItem'

minItems: 1

slicePolicyData:

$ref: '#/components/schemas/SlicePolicyData'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

PlmnRouteSelectionDescriptor:

description: >

Contains the route selection descriptors (combinations of SNSSAI, DNNs, PDU session types,

SSC modes and ATSSS information) allowed by subscription to the UE for a serving PLMN

type: object

properties:

servingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

snssaiRouteSelDescs:

type: array

items:

$ref: '#/components/schemas/SnssaiRouteSelectionDescriptor'

minItems: 1

required:

- servingPlmn

SnssaiRouteSelectionDescriptor:

description: >

Contains the route selector parameters (DNNs, PDU session types, SSC modes and ATSSS

information) per SNSSAI

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnRouteSelDescs:

type: array

items:

$ref: '#/components/schemas/DnnRouteSelectionDescriptor'

minItems: 1

required:

- snssai

DnnRouteSelectionDescriptor:

description: >

Contains the route selector parameters (PDU session types, SSC modes and ATSSS

information) per DNN

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sscModes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

minItems: 1

pduSessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

atsssInfo:

description: >

Indicates whether MA PDU session establishment is allowed for this DNN.

When set to value true MA PDU session establishment is allowed for this DNN.

type: boolean

default: false

required:

- dnn

SmPolicyDataPatch:

description: Contains the SM policy data for a given subscriber.

type: object

properties:

umData:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonData'

minProperties: 1

description: >

Contains the remaining allowed usage data associated with the subscriber.

The value of the limit identifier is used as the key of the map.

nullable: true

smPolicySnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicySnssaiDataPatch'

minProperties: 1

description: >

Modifiable Session Management Policy data per S-NSSAI for all the SNSSAIs

of the subscriber. The key of the map is the S-NSSAI.

SmPolicySnssaiDataPatch:

description: Contains the SM policy data for a given subscriber and S-NSSAI.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

smPolicyDnnData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicyDnnDataPatch'

minProperties: 1

description: >

Modifiable Session Management Policy data per DNN for all the DNNs of the

indicated S-NSSAI. The key of the map is the DNN.

required:

- snssai

SmPolicyDnnDataPatch:

description: Contains the SM policy data for a given DNN (and S-NSSAI).

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

bdtRefIds:

type: object

additionalProperties:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceIdRm'

minProperties: 1

description: >

Contains updated transfer policies of background data transfer.

Any string value can be used as a key of the map.

nullable: true

required:

- dnn

ResourceItem:

description: >

Identifies a subscription to policy data change notification when the change occurs

in a fragment (subset of resource data) of a given resource.

type: object

properties:

monResourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

items:

type: array

items:

$ref: '#/components/schemas/ItemPath'

minItems: 1

required:

- monResourceUri

- items

NotificationItem:

description: >

Identifies a data change notification when the change occurs in a fragment

(subset of resource data) of a given resource.

type: object

properties:

resourceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifItems:

type: array

items:

$ref: '#/components/schemas/UpdatedItem'

minItems: 1

required:

- resourceId

- notifItems

UpdatedItem:

description: Identifies a fragment of a resource.

type: object

properties:

item:

$ref: '#/components/schemas/ItemPath'

value: {}

required:

- item

- value

BdtDataPatch:

description: Contains the modified background data transfer data.

type: object

properties:

transPolicy:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/TransferPolicy'

bdtpStatus:

$ref: '#/components/schemas/BdtPolicyStatus'

SlicePolicyData:

description: Contains the network slice specific policy control information.

type: object

properties:

mbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

mbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

remainMbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

remainMbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resetIds:

type: array

items:

type: string

minItems: 1

SlicePolicyDataPatch:

description: Contains the modified network slice specific policy control information.

type: object

properties:

remainMbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

remainMbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

oneOf:

- required: [remainMbrUl]

- required: [remainMbrDl]

MbsSessPolCtrlData:

description: Represents MBS Session Policy Control Data.

type: object

properties:

5qis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

minItems: 1

maxMbsArpLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ArpPriorityLevel'

maxMbsSessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxGbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

MbsSessPolDataId:

description: Represents the identifier used to access the MBS Session Policy Control Data.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

afAppId:

type: string

oneOf:

- required: [mbsSessionId]

- required: [afAppId]

# SIMPLE TYPES:

IpIndex:

description: >

Represents information that identifies which IP pool or external server

is used to allocate the IP address.

type: integer

OsId:

description: Represents the Operating System of the served UE.

type: string

format: uuid

ItemPath:

description: Identifies a fragment (subset of resource data) of a given resource.

type: string

# ENUMS:

UsageMonLevel:

description: Represents the usage monitoring level.

anyOf:

- type: string

enum:

- SESSION_LEVEL

- SERVICE_LEVEL

- type: string

Periodicity:

description: Represents the time period.

anyOf:

- type: string

enum:

- YEARLY

- MONTHLY

- WEEKLY

- DAILY

- HOURLY

- type: string

BdtPolicyStatus:

description: Indicates the validation status of a negotiated BDT policy.

anyOf:

- type: string

enum:

- INVALID

- VALID

- type: string

PolicyDataSubset:

description: Indicates a policy data subset.

anyOf:

- type: string

enum:

- AM_POLICY_DATA

- SM_POLICY_DATA

- UE_POLICY_DATA

- UM_DATA

- OPERATOR_SPECIFIC_DATA

- type: string

# * * * * End of changes * * * *