3GPP TSG-CT WG3 Meeting #125	C3-225314

# Toulouse, France, 14th - 18th, November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.537 | CR | 0012 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to reponse of create operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As defined in clause 6.1.3.2.3.1, the MbsPolicyData data type shall be returned with 201 status code and 308 status code may be returned by the PCF. The data type for the Headers supported by the 308 response code shall be "string" not "String". |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MbsPolicyData data type is returned with 201 status code and 308 status code may be returned by the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. And the PCF can’t return correct information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.3.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to Npcf_MBSPolicyControl API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of Changes * * * *

6.1.3.2.3.1	POST

This method enables an NF service consumer (e.g. MB-SMF) to request the creation of an MBS Policy Association at the PCF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPolicyCtxtData | M | 1 | Contains the parameters to request the creation of an MBS Policy Association. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPolicyData | M | 1 | 201 Created | Successful case. An Individual MBS Policy resource is successfully created. |
| n/a |  |  | 308 Permanent Redirect | Permanent Redirection. Another PCF already serves the MBS Session. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| MbsExtProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 6.1.7. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-mbspolicycontrol/<apiVersion>/mbs-policies/{mbsPolicyId} |

Table 6.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI (i.e. "apiRoot") of the PCF currently handling the MBS Session and towards which the request is redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the PCF (service) instance towards which the request is redirected. |

# * * * * Next change * * * *

A.2	Npcf_MBSPolicyControl API

openapi: 3.0.0

info:

title: Npcf_MBSPolicyControl API

version: 1.0.0

description: |

MBS Policy Control Service

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.537 V17.0.0; 5G System; Multicast/Broadcast Policy Control Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.537/'

security:

- {}

- oAuth2ClientCredentials:

- npcf-mbspolicycontrol

servers:

- url: '{apiRoot}/npcf-mbspolicycontrol/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

paths:

/mbs-policies:

post:

summary: Request the creation of a new MBS Policy Association.

operationId: CreateMBSPolicy

tags:

- MBS Policy Associations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyCtxtData'

responses:

'201':

description: >

Created. An Individual MBS Policy resource is successfully created.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyData'

headers:

Location:

description: >

Contains the URI of the newly created Individual MBS Policy Association resource.

required: true

schema:

type: string

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: Forbidden.

content:

application/problem+json:

schema:

$ref: 'TS29537_Npcf_MBSPolicyAuthorization.yaml#/components/schemas/MbsExtProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/mbs-policies/{mbsPolicyId}:

parameters:

- name: mbsPolicyId

in: path

description: >

Contains the identifier of the concerned Individual MBS Policy Association.

required: true

schema:

type: string

get:

summary: Read an Individual MBS Policy.

operationId: GetIndMBSPolicy

tags:

- Individual MBS Policy Association (Document)

responses:

'200':

description: >

OK. The requested Individual MBS Policy Association resource is successfully returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual MBS Policy resource.

operationId: DeleteIndMBSPolicy

tags:

- Individual MBS Policy Association (Document)

parameters:

- name: mbsPolicyId

in: path

description: >

Contains the identifier of the concerned Individual MBS Policy Association.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The concerned Individual MBS Policy Association resource is successfully

deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/mbs-policies/{mbsPolicyId}/update:

post:

summary: Request the update of an existing MBS Policy Association.

operationId: UpdateIndMBSPolicy

tags:

- Individual MBS POlicy Association (Document)

parameters:

- name: mbsPolicyId

in: path

description: >

Contains the identifier of the concerned Individual MBS Policy Association.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyCtxtDataUpdate'

responses:

'200':

description: >

OK. The targeted Individual MBS Policy Association resource is successfully updated.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: Forbidden.

content:

application/problem+json:

schema:

$ref: 'TS29537_Npcf_MBSPolicyAuthorization.yaml#/components/schemas/MbsExtProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-mbspolicycontrol: Access to the Npcf_MBSPolicyControl API

schemas:

MbsPolicyCtxtData:

description: >

Contains the parameters used to request the creation of an MBS Policy

Association.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

mbsServInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceInfo'

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsSessionId

MbsPolicyData:

description: >

Contains the MBS policy data provisioned as part of an MBS Policy Association.

type: object

properties:

mbsPolicyCtxtData:

$ref: '#/components/schemas/MbsPolicyCtxtData'

mbsPolicies:

$ref: '#/components/schemas/MbsPolicyDecision'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsPolicyCtxtData

MbsPolicyDecision:

description: >

Represents the parameters constituting an MBS Policy Decision.

type: object

properties:

mbsPccRules:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsPccRule'

minProperties: 1

description: >

A map of MBS PCC rule(s) with each map entry containing the MbsPccRule data structure.

The key of the map for each entry is the mbsPccRuleId attribute of the corresponding

MbsPccRule data structure.

nullable: true

mbsQosDecs:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsQosDec'

minProperties: 1

description: >

A map of MBS QoS Decision(s) with each map entry containing the MbsQosDecdata data

structure.The key of the map for each entry is the mbsQosId attribute of the corresponding

to the MbsQosDec data structure.

mbsQosChars:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsQosChar'

minProperties: 1

description: >

A map of MBS QoS Characteristics set(s) with each map entry containing the MbsQosChar data

structure. The key of the map for each entry is the 5QI attribute of the

corresponding MbsQosDec data structure.

authMbsSessAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

mbsPcrts:

type: array

items:

$ref: '#/components/schemas/MbsPcrt'

minItems: 1

nullable: true

MbsPccRule:

description: Represents the parameters constituting an MBS PCC rule.

type: object

properties:

mbsPccRuleId:

type: string

mbsDlIpFlowInfo:

type: array

items:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/FlowDescription'

minItems: 1

precedence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

refMbsQosDec:

type: array

items:

type: string

minItems: 1

maxItems: 1

required:

- mbsPccRuleId

MbsQosDec:

description: Represents the parameters constituting an MBS QoS Decision.

type: object

properties:

mbsQosId:

type: string

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

priorityLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5QiPriorityLevel'

mbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

gbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

averWindow:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AverWindow'

mbsMaxDataBurstVol:

$ref: '#/components/schemas/MbsMaxDataBurstVol'

required:

- mbsQosId

MbsQosChar:

description: Represents the parameters constituting a set of explicitly signalled QoS characteristics.

type: object

properties:

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

priorityLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5QiPriorityLevel'

resourceType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/QosResourceType'

packetDelayBudget:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

packetErrorRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

averWindow:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AverWindow'

mbsMaxDataBurstVol:

$ref: '#/components/schemas/MbsMaxDataBurstVol'

required:

- 5qi

- resourceType

- priorityLevel

- packetDelayBudget

- packetErrorRate

- mbsMaxDataBurstVol

MbsMaxDataBurstVol:

type: integer

minimum: 1

maximum: 2000000

description: Represents MBS Maximum Data Burst Volume, expressed in Bytes.

MbsPolicyCtxtDataUpdate:

description: >

Contains the parameters to request the modification of an existing MBS Policy Association.

type: object

properties:

mbsServInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceInfo'

mbsPcrts:

type: array

items:

$ref: '#/components/schemas/MbsPcrt'

minItems: 1

mbsErrorReport:

$ref: '#/components/schemas/MbsErrorReport'

MbsErrorReport:

description: >

Represents the reporting of MBS Policy decision level failure(s) and/or MBS PCC rule level

failure(s).

type: object

properties:

mbsReports:

type: array

items:

$ref: '#/components/schemas/MbsReport'

minItems: 1

MbsReport:

description: >

Contains information about the MBS Policy Decision level failure(s) and/or the MBS PCC

rule level failure(s).

type: object

properties:

mbsPccRuleIds:

type: array

items:

type: string

minItems: 1

mbsPccRuleStatus:

$ref: '#/components/schemas/MbsPccRuleStatus'

failureCode:

$ref: '#/components/schemas/MbsFailureCode'

# ENUMS:

MbsPcrt:

anyOf:

- type: string

enum:

- MBS_SESSION_UPDATE

- type: string

description: >

Represents MBS Policy Control Request Triggers.

description: |

Possible values are

- MBS_SESSION_UPDATE: Indicates the MBS Session Update policy control request trigger.

MbsPccRuleStatus:

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

description: >

Represents the MBS PCC rule status.

description: |

Possible values are

- ACTIVE: Indicates that the MBS PCC rule(s) are successfully installed.

- INACTIVE: Indicates that the MBS PCC rule(s) are removed.

MbsFailureCode:

anyOf:

- type: string

enum:

- NF_MALFUNCTION

- NF_RESOURCES_UNAVAILABLE

- RESOURCE_ALLOCATION_FAILURE

- MBS_QOS_VALIDATION_FAILURE

- NO_MBS_QOS_FLOW

- MBS_QOS_DECISION_ERROR

- MBS_POLICY_PARAM_ERROR

- type: string

description: >

Represents the reason for the MBS Policy Decision(s) enforcement failure or the MBS PCC

rule(s) installation failure.

description: |

Possible values are:

- NF_MALFUNCTION: Indicates that the MBS PCC rule could not be successfully installed due to MB-SMF/MB-UPF malfunction.

- NF_RESOURCES_UNAVAILABLE: Indicates that the MBS PCC rule could not be successfully installed due to resources unavailable at MB-SMF/MB-UPF.

- RESOURCE_ALLOCATION_FAILURE: Indicates that the MBS PCC rule could not be successfully installed or maintained since the associated MBS QoS flow establishment/modification failed or the associated MBS QoS flow was released.

- MBS_QOS_VALIDATION_FAILURE: Indicates that MBS QoS validation has failed.

- NO_MBS_QOS_FLOW: Indicates that there is no MBS QoS flow to which the MB-SMF can bind the MBS PCC rule(s).

- MBS_QOS_DECISION_ERROR: Indicates failure in the provisioning of MBS QoS Decision data.

- MBS_POLICY_PARAM_ERROR: Indicates that the information related to the provisioned MBS policy parameter(s) is incorrect, incomplete or inconsistent.

# * * * * End of change * * * *