3GPP TSG-CT WG3 Meeting #125	C3-225195_R3

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0121 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PATCH and PUT methods for the SS_NetworkResourceMonitoring API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The VAL server needs a mechanism to update an existing individual unicast QoS monitoring subscription on the NRM server side. The clause 14.4.4.6 of 3GPP TS 23.434 (CR #0112 of 3GPP TS 23.434) defines this functionality. However, the necessary procedure, HTTP metods and data structures are not defined in the current version of 3GPP TS 29.549. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR intoroduces the PUT and PATCH methods for the Update_Unicast_QoS_Monitoring service operation and related data structure in the SS_NetworkResourceMonitoring API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Impossiblity to replace or partially modify a unicast QoS monitoring subscription in the SS_NetworkResourceMonitoring API. - Misaligment with the Stage 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.2.1; 7.4.2.2.3.3.3 (new); 7.4.2.2.3.3.4 (new); 7.4.2.4.1; 7.4.2.4.2.11 (new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR does not affect any OpenAPI file. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

##### 7.4.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.4.2.2.1-1 depicts the resource URIs structure for the SS_NetworkResourceMonitoring API.

Figure 7.4.2.2.1-1: Resource URI structure of the SS_NetworkResourceMonitoring API

Table 7.4.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.4.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Unicast Monitoring Subscriptions | /subscriptions | POST | Create individual unicast monitoring subscription resource or obtain unicast QoS monitoring data for VAL UEs, VAL Group, or VAL Streams. |
| Individual Unicast Monitoring Subscription | /subscriptions/{subscriptionId} | DELETE | Remove an existing individual unicast monitoring subscription resource according to the subscriptionId. |
|  |  | GET | Read an existing individual unicast monitoring subscription resource according to the subscriptionId. |
|  |  | PUT | Update an individual unicast monitoring subscription identified by the subscriptionId. |
|  |  | PATCH | Modify an individual unicast monitoring subscription identified by the subscriptionId. |

* * * Next Change * * * *

7.4.2.2.3.3.3	PUT

This operation updates the "Individual Unicast Monitoring Subscription". This method shall support the URI query parameters specified in table 7.4.2.2.3.3.3-1.

Table 7.4.2.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.2.2.3.3.3-2 and the response data structures and response codes specified in table 7.4.2.2.3.3.3-3.

Table 7.4.2.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | Updated details of the unicast QoS monitoring subscription. The target identifiers, i.e., the "valUeIds", "valGroupId”, and "valStreamIds" attributes shall not be changed in the MonitoringSubscription data structure. |

Table 7.4.2.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | 200 OK | The subscription is updated successfully, and the updated subscription information returned in the response. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.2.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.4.2.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

* * * Next Change * * * *

7.4.2.2.3.3.4	PATCH

This operation modifies the "Individual Unicast Monitoring Subscription". This method shall support the URI query parameters specified in table 7.4.2.2.3.3.4-1.

Table 7.4.2.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.2.2.3.3.4-2 and the response data structures and response codes specified in table 7.4.2.2.3.3.4-3.

Table 7.4.2.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitoringSubscriptionPatch | M | 1 | Contains the modifications to be applied to the individual unicast QoS monitoring subscription resource. |

Table 7.4.2.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | 200 OK | Individual individual unicast QoS monitoring subscription resource is modified successfully and representation of the modified individual unicast QoS monitoring subscription resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.2.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.4.2.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

* * * Next Change * * * *

##### 7.4.2.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.4.2.4.1-1 specifies the data types defined specifically for the SS_NetworkResourceMonitoring API service.

Table 7.4.2.4.1-1: SS_NetworkResourceMonitoring API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| FailureReport | 7.4.2.4.2.9 | Represents the failure report indicating the VAL UE(s) or VAL Stream ID(s) for which the NRM server failed to obtain the requested data. |  |
| FailureReason | 7.4.2.4.3.3 | Represents the failure reason. |  |
| MeasurementData | 7.4.2.4.2.3 | Presents the aggregated measurement data. |  |
| MeasurementDataType | 7.4.2.4.3.1 | Indicates the requested measurement data type. |  |
| MeasurementPeriod | 7.4.2.4.2.4 | Indicates the measurement time period. |  |
| MeasurementRequirements | 7.4.2.4.2.6 | Indicates the measurement requirements. |  |
| MonitoringReport | 7.4.2.4.2.2 | Indicates the monitoring report for VAL UEs list, VAL Group, or VAL Stream. |  |
| MonitoringSubscription | 7.4.2.4.2.7 | The monitoring subscription request. |  |
| MonitoringSubscriptionPatch | 7.4.2.4.2.11 | Represents the monitoring subscription modification request. | UpdateSupport |
| ReportingRequirements | 7.4.2.4.2.5 | Indicates the requested requirements of reporting. |  |
| ReportingThreshold | 7.4.2.4.2.10 | Indicates a requested reporting threshold. |  |
| TerminationMode | 7.4.2.4.3.2 | Indicates the termination mode. |  |
| ThresholdHandlingMode | 7.4.2.4.3.4 | Indicates the multi-parameter threshold handling mode. |  |

Table 7.4.2.4.1-2 specifies data types re-used by the SS_NetworkResourceMonitoring API service.

Table 7.4.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AverWindow | 3GPP TS 29.571 [21] |  |  |
| BitRate | 3GPP TS 29.571 [21] |  |  |
| DateTime | 3GPP TS 29.571 [21] |  |  |
| DurationSec | 3GPP TS 29.571 [21] |  |  |
| MatchingDirection | 3GPP TS 29.520 [33] |  |  |
| NotificationMethod | 3GPP TS 29.508 [32] |  |  |
| PacketLossRate | 3GPP TS 29.571 [21] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [21] |  |  |
| Uinteger | 3GPP TS 29.571 [21] |  |  |
| Uri | 3GPP TS 29.571 [21] |  |  |
| ValTargetUe | clause 7.3.1.4.2.3 | Used to identify either a VAL User ID or a VAL UE ID. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] |  |  |

* * * Next Change * * * *

###### 7.4.2.4.2.11	Type: MonitoringSubscriptionPatch

Table 7.4.2.4.2.11-1: Definition of type MonitoringSubscriptionPatch

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measReqs | MeasurementRequirements | O | 0..1 | Indicates the measurement requirements. |  |
| reportReqs | ReportingRequirements | O | 0..1 | Indicates the reporting requirements. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI where the notification should be delivered to. |  |

* * * End of changes * * * *