3GPP TSG-CT WG3 Meeting #127e	C3-231410r1

E-meeting, 17th – 21st April, 2023													(Revision of C3-231410)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0680 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections in NpConfiguration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI18 |  |  |  |  |  | Date: |  |  | 2023-3-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing application errors from UDM and missing re-use table description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections in the existing clauses including adding the missing Snssai in the re-use table, adding the missing applicable application errors from UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing application errors from UDM and missing re-use table description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13.2.1.1, 5.13.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

##### 5.13.2.1.1	Introduction

This clause defines data structures to be used in resource representations.

Table 5.13.2.1.1-1 specifies data types re-used by the NetworkParameterConfiguration API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the NetworkParameterConfiguration API.

Table 5.13.2.1.1-1: NetworkParameterConfiguration API re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [45] | Identifies a DNN. |
| IpAddr | 3GPP TS 29.571 [45] | UE IP Address. |
| MacAddr48 | 3GPP TS 29.571 [45] | MAC Address. |
| Snssai | 3GPP TS 29.571 [45] | Identifies an S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [45] | Used to negotiate the applicability of the optional features defined in table 5.13.4-1. |

Table 5.13.2.1.1-2 specifies the data types defined for the NpConfiguration API.

Table 5.13.2.1.1-2: NpConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ConfigurationNotification | 5.13.2.1.4 | Represents a configuration result notification. |  |
| NpConfiguration | 5.13.2.1.2 | Represents a network parameters configuration. |  |
| NpConfigurationPatch | 5.13.2.1.3 | Represents parameters used to request the modification of a network parameters configuration resource. |  |

# *** 2nd Change ***

#### 5.13.5.3	Application Errors

The application errors defined for the NpConfiguration API are listed in table 5.13.5.3-1.

Table 5.13.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| PARAMETER_OUT_OF_RANGE | 403 Forbidden | Indicates that the resource is not allowed to be created since one or more of the received parameters are out of range defined by operator policies. |  |
| MODIFICATION_NOT_ALLOWED | 403 Forbidden | The subscriber does not have the necessary subscription for external parameter provisioning. |  |
| USER_NOT_FOUND | 404 Not Found | The User does not exist. |  |
| GROUP_IDENTIFIER_NOT_FOUND | 404 Not Found | The group does not exist |  |
| AF_NOT_ALLOWED | 403 Forbidden | This AF is not allowed to perform external provisioning. |  |
| MTC_PROVIDER_NOT_ALLOWED | 403 Forbidden | MTC Provider not authorized to perform external provisioning or 5G VN Group creation. |  |
| REQUEST_NOT_AUTHORIZED | 403 Forbidden | Indicates that the AF specific UE ID retrieval request is not authorized. | UEId_retrieval |
| UE_ID_NOT_AVAILABLE | 404 Not Found | Indicates that the AF specific UE ID is not available. | UEId_retrieval |
| UE_NOT_FOUND | 404 Not Found | Indicates that the requested UE address is not found. | UEId_retrieval |

*** End of Changes ***