3GPP TSG CT WG3 Meeting #134	C3-242183r1

# Changsha, China, 15-19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.508 | CR | 0264 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing applicable feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNetAE |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 5.6.1-2, RatType data type is missing the applicable feature EneNA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicable feature EneNA is added to Table 5.6.1-2 for RatType data type. |  |  |  |  |  |  |  |  |
|  |  | on |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nsmf_EventExposure service based interface protocol.

Table 5.6.1-1: Nsmf_EventExposure specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AckOfNotify |  | 5.6.2.7 |  | Acknowledgement information of event notification |  |  |  |
| AppliedSmccType |  | 5.6.3.6 |  | The type of applied SM congestion control. |  | SMCCE |  |
| EventNotification |  | 5.6.2.5 |  | Describes notifications about a single event that occurred. |  |  |  |
| EventSubscription |  | 5.6.2.4 |  | Represents the subscription to a single event |  |  |  |
| NotificationMethod |  | 5.6.3.4 |  | Represents the notification methods that can be subscribed |  |  |  |
| NsmfEventExposure |  | 5.6.2.2 |  | Represents an Individual SMF Notification Subscription resource |  |  |  |
| NsmfEventExposureNotification |  | 5.6.2.3 |  | Describes Notifications about events that occurred. |  |  |  |
| PduSessionInfo |  | 5.6.2.12 |  | Represents session information. |  | UeCommunication |  |
| PduSessionInformation |  | 5.6.2.11 |  | Represents the PDU session related information. |  | UeCommunication |  |
| PduSessionStatus |  | 5.6.3.8 |  | Status of the PDU Session. |  | UeCommunication |  |
| SmfEvent |  | 5.6.3.3 |  | Represents the types of events that can be subscribed |  |  |  |
| SubId |  | 5.6.3.2 |  | Identifies an Individual SMF Notification Subscription. |  |  |  |
| SmNasFromSmf |  | 5.6.2.9 |  | Describes the information of the SM NAS messages from SMF with backoff timer |  | SMCCE |  |
| SmNasFromUe |  | 5.6.2.8 |  | Describes the information of the SM NAS requests from UE |  | SMCCE |  |
| TrafficCorrelationNotification |  | 5.6.2.14 |  | Represents the traffic correlation Information for Notification. |  | CommonEASDNAI |  |
| TransactionInfo |  | 5.6.2.10 |  | UE Session Management transaction information. |  | Dispersion |  |
| TransactionMetric |  | 5.6.3.7 |  | Metric on UE Session Management transactions. |  | Dispersion |  |
| UpfInformation |  | 5.6.2.13 |  | The information of the UPF serving the UE. |  | ServiceExperience DnPerformance |  |

Table 5.6.1-2 specifies data types re-used by the Nsmf_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf_EventExposure service based interface.

Table 5.6.1-2: Nsmf_EventExposure re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5Qi |  | 3GPP TS 29.571 [11] |  | The 5G QoS Identifier. |  | EnQfiAllocation |  |  |
| AccessType |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| AfResultInfo |  | 3GPP TS 29.522 [20] |  | Represents application handling information. |  |  |  |
| ApplicationId |  | 3GPP TS 29.571 [11] |  | The application identifier. |  | QfiAllocation PduSessionInfo |  |
| BitRate |  | 3GPP TS 29.571 [11] |  | Represents the bit rate. |  | EnQoSMon |  |  |
| CommunicationFailure |  | 3GPP TS 29.518 [13] |  | Represents the communication failure information. |  | CommunicationFailure |  |
| DateTime |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| DlDataDeliveryStatus |  | 3GPP TS 29.571 [11] |  | Status of downlink data delivery |  | DownlinkDataDeliveryStatus |  |
| DddTrafficDescriptor |  | 3GPP TS 29.571 [11] |  | Traffic descriptor of source of downlink data |  | DownlinkDataDeliveryStatus |  |
| Dnai |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| DnaiChangeType |  | 3GPP TS 29.571 [11] |  | Describes the types of DNAI change. |  |  |  |
| Dnn |  | 3GPP TS 29.571 [11] |  |  |  | QfiAllocation, PduSessionStatus |  |
| DurationSec |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| EthFlowDescription |  | 3GPP TS 29.514 [22] |  | Ethernet flow description |  | QfiAllocation |  |
| FlowDescription |  | 3GPP TS 29.514 [22] |  | IP flow description |  | QfiAllocation |  |
| Fqdn |  | 3GPP TS 29.571 [11] |  | FQDN |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| GroupId |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Guami |  | 3GPP TS 29.571 [11] |  | Globally Unique AMF Identifier |  |  |  |
| IpAddr |  | 3GPP TS 29.571 [11] |  | UE IP address. |  | Dispersion CommonEASDNAI |  |
| Ipv4Addr |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| MacAddr48 |  | 3GPP TS 29.571 [11] |  | MAC Address. |  |  |  |
| MutingExceptionInstructions |  | 3GPP TS 29.571 [11] |  | Contains instructions to be executed upon the occurrence of an event muting exception (e.g. full buffer). |  | EnhDataMgmt |  |
| MutingNotificationsSettings |  | 3GPP TS 29.571 [11] |  | Contains setting related to the muting of notifications. |  | EnhDataMgmt |  |
| NetworkAreaInfo |  | 3GPP TS 29.554 [27] |  | Identifies the network area. |  | AreaFilter UPEAS |  |  |
| NfInstanceId |  | 3GPP TS 29.571 [11] |  | Instance identity of the Network Function |  | UPEAS CommonEASDNAI |  |
| NotificationFlag |  | 3GPP TS 29.571 [11] |  | Notification flag. |  | EneNA |  |
| PartitioningCriteria |  | 3GPP TS 29.571 [11] |  | Used to partition UEs before applying sampling. |  | EneNA |  |
| PduSessionId |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| PduSessionType |  | 3GPP TS 29.571 [11] |  | PDU session type. |  | PduSessionStatus PduSessionInfo |  |
| PlmnIdNid |  | 3GPP TS 29.571 [11] |  | Identification of a network: the PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| Qfi |  | 3GPP TS 29.571 [11] |  | QoS flow identifier. |  | QfiAllocation |  |
| RatType |  | 3GPP TS 29.571 [11] |  | RAT type. |  | EneNA |  |
| RedirectResponse |  | 3GPP TS 29.571 [11] |  | Contains redirection related information. |  | ES3XX |  |
| RouteToLocation |  | 3GPP TS 29.571 [11] |  | A traffic route to/from an DNAI |  |  |  |
| SamplingRatio |  | 3GPP TS 29.571 [11] |  | Sampling Ratio. |  |  |  |
| SatelliteBackhaulCategory |  | 3GPP TS 29.571 [11] |  | Indicates the satellite backhaul category or non-satellite backhaul. |  | EnSatBackhaulCategoryChg |  |
| ServiceName |  | 3GPP TS 29.510 [12] |  | Name of the service instance. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [11] |  | S-NSSAI |  | QfiAllocation |  |
| SscMode |  | 3GPP TS 29.571 [11] |  | SSC Mode selected for the PDU Session. |  | PduSessionInfo |  |
| Supi |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [11] |  | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |  |  |
| TimeWindow |  | 3GPP TS 29.122 [24] |  | A start time and a stop time of a time window. |  | SMCCE |  |
| Uinteger |  | 3GPP TS 29.571 [11] |  |  |  |  |  |
| UpfEvent |  | 3GPP TS 29.564 [26] |  | Contains UPF event information. |  | UPEAS |  |
| Uri |  | 3GPP TS 29.571 [11] |  |  |  |  |  |

*** End of Changes ***