3GPP TSG-CT WG3 Meeting #140	C3-251218

# Wuhan, P.R. China; 07th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1571 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NATed UE public IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Notification Correlation ID is named as "notifId" attribute in UeAddrSubsc data type, while it is named as "notifCorrId" attribute in UeAddrNotif type. "ueNatAddrInfo" attribute is defined in the definition of UeAddrNotif type in the mainbody, but "uePubAddrInfo" attribute is defined in the UeAddrNotif type in the openAPI file. Table 5.35.5.2.4-1 mentions UE Address Subscription update, which is not supported. Uinteger instead of Uint16 is included in the re-used data type table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace "notifCorrId" with "notifId". Replace "ueNatAddrInfo" with "uePubAddrInfo". Replace Uinteger with Uint16. Remove UE Address Subscription update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment attribute names. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.5.1, 5.35.5.2.4, 5.35.5.2.6, A.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following API: - TS29522_UEAddress.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.35.5.1	General

This clause specifies the application data model supported by the UeAddress API. Table 5.35.5.1-1 specifies the data types defined for the UeAddress API.

Table 5.35.5.1-1: UeAddress service specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UeAddressInfo | 5.35.5.2.3 | Represents UE Address information. |  |
| UeAddressReq | 5.35.5.2.2 | Represents the parameters to request UE Address information. |  |
| UeAddrInfo | 5.35.5.2.5 | Represents the UE Public Address information. |  |
| UeAddrNotif | 5.35.5.2.6 | Represents the UE Address Notification. |  |
| UeAddrSubsc | 5.35.5.2.4 | Represents the UE Address Subscription. |  |

Table 5.35.5.1-2 specifies data types re-used by the UeAddress API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UeAddress API.

Table 5.35.5.1-2: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |
| IpAddr | 3GPP TS 29.571 [8] | Represents UE IP address. |
| Ipv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |
| Ipv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |
| UeNatMappingInfo | 3GPP TS 29.564 [61] | Represents UE NAT Mapping Information. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |
| Uint16 | 3GPP TS 29.571 [8] | Represents an unsigned 16-bit integer. |
| Uri | 3GPP TS 29.122 [4] | Represents a URI. |

# *** 2nd Change ***

5.35.5.2.4	Type: UeAddrSubsc

Table 5.35.5.2.4-1: Definition of type UeAddrSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Contains the URI for via which UE Address notifications shall be delivered. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| afId | string | M | 1 | Represents the identifier of the AF that is sending the request. |  |
| ueAddrInfo | UeAddrInfo | C | 0..1 | Contains the UE private address information and IP address information of the remote end. This attribute shall be present only if the UE public address information is subscribed. |  |
| immRepInd | boolean | C | 0..1 | Indicates whether immediate reporting is requested. - "true": immediate reporting is requested; - "false"(default): immediate reporting is not requested. It shall be present and set to "true" if the "ueAddrInfo" attribute is present. |  |
| immReport | UeNatMappingInfo | C | 0..1 | Contains the UE Address report. This attribute shall be present only in the response to a UE Address Subscription creation request and the information is available. (NOTE). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.35.6. It shall be present if feature negotiation needs to take place. |  |
| NOTE: The UPF may report a list of NATed public IP and port information for the same private UE IP address and the same remote IP address, for different destination port numbers, if no remote port number was received in the subscription within the ueAddrInfo attribute. |  |  |  |  |  |

# *** 3rd Change ***

5.35.5.2.6	Type: UeAddrNotif

Table 5.35.5.2.6-1: Definition of type UeAddrNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | string | M | 1 | Contains the identifier of the subscription to which the notification is related. |  |
| uePubAddrInfo | UeNatMappingInfo | M | 1 | Contains the list of NATed public address and port Information. The NEF shall return an empty value when there is no UeNatMappingInfo available. (NOTE) |  |
| NOTE: The UPF may return a list of NATed public IP and port information for the same private UE IP address and the same remote IP address, for different destination port numbers, if no remote port number was received in the subscription. |  |  |  |  |  |

# *** 4th Change ***

A.33	UEAddress API

openapi: 3.0.0

info:

title: 3gpp-ue-address

version: 1.1.0-alpha.2

description: |

API for UE Address service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V19.2.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-ue-address/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/retrieve:

post:

summary: Request to retrieve UE Address information.

operationId: RetrieveUEAddress

tags:

- UE Address(es) Information Retrieval

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddressReq'

responses:

'200':

description: >

OK. The UE Address information request is successfully processed, and UE address

information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddressInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

get:

summary: Request to retrieve all the active UE Address Subscriptions.

operationId: GetUEAddrSubscs

tags:

- UE Address Subscriptions (Collection)

parameters:

- name: supp-feat

in: query

description: >

Contains the list of supported features among the ones defined in clause 5.35.6.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. All the Individual UE Address Subscription resources managed by the NEF are

returned.

If there are no existing Individual UE Address Subscription resources managed at the

NEF, an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/UeAddrSubsc'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new UE Address Subscription at the NEF.

operationId: CreateUEAddrSubsc

tags:

- UE Address Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrSubsc'

responses:

'201':

description: >

Created. A representation of the created Individual UE Address Subscription resource is

returned in the response body

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrNotif'

responses:

'204':

description: >

No Content. The UE Address Notification is successfully received and

processed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: Represents the identifier of the Individual UE Address Subscription resource.

required: true

schema:

type: string

get:

summary: Request to retrieve an existing Individual UE Address Subscription resource.

operationId: GetIndUEAddrSubsc

tags:

- Individual UE Address Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual UE Address Subscription resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual UE Address Subscription resource.

operationId: DeleteIndUEAddrSubsc

tags:

- Individual UE Address Subscription (Document)

responses:

'204':

description: >

No Content. . The Individual UE Address Subscription resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

UeAddressReq:

description: Represents the parameters to request the retrieval of UE Address information.

type: object

properties:

afId:

type: string

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- gpsi

UeAddressInfo:

description: Represents UE Address information.

type: object

properties:

ueIpAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueIpAddrs

UeAddrSubsc:

description: Represents the UE Address Subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

afId:

type: string

ueAddrInfo:

$ref: '#/components/schemas/UeAddrInfo'

immRepInd:

type: boolean

description: >

Indicates whether immediate reporting shall be included in the corresponding response

if Available.

immReport:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/UeNatMappingInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- notifId

- afId

UeAddrInfo:

description: Represents the UE Public Address information.

type: object

properties:

uePrivateIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

ipDomain:

type: string

remoteIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

remoteIpv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

remotePortNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16'

required:

- uePrivateIpAddr

oneOf:

- required: [remoteIpv4Addr]

- required: [remoteIpv6Addr]

UeAddrNotif:

description: Represents the UE Address Notification.

type: object

properties:

notifId:

type: string

uePubAddrInfo:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/UeNatMappingInfo'

required:

- notifCorrId

- uePubAddrInfo

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

*** End of Changes ***