3GPP TSG-CT WG4 Meeting #127	C4-250453

# Athens, GREECE; 17th – 21st February 2025									   Revision of C4-250273

Source:	Nokia

Title:	Pseudo-CR on Optimized UPF data collection by bundling event reports of different subscriptions in a same notification message

Spec:	3GPP TR 29.889 v0.3.0

Agenda item:	20.1

Document for:	Decision

1. Reason for Change

The pCR proposes a new solution for optimizing UPF data collection by bundling event reports of different UPF event exposure subscriptions in a same notification message.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 29.889 v0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 29.564: "User Plane Function Services; Stage 3".

[6]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[7]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[10]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[x]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

* * * Next Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | 1 | 2 |
| 1 | X |  |
| 2 | X |  |
| 3 |  | X |
| 4 | X |  |
| 5 | X |  |
| 6 | X |  |
| X | X |  |

* * * Next Change * * * *

## 6.x	Solution #X: Optimized UPF data collection by bundling event reports of different subscriptions in a same notification message

### 6.x.1	Description

The solution addresses the Key Issue #1 on Identifying and lowering the network performance impacts of intensive data collection from UPF.

The solution proposes to enable the UPF to bundle event reports of different UPF event exposure subscriptions with a same eventNotifyUri in a same Nupf_EventExposure_Notify request message, to reduce the number of HTTP notify request/response messages that need to be exchanged between the UPF and the NF service consumer (e.g. NWDAF).

EXAMPLE:	Assuming an example NotificationData size of 4k octets (see Table 6.1.6.2.2-1 of 3GPP TS 29.564 [5]) and a maximum JSON payload of 16 million octets before compression is applied (see clause 6.2 of 3GPP TS 29.501 [x]), this provides the potential for bundling up to 4000 NotificationData from different subscriptions in a same Nupf_EventExposure_Notify request message.

### 6.x.2	Procedures

Figure 6.x.2-1 depicts an example call flow of an NF service consumer (e.g. NWDAF, SMF) creating a UPF event exposure subscription allowing the UPF to bundle notification data from multiple subscriptions in a same Nupf_EventExposure_Notify request message.

Figure 6.x.2-1: Bundling event reports of different subscriptions in a same Nupf_EventExposure_Notify request

1.	The NF Service Consumer sends a Nupf_EventExposure_Suscribe request to the UPF to subscribe to UPF event exposure, e.g. for a particular UE's PDU session. The request contains the following new parameters:

-	an indication of whether event reports generated for this subscription are allowed (not allowed by default) to be bundled with event reports generated for other subscriptions with a same eventNotifyUri in a same Nupf_EventExposure_Notify request message; and

-	if bundling is allowed, optionally, a Bundle Identifier (ID) to identify subscriptions (with the same notification URI) whose event reports may be bundled together, if the NF service consumer only allows to bundle event reports from specific (but not all) subscriptions, e.g. if the NWDAF wishes to only bundle event reports generated for a same analytic service request.

2.	The UPF creates an event subscription resource if the request is accepted and sends 201 Created as specified in 3GPP TS 29.564 [5].

3.	The UPF bundles multiple NotificationData generated for different UPF event exposure subscriptions with a same eventNotifyUri and, with a same Bundle ID if a Bundle ID was provided in the subscriptions, in a same Nupf_EventExposure_Notify Request message.

Editor's note:	It is FFS whether/how to enable bundling event reports from different subscriptions with different notification URIs (e.g. with a same consumer's endpoint when the consumer would assign different notification URI's paths to different subscriptions).

The bundling of multiple NotificationData in a same Nupf_EventExposure_Notify request may be defined e.g. by defining an ExtNotificationData (Extended NotificationData) data type and by defining the HTTP POST request (of the Nupf_EventExposure_Notify Request) as using this data type, as follows:

Table 6.x.2-1: Definition of type ExtNotificationData as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| NotificationData | 1 | Notification Data as defined in 3GPP TS 29.564 [5] |
| array(NotificationData) | 2..N | List of Notification Data |

Table 6.x.2-2: Data structures supported by the POST Request Body (Nupf_EventExposure_Notify Request)

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ExtNotificationData | M | 1 | Representation of one or more event notifications |

The solution does not modify the NotificationData information, i.e. the same NotificationData content is sent towards the NF service consumer regardless of whether the NotificationData is sent alone, or bundled with other NotificationData, in a Nupf_EventExposure_Notify request message.

When bundling is allowed and possible, the UPF bundles event reports from different subscriptions, taking the following aspects into consideration:

- bundling should not add latency/delay for delay critical events, i.e. the UPF may bundle delay critical events which happen nearly at the same time;

- bundling of delay tolerant events may happen as long as the bundled events are sent before their last allowed reporting time (indicated by the existing reportingTimeInfo IE).

The Nsmf_EventExposure_Subscribe request is enhanced with the same new parameters, to support bundling UPF event reports for subscriptions performed via the SMF, e.g. for NWDAF subscriptions to UPF event exposure for UEs in a certain Area of Interest. The SMF forwards the bundlingAllowed indication and if provided, the Bundle ID, to the UPF when subscribing to the UPF event exposure.

### 6.x.3	Impacts on services, entities and interfaces

UPF:

-	the Nupf_EventExposure service is enhanced to support bundling of event reports of different UPF event exposure subscriptions;

SMF:

-	the Nsmf_EventExposure_Subscribe request is enhanced with the new bundling parameters. The SMF forwards these parameters to the UPF.

Consumer of Nupf_EventExposure (e.g. NWDAF):

-	provides the new bundling parameters in the subscription creation request and supports notification message with bundled event reports

### 6.x.4	Pros

The solution provides the following benefits:

-	UPF and NF service consumer performance improvements:

-	Bundling event reports in a single HTTP request enables to enhance performance and scalability (reduced CPU and memory cost and faster time to transmit the event reports thanks to reducing the number of packets to be encoded/decoded, compressed/decompressed (e.g. gzip), protected (e.g. integrity protected and ciphered using TLS), packetized, exchanged and processed over Nupf.

-	Processing overhead is reduced at the UPF and NF service consumer (e.g. NWDAF) since the number of notifications is reduced.

-	Network performance improvements:

-	The UPF can bundle event reports/measurements from multiple PDU sessions in a same notification message, when individual UPF event exposure subscriptions are created for each PDU session, significantly reducing the number of notification messages (and therefore HTTP requests/responses) required to report the requested data.

-	The solution can be supported with minimal changes to the system.

Annex X provides further insights (test results) on the performance improvements that can be achieved by bundling event reports in notification messages.

### 6.x.5	Cons

The solution has the following drawbacks:

-	The solution does not enable to reduce the number of UPF event exposure subscriptions.

* * * Next Change * * * *

Annex X (informative):
Performance measurements of Bundling event reports in notification messages

# X.1	Bundling Test Setup

A test client software:

-	creates as many NotificationData objects as desired for the test and generates NotificationData bundles according to the bundling factor desired for the test (i.e., how many objects are bundled in a single request) and so that all existing objects are handled.

-	Each bundle is then sent in the HTTP POST request’s body to the server, which then acknowledges the reception of the request back to the client.

- 	The client sends HTTP POST requests as fast as possible, with the limit that there can be up to 2000 on-going in-flight requests, i.e., requests for which response has not yet been received.

Test client is implemented using Golang [https://go.dev/] using the following (main) SW components: i) for TLS: "crypto/tls", ii) for JSON: "encoding/json", and iii) for HTTP2: "golang.org/x/net/http2". For the server, open-source high performing HTTP server, called h2o, was used [https://github.com/h2o/h2o].

The client and the server are interconnected using HTTP/2 (with TLS). The duration of a single test case ("total duration") includes the following steps:

-	Creation of bundles for the event reports.

-	Serialization of the created bundles as JSON string.

-	For each bundle:

-	Creation of HTTP POST request message with the bundle in its body.

-	Sending HTTP POST request to the server.

-	Receiving acknowledgement back from the server.

-	Checking possible HTTP error.

Test parameters:

- Number of Objects: How many NotificationData objects are created and sent in the HTTP POST requests.

- Bundle size (# of Objects): How many objects are bundled in the same HTTP POST request.

Test Environment:

- 2x HPe Proliant DL360 G9 server HW; 40 cores, 128 RAM.

  - Interconnections: 40GBps (direct) - "Lossless" network; ~0.2ms RTT.

Notification Report:

Each notification report consists of one or more NotificationData objects that were based on UPF Event Exposure Service API (version: 1.1.0-alpha.5) [https://github.com/jdegre/5GC_APIs/tree/Rel-18] and the following Rel-18 APIs were used:

-	TS29564_Nupf_EventExposure.yaml

-	TS29510_Nnrf_AccessToken.yaml

-	TS29510_Nnrf_NFDiscovery.yaml

-	TS29510_Nnrf_NFManagement.yaml

-	TS29512_Npcf_SMPolicyControl.yaml

-	TS29514_Npcf_PolicyAuthorization.yaml

-	TS29571_CommonData.yaml

For the code generation of the UPF Event Exposure Service API, Openapi-generator-cli v7.9.0 [https://hub.docker.com/r/openapitools/openapi-generator-cli/] was used to generate Golang code.

The used structure of NotificationData object was as follows:

-	NotificationData:

-	NotificationItem:

-	UserDataUsageMeasurements:

-	FlowInformation

-	EthFlowDescription

-	VolumeMeasurement

-	ThroughputMeasurement

-	ApplicationRelatedInformation (with a single url and DomainInformation)

-	ThroughputStatisticsMeasurement

Each NotificationData object was constructed with some "reasonable" default values in such way that the size was ~2258 bytes (as serialized JSON string) depending on the construction order.

# X.2	Bundling Test Results

Figure X.2-1 illustrates the results of "Bundling vs. Non-bundling" tests consisting of 4 test cases: "non-bundling" case and three bundling cases ("bundle 5", "bundle 20" and "bundle 4000"), where "bundle N" just indicates how many NotificationData objects are bundled in a single HTTP message and respectively "non-bundling" means "bundle 1" (as per current standard).

In these test results, Number of Objects value was 200 000 and maximum Bundle Size value 4000. The size of the transferred data in each was ~453 MB. While running the tests, different Number of Objects values were also tested and the results showed that the trends between the test cases are similar.

Following four metrics were measured in the test client during test run:

- (JSON) serialization time;

- total duration (see above for the exact steps that were measured);

- CPU load (average) and

- Number of HTTP POST requests sent.

In the figure, percentual values are used, because the trend is the interesting factor between the cases and actual values are dependent on e.g. software, hardware and measurement intervals used in the test. Non-bundling represents the reference value 100 (%) against which the bundling cases are compared to.

Figure X.2-1: Bundling Test Results

The effectiveness of bundling against the non-bundling case is evident already in the "bundle 5" case. Clear reductions are visible in total duration, CPU load and sent HTTP request packets:

- Total duration drops 26 %,

- CPU load drops 54.1%, and

- Number of sent HTTP POST requests drops 80 %.

As expected, the effect to serialization is small compared to other metrics:

- JSON serialization time drops 3.9%,

For "bundle 20" and "bundle 4000" cases the trend continues, and respectively the benefits are even more significant. The actual values can be seen in the figure X.2-1.

These results clearly show that even a small amount of bundling would be very effective to reduce the number of sent messages, the used time for local processing and communication time, and CPU consumption.

* * * Next Change * * * *

Annex Y (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | CT4#124 | C4-243602 |  |  |  | TR skeleton | 0.0.0 |
| 2024-08 | CT4#124 | C4-243672 |  |  |  | Inclusion of pCRs agreed at CT4#124, including C4-243556, C4-243595, C4-243600, C4-243603, C4-243605. | 0.1.0 |
| 2024-10 | CT4#125 | C4-244500 |  |  |  | Inclusion of pCRs agreed at CT4#125, including C4-244226, C4-244451, C4-244464, C4-244529, C4-244530, C4-244531, C4-244548 | 0.2.0 |
| 2024-11 | CT4#126 | C4-245538 |  |  |  | Inclusion of pCRs agreed at CT4#126, including C4-245388, C4-245389, C4-245390. | 0.3.0 |

* * * End of Changes * * * *