3GPP TSG-CT4 Meeting #129	C4-252311

Bratislava, Slovakia; 19th – 23rd May 2025	was C4-252094

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0348 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to Nlmf_DataExposure API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 05-05-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes enhancements to Nlmf_DataExposure API. Model performance monitoring is a continuous process where NWDAF would require data samples on regular intervals of time. Currently, this requirement can be met by NWDAF initiating multiple requests at regular time intervals. As a stage 3 level optimization, to avoid multiple API invocations, a provision is introduced in the LMF Data Exposure API request to indicate the interval(s) for which the sample data collection has to be initiated by the LMF. The time interval could be provisioned in this API by re-using the data type RecurTime from TS 29.503. This CR proposes the following changes in LmfDataExposureSubscription as follows, - Updating the timeWindow attribute’s name and data type to initiate data sample collection at specific time interval. - Provide description to clarify on report generation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of reference to TS 29.503 in section 2 - Addition of service description in clause 5.4.2.2. - Removal of data type timeWindow in re-used data types section 6.3.6.1 - Addition of data type RecurTime in re-used data types section 6.3.6.1 - Updating name and datatype of attribute timeWindow in clause 6.3.6.2.2 LmfDataExposureSubscription - Corresponding OpenAPI impacts in clause A.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, then multiple subscriptions need to initiated by LMF for Model performance monitoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.2.2, 6.3.6.1, 6.3.6.2.2, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible.correction to the following APIs: TS29572_Nlmf_DataExposure.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Removing modification of changes in attribute timeWindow and removing addition of sampleCollecIntrvl in clause 6.3.6.2.2. Updation of name and data type of timeWindow attribute in clause 6.3.6.2.2 and addition of description in clause 5.4.2.2. Add co-signees in cover sheet. |  |  |  |  |  |  |  |  |

# * * * * First Change* * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 9457: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 29.518: "Access and Mobility Management Services".

[24]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[25]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[26]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[27]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[28]	Void

[29]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[30]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[31]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[32]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[XX]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

# * * * * Next Change* * * *

#### 5.4.2.2	LMF Data Exposure Subscription

This procedure allows NF Service Consumer to subscribe by using HTTP POST method with the URI of individual subscription resource.

Figure 5.4.2.2-1: Subscription of NF service consumer to LMF data exposure

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the subscription collection. The input parameters for the request shall include notification target address and may include notification correlation ID and AoI.

2a.	On success, "201 Created" shall be returned. The response body shall include a HTTP Location header including the subscription ID together with the status code 201 indicating the requested resource is created in the response message.

-	If the LmfDataExposureSubscription contains "recurTime" IE, then LMF shall generate a report at the end of every time interval provided in the "recurTime" IE.

-	If the LmfDataExposureSubscription contains "recurTime" IE and "numOfSamples" IE, then LMF shall generate a report when the requested number of samples are met or at the end of every time interval provided in the "recurTime" IE.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.3.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

# * * * * Next Change* * * *

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Nlmf_DataExposure service-based interface protocol.

Table 6.3.6.1-1: Nlmf_DataExposure specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| LmfDataExposureSubscription | 6.3.6.2.2 | Represents an individual data collection subscription resource on LMF |
| LmfDataExposureNotification | 6.3.6.2.4 | Notify within an LMF exposure for data collection notification request. |
| LmfDataExposureReport | 6.3.6.2.5 | Represents the data collection exposure report. |

Table 6.3.6.1-2 specifies data types re-used by the Nlmf_DataExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_DataExposure service based interface.

Table 6.3.6.1-2: Nlmf_DataExposure re-used Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| PresenceInfo | 3GPP TS 29.571 [8] | Area of Interest |
| Uri | 3GPP TS 29.571 [8] | URI |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |
| DateTime | 3GPP TS 29.571 [8] | Date time |
| PatchItem | 3GPP TS 29.571 [8] | Patch item |
| Uinteger | 3GPP TS 29.571 [8] |  |
| RecurTime | 3GPP TS 29.503 [XX] | Recurring Time window |

# * * * * Next Change* * * *

##### 6.3.6.2.2	Type: LmfDataExposureSubscription

Table 6.3.6.2.2-1: Definition of type LmfDataExposureSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | This IE shall be included to identify the recipients of notifications sent by LMF for data exposure services. |  |
| notifyCorrelationId | CorrelationID | M | 1 | This IE shall be included to identify the notification correlation ID. The LMF shall include this ID in the notifications. The value of this IE shall be unique per subscription for a given NF service consumer. |  |
| aoi | PresenceInfo | M | 1 | Area of Interest |  |
| recurTime | RecurTime | O | 0..1 | When present, it shall indicate the list of time interval(s) during which LMF needs to initiate the collection of input data samples. . |  |
| numOfSamples | Uinteger | O | 0..1 | Number of samples. If the recurTime IE is present, then this IE shall be applied per each time interval inside recurTime IE. |  |

# * * * * Next Change* * * *

# A.4	Nlmf_DataExposure API

**************** Text Skippef for Clarity *****************

schemas:

#

# STRUCTURED TYPES

#

LmfDataExposureSubscription:

description: Describes subscription operation of LMF data exposure

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

aoi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

recurTime:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/RecurTime'

numOfSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- notificationUri

- notifyCorrelationId

- aoi

LmfDataExposureNotification:

description: Describes Notification for LMF data collection

properties:

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

reports:

type: array

items:

$ref: '#/components/schemas/LmfDataExposureReport'

minItems: 1

required:

- notifyCorrelationId

- reports

LmfDataExposureReport:

description: Describes an LMF data collection exposure report

properties:

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- timestamp

# * * * * End of Changes* * * *