3GPP TSG-CT WG4 Meeting #106-e	C4-215437

# E-Meeting, 11th – 15th October 2021					Revision of C4-215152

Title:	LS-Reply on Home Network triggered re-authentication

Release:	Release-17

Work Item:	SBIProtoc17

Source:	CT4

To:	SA3

Cc:	CT1

Contact person:	Varini Gupta

varini.gupta@Samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT4 thanks SA3 for the LS on Home Network triggered re-authentication. CT4 would like to provide following responses to the questions:

1) Whether it is possible for the UDM to send the application error (REAUTHENTICATION_REQUIRED) to the AMF without a request from the AMF.

[CT4] No, this error cannot be sent without a request from the AMF.

2) Whether CT4 has already specified any procedure that allows the home network (particularly UDM and/or AUSF) to trigger re-authentication procedure.

[CT4] CT4 defines UDM triggered AMF de-registration procedure which can cause de-registration and re-registration of the UE if the security context in the UE is removed, so that UDM can insist for re-authentication during re-registration. Though this procedure is not intended for triggering re-authentication, as it results in service impact.

# 2	Actions

To SA3

ACTION: 	CT4 requests SA3 to take above information into account.

# 3	Dates of next TSG CT WG4 meetings

CT4#107-e		15 - 23 November 2021		Electronic meeting