3GPP TSG CT4 Meeting #53	C4-111208

- Tallinn, ESTONIA
9th  – 13th May 2011

| CR-Form-v9.8 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.002 | CR | CRNum |  rev | - |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | HLR/HSS domain selection function alignment with Stage 2 requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  | Date:  |  | 26/04/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | The section 7.2.1 of TS 23.221 states, The ability to select whether the CS domain or the IMS will provide the services is termed the Service Domain Selection (SDS) (see clause 7.2.2) and whether the CS domain or the IMS/IP-CAN is used to communicate call termination from the network and the UE is termed access domain selection (ADS) (see clauses 7.2a and 7.2b). In section 7.2.2 of TS 23.221, SDS functionality is defined as followed, The G-MSC Server or G-MSC interrogates the HSS for routing information. The HSS decides on the way the call shall be treated next (e.g. IM CN subsystem, CS domain (e.g. the subscriber is roaming in a legacy network), service platform involvement). According to the decision, the HSS returns information that will make the G-MSC progress the call towards an MGCF (for onward handling in the IMS), a VMSC or to provide further processing (e.g. invoke CAMEL G-MSC processing). |  |  |  |  |  |  |  |  |
| Summary of change:  |  | However, there is no specification to specify how the HLR/HSS should handle the interrogation mentioned in section 7.2.2 of TS 23.221. Therefore, it is supposed to add the following description in the section 21.2.8 of TS 29.002 and relevant modifications to the procedure figures. On receiving MAP_SRI message from GMSC, the HLR should execute SDS, i.e. to check if the UE being terminated is located in CS or IMS and then decide to request VMSC or MGCF to provide MSRN. In that case, the UE could be correctly paged. Note 1: How HLR gets MSRN when UE is in IMS is still an open issue. Note 2: the procedure figures involve many implementation details and should be an open issue. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | misalignment with Stage 2 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 24.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  | TS/TR ... CR ... rev ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... rev ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... rev ... |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

### 21.2.8	Procedure in the old VLR to request a Roaming Number (MTRF)

The MAP process in the old VLR for Mobile Terminating Roaming Forwarding is shown in figure 21.2/11. The MAP process invokes a macro not defined in this clause; the definition of this macro can be found as follows:

Receive_Open_Cnf			see subclause 25.1.2

Check_Confirmation		see subclause 25.2.2

On receiving MAP_SRI message from GMSC, the HLR should execute SDS, i.e. to check if the UE being terminated is located in CS or IMS and then decide to request VMSC or MGCF to provide MSRN. In that case, the UE could be correctly paged.