3GPP TSG CT4 Meeting #66bis	C4-141966

Sophia-Antipolis, FR; 20th – 24th October 2014	   Revision of C4-141696

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.334 | CR | 0051 | rev | 4 | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RTP transport multiplexing (Iq, stage 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RTCP-MUX |  |  |  |  |  |  | Date: |  | 2014-10-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | An UE may request usage of RTP transport multiplexing (via SIP signalling). When RTP transport multiplexing is supported by the IMS-AGW, then the IMS-ALG needs to control RTP transport multiplexing during the creation of H.248 termination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of RTP transport multiplexing capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An UE would have to use the default of unmultiplexed RTP / RTCP, which double the number of required UDP transport connections, and could reduce the likelihood of successful NAT traversal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.16, 5.17.1, 5.17.2.2, 5.17.2.3, 5.17.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 23.334 CR #0039 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	ETSI TS 183 018 V3.5.1 (2009-07): "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile Version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification".

[4]	ITU-T Recommendation H.248.37 (06/2008): "Gateway control protocol: IP NAPT traversal package".

[5]	ITU-T Recommendation H.248.57 (06/2008): "Gateway control protocol: RTP Control Protocol Package".

[6]	ITU-T Recommendation H.248.43 (06/2008): "Gateway control protocol: Gate Management and Gate Control packages".

[7]	ITU-T Recommendation H.248.53 (03/2009): "Gateway control protocol: Traffic management packages".

[8]	ITU-T Recommendation H.248.41 Amendment 1 (06/2008): "Gateway control protocol: IP domain connection package: IP Realm Availability Package".

[9]	ITU-T Recommendation H.248.36 (09/2005): "Gateway control protocol: Hanging Termination Detection package".

[10]	ITU-T Recommendation H.248.1 (05/2002): "Gateway Control Protocol: Version 2" including the Corrigendum1 for Version 2 (03/04).

[11]	ITU-T Recommendation H.248.14 (03/2009): "Gateway control protocol: Inactivity timer package".

[12]	ITU-T Recommendation H.248.52 (06/2008): "Gateway control protocol: QoS support packages".

[13]	ITU-T Recommendation H.248.11 (11/2002): "Gateway control protocol: Media gateway overload control package". 
Inclusive Corrigendum 1 (06/2008) to H.248.11 " Gateway control protocol: Media gateway overload control package: Clarifying MG-overload event relationship to ADD commands".

[14]	ITU-T Recommendation H.248.10 (07/2001): "Media gateway resource congestion handling package".

[15]	IETF RFC 5234 (2008): "Augmented BNF for Syntax Specifications: ABNF".

[16]	IETF RFC 4960 (2007): "Stream control transmission protocol".

[17]	IETF RFC 4566 (2006): "SDP: Session Description Protocol".

[18]	IETF RFC 4975 (2007): "The Message Session Relay Protocol (MSRP)".

[19]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control".

[20]	IETF RFC 4145 (2005): "TCP-Based Media Transport in the Session Description Protocol (SDP)".

[21]	IETF RFC 3605 (2003): "Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP)".

[22]	ITU-T Recommendation X.690 (11/2008): "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

[23]	3GPP TS 23.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface: Procedures Descriptions".

[24]	ITU-T Recommendation H.248.40 (01/2007): "Gateway control protocol: Application Data Inactivity Detection package".

[25]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP) - Based Feedback (RTP/AVPF)".

[26]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[27]	3GPP TS 33.210: "Technical Specification Group Services and System Aspects;3G Security; Network Domain Security; IP Network Layer Security".

[28]	IETF RFC 3556 (2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth".

[29]	IETF RFC 4568 (2006): "Session Description Protocol (SDP) Security Descriptions for Media Streams".

[30]	IETF RFC 3711 (2004): "The Secure Real-time Transport Protocol (SRTP)".

[31]	IETF RFC 5124 (2008): "Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF)".

[32]	IETF RFC 2216 (1997): "Network Element Service Specification Template".

[33]	Supplement 7 to ITU-T H-series Recommendations H.Sup7 (05/2008):" Gateway control protocol: Establishment procedures for the H.248 MGC-MG control association".

[34]	3GPP TS 33.328: "IMS Media Plane Security".

[35]	Void

[36]	Void

[37]	Void

[38]	3GPP TS 23.237: "IP Multimedia subsystem (IMS) Service Continuity; Stage 2".

[39]	3GPP TS 22.153: "Multimedia Priority Service".

[40]	ITU-T Recommendation H.248.82 (03/2013): "Gateway control protocol: Explicit Congestion Notification Support".

[41]	IETF RFC 5285 (2008): "A General Mechanism for RTP Header Extensions".

[42]	IETF RFC 6236: "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)".

[43]	ITU-T Recommendation H.248.50 (09/2010) Corrigendum 1 (02/12): "Gateway control protocol: NAT traversal toolkit packages".

[44]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[45]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP".

[46]	ITU-T Recommendation H.248.84 (07/2012): "Gateway control protocol: NAT traversal for peer-to-peer services".

[47]	ITU-T Recommendation H.248.89 (07/2014): "Gateway control protocol: TCP support packages".

Editor's Note: The above document cannot be formally referenced until it is published as an ITU-T Recommendation.

[48]	ITU-T Recommendation H.248.90 (07/2014): "Gateway control protocol: H.248 packages for control of transport security using TLS".

Editor's Note: The above document cannot be formally referenced until it is published as an ITU-T Recommendation.

[49]	ITU-T Recommendation H.248.92 (07/2014): "Gateway control protocol: Stream endpoint interlinkage package".

Editor's Note: The above document cannot be formally referenced until it is published as an ITU-T Recommendation

[50]	ITU-T Recommendation H.248.93 (07/2014): "Gateway control protocol: H.248 packages for control of transport security using DTLS".

Editor's Note: The above document cannot be formally referenced until it is published as an ITU-T Recommendation.

[51]	IETF RFC 793: "Transmission Control Protocol – DARPA Internet Program – Protocol Specification".

[52]	IETF RFC 4582: "The Binary Floor Control Protocol (BFCP)".

[53]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[54]	IETF draft-schwarz-mmusic-sdp-for-gw-00: "SDP codepoints for gateway control".

Editor's Note: The above document cannot be formally referenced until it is published as an RFC.

[55]	IETF RFC 4572: "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[56]	Draft ITU-T Recommendation H.248.78 (07/2014): "Gateway control protocol: Bearer-level application level gateway".

Editor's Note: The above document is currently under revision by ITU-T.

[57]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[x]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

* * * Next Change * * * *

## 5.16	Optional support of SDP and Annex C information elements

Specifies what SDP attributes and Annex C information elements may be supported.

Table 5.16.1: Optional Annex C and SDP information elements

| Information Element | Annex C Support | SDP Support |
| --- | --- | --- |
| a-line | "SDP_A " | 1) Application "RTCP transport address control": a) Default mode "without RTP/RTCP transport multiplexing": The attribute "a=rtcp" line may either contain (a=rtcp: <port>) or (a=rtcp: <port> <network type> <address type> <connection address>) when the "a=" line is used for RTCP transport port and optionally network address transmission (see IETF RFC 3605 [21]). . The MGC shall supply the "a=rtcp" line in the RD when non-default RTCP network address or transport port values are used by the peer media entity. "RTCP transport address control" should be supported by MG. b) Optional extension mode "with RTP/RTCP transport multiplexing": The attribute "a=rtcp-mux" (see IETF RFC 5761 [x]) is used for indicating RTP/RTCP transport multiplexing. Tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5] define the appropriate RTCP port allocation rules. 2) Media related parameters in general: The "a=" line provides the complementary information for the "m=" line with regards to a specified media type/format (e.g. an optional SDP „a=ptime" line for a particular media format). For a dynamic RTP payload type, for each media information on the codec type shall be provided in a separate SDP "a=rtpmap"line and possibly additional SDP "a=fmtp"-line(s). 3) Application " Media interworking (transcoding)": See "a=" line specification in (2). Media interworking is limited to audio transcoding only. 4) IMS media plane security related parameters: 4.1) SRTP-specific security parameters: The attribute "a=crypto" (see IETF RFC 4568 [29]) shall be provided for an m-line in the local and remote descriptor of an access network termination if the IMS-ALG wants that the corresponding media is encrypted, decrypted and/or integrity protected by the IMS-AGW (IMS end-to-access-edge media plane security). For each m-line, only a single "a=crypto" attribute shall be provisioned (i.e. only information related to a single crypto suite is provisioned to the IMS-AGW). The "a=crypto" attribute may contain several master keys. An IMS-AGW supporting end-to-access-edge media plane security shall support parameters within the "a=crypto" attribute in accordance with the profile in Annex of 3GPP TS 33.328 [34]. 4.2) (D)TLS-specific security parameters: The attribute "a=fingerprint" (see IETF RFC 4572 [55]) shall be provided in accordance with ITU-T Recommendation H.248.90 [48] for an "m="-line in the local and remote descriptor of an access network termination if the IMS-ALG wants that the corresponding media is encrypted, decrypted and/or integrity protected by the IMS-AGW (IMS end-to-access-edge media plane security). 5) Coordination of Video Orientation The attribute "a=extmap" (see IETF RFC 5285 [41]) may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information, see also 3GPP TS 26.114 [26]. 6) Generic Image Attribute The attribute "a=imageattr" (see IETF RFC 6236 [42]) may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the generic image attributes, see also 3GPP TS 26.114 [26]. The local descriptor indicates the image sizes which the IMS-AGW supports in the receiving direction for the selected payload type and corresponds to the "recv" keyword (see IETF RFC 6236 [42]) in the "a=imageattr" that the IMS-ALG will send within the SDP body on the Mw/Mx interface. The remote descriptor indicates the image sizes which the IMS-AGW supports in the sending direction for the selected payload type and corresponds to the "send" keyword (see IETF RFC 6236 [42]) in the "a=imageattr" that the IMS-ALG will send within the SDP body on the Mw/Mx interface. 7) ICE support The attributes "a=candidate", "a=ice-pwd", and "a=ice-ufrag" (see IETF RFC 5245 [44]) may be provided for an SDP m-line in the local and remote descriptor if the IMS-AGW supports ICE, see also 3GPP TS 24.229 [45]. In the local descriptor, the IMS-ALG shall provide "a=ice-pwd", and "a=ice-ufrag" with wildcard sign "$" to request the allocation of a password and user name fragment, and the "a=candidate" of type "host" with the transport, port and priority parameters with wildcard sign "$" to request the allocation of a host candidate. The IMS-AGW shall then reply with completed "a=ice-pwd", and "a=ice-ufrag" and "a=candidate" attributes in the local descriptor, and shall include "a=ice-lite" if it only supports ICE lite. In the remote descriptor, the IMS-ALG may provide the "a=candidate", "a=ice-pwd", and "a=ice-ufrag". 8) state-agnostic and state-aware TCP handling: The attribute "a=setup" (see IETF RFC 4145 [20]) shall be provided for TCP-based media, in accordance with ITU-T Recommendation H.248.84 [46], when triggering an end-to-end TCP simultaneous open (leading to a TCP merge mode in the IMS-AGW) or other TCP modes of operation. 9) Application-aware interworking for MSRP traffic: The attribute "a=path" (see IETF RFC 4975 [11] shall be provided, when enabling a bearer level application gateway (B-ALG) function for MSRP traffic, according to ITU-T Recommendation H.248.78 [56]. |
| NOTE: Media Interworking is optional. |  |  |

Editor's Note:	The support for video transcoding is required for vSRVCC but should be changed from Rel-11, separate CRs would be required for this change.

* * * Next Change * * * *

### 5.17.1	Formats and Codes

Table 5.17.1.1 shows the parameters which are required for the procedures defined in the following clauses.

The coding rules applied in ITU-T Recommendation H.248.1 [10] for the applicable coding technique shall be followed for the UMTS capability set.

The binary encoding rules which are applicable to the defined Abstract Syntaxes are the Basic Encoding Rules for Abstract Syntax Notation One, defined in ITU-T Recommendation X.690 [22].  Specifically in accordance with ITU-T Recommendation X.690 [22] section 7.3, alternative encodings based on the definite and indefinite form of length are permitted by the basic encoding rules as a sender's option. Receivers shall support both alternatives.

Unsupported values of parameters or properties may be reported by the IMS-AGW and shall be supported by the IMS-ALG as such by using H.248.1 error code #449 " Unsupported or Unknown Parameter or Property Value ". The unsupported or unknown value is included in the error text in the error descriptor.

Table 5.17.1.1: Information Elements Used in Procedures

| Signalling Object | H.248 Descriptor | Coding |
| --- | --- | --- |
| Alternate MGC Id | ServiceChange | The MGCIdToTry parameter in ITU-T Recommendation H.248.1 [10]. |
| Available Realms | Termination State | According to Available Realms property in ITU-T Recommendation H.248.41 [8]. |
| Application-aware MSRP interworking request | LocalControl | This is the ptbalg property from ITU-T Recommendation H.248.78 [56] concerning the configuration of a B-ALG service (for MSRP traffic). |
| BNC Release | Events, ObservedEvents | As for the Events/ObservedEvents Descriptor in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "Cause" |
| Cause | ObservedEvents | As for the ObservedEvent Parameter in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "General cause" |
| Changed Realms | Observed Events | According to Observed Events Parameters for Available Realms Changed event in ITU-T Recommendation H.248.41 [8]. |
| Codec List | Local Descriptor or Remote Descriptor | <fmt list> in a single SDP m-line. For a static RTP payload type, the codec type should be implied by the RTP payload type, if not then each codec type shall be provided in a separate SDP "a=rtpmap"-line and possibly additional SDP "a=fmtp"-line(s). For a dynamic RTP payload type, for each codec information on the codec type shall be provided in a separate SDP "a=rtpmap"-line and possibly additional SDP "a=fmtp"-line(s). |
| Connectivity Mode | LocalControl | ITU-T Recommendation H.248.1 [10] Mode property. Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 Annex A [10] "streamMode" Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 Annex B [10] "streamMode". |
| Context ID | NA | Binary Encoding: As per ITU-T Recommendation H.248.1 [10] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [10] Annex B. |
| Cryptographic SDES Attribute | Local Descriptor or Remote Descriptor | "crypto" attribute in SDP a-line as defined in IETF RFC 4568 [29], see 5.16 |
| Delay Variation Tolerance | LocalControl | This is the tman/dvt property from ITU-T Recommendation H.248.53 [7]. |
| Diffserv Code Point | LocalControl | Defined according to the Differentiated Services Code Point property in ITU-T Recommendation H.248.52 [12]. |
| Diffserv Tagging Behaviour | LocalControl | Defined according to the Tagging Behaviour property in ITU-T Recommendation H.248.52 [12]. |
| Discard Incoming TCP Connection Establishment Requests Indicator | LocalControl | Defined according to the Incoming bearer connection establishment blocking property (tcpbcc/bceb) in ITU-T Recommendation H.248.89 [47]. |
| ECN Enabled | Local Descriptor or Remote Descriptor | Defined according to the "ECN Enabled" property in ITU-T Recommendation H.248.82 [40]. |
| ECN Failure | Events, Observed Events | Defined according to the "ECN Failure" Event in ITU-T Recommendation H.248.82 [40]. |
| ECN Failure Type | ObservedEvents Descriptor | As for the ObservedEventsDescriptor Parameter "Failure Type" in ITU-T Recommendation H.248.82 [40]. |
| ECN Initiation Method | Local Descriptor or Remote Descriptor | Defined according to "Initiation Method" property in ITU-T Recommendation H.248.82 [40]. |
| Emergency Call Indication | NA | ITU-T Recommendation H.248.1 [10] 6.1.1 Emergency Call Indicator Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex A "Emergency” context attribute Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex B " EmergencyToken” context attribute |
| Establish (D)TLS session | Signals | Defined according to the Establish BNC signal (tlsbsc/EstBNC) in ITU-T Recommendation H.248.90 [48]. |
| Extended Header For CVO | Local Descriptor or Remote Descriptor | "extmap" attribute in SDP a-line as defined in IETF RFC 5285 [41], see 5.16 |
| Forward Incoming TCP Connection Establishment Requests Indicator | LocalControl | Defined according to the Interlinkage topology property (seplink/linktopo) in ITU-T Recommendation H.248.93 [50]. |
| Generic Image Attribute | Local Descriptor or Remote Descriptor | "imageattr" attribute in SDP a-line as defined in IETF RFC 6236 [46], see table 5.16.1. |
| ICE host candidate request | Local Descriptor | The "a=candidate" SDP attribute defined in IETF RFC 5245 [44] of type "host" with the transport, port and priority parameters with wildcard sign "$" to request the allocation of a host candidate |
| ICE host candidate | Local Descriptor | The "a=candidate" SDP attribute defined in IETF RFC 5245 [44] |
| ICE lite indication | Local Descriptor | The "a=ice-lite" SDP attribute defined in IETF RFC 5245 [44]. |
| ICE password request | Local Descriptor | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [44] with wildcard sign "$". |
| ICE password | Local Descriptor | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [44]. |
| ICE received candidate | Remote Descriptor | The "a=candidate" SDP attribute defined in IETF RFC 5245 [44] |
| ICE received password | Remote Descriptor | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [44]. |
| ICE received Ufrag | Remote Descriptor | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [44]. |
| ICE Ufrag request | Local Descriptor | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [44] with wildcard sign "$". |
| ICE Ufrag | Local Descriptor | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [44]. |
| ICE Connectivity Check Result | Events, Observed Events | Defined according to Connectivity Check Result event in ITU-T Recommendation H.248.50 [43]. |
| ICE Send Connectivity Check | Signals | Defined as the ostuncc/scc signal in ITU-T Recommendation H.248.50 [43]. |
| ICE New Peer Reflexive Candidate | Events, Observed Events | Defined according to New Peer Reflexive Candidate event in ITU-T Recommendation H.248.50 [43], only applicable for full ICE. |
| ICE Send Additional Connectivity Check | Signals | Defined as the ostuncc/sacc signal in ITU-T Recommendation H.248.50 [43], only applicable for full ICE. |
| Inactivity Timer | Events, Observed Events | Defined according to Inactivity Timeout event in ITU-T Recommendation H.248.14 [11]. |
| IP Address | Local Descriptor or Remote Descriptor | <connection address> in SDP "c-line" |
| IP Realm | LocalControl | According to IP Realm Identifier property in ITU-T Recommendation H.248.41 [8]. |
| IP Version | Local Descriptor or Remote Descriptor | <address type> in SDP "c-line", see 5.15 |
| Latching | Signals | This is the ipnapt/latch signal in ITU-T Recommendation H.248.37 [4]. |
| Local certificate fingerprint | Local Descriptor | "fingerprint" attribute in SDP "a="-line as defined in IETF RFC 4572 [55] see table 5.16.1. |
| Local certificate fingerprint Request | Local Descriptor | "fingerprint" attribute in SDP "a="-line as defined in IETF RFC 4572 [55] with wildcard choose "$". |
| Maximum Burst Size | LocalControl | This is the tman/mbs property from ITU-T Recommendation H.248.53 [7] |
| Media Inactivity Detection | Events, Observed Events | Defined according to ipstop event in ITU-T Recommendation H.248.40 [24]. |
| Media Inactivity Detection Time | Events | As for the Event Parameter in ITU-T Recommendation H.248.40 [24] "Detection Time" |
| Media Inactivity Detection Direction | Events | As for the Event Parameter in ITU-T Recommendation H.248.40 [24] "Direction" |
| Media Type | Local Descriptor or Remote Descriptor | <media> in SDP m-line "audio" or “video” or “-“ |
| MSRP Path | Remote Descriptor | The "a=path" SDP attribute defined in IETF RFC 4975 [18]. |
| Notify (D)TLS session establishment Failure Event | ObservedEvents | As for the ObservedEvent Parameter in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "General cause" |
| Notify TCP Connection Establishment Failure Event | ObservedEvents | As for the ObservedEvent Parameter in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "General cause" |
| Overload Notification | Events, ObservedEvents | This is the chp/mgcon event from ITU-T Recommendation H.248.10 [14] or the ocp/mg_overload event from ITU-T Recommendation H.248.11 [13]. |
| Peak Data Rate | LocalControl | This is the tman/pdr property from ITU-T Recommendation H.248.53 [7]. |
| Policing Required | LocalControl | This is the tman/pol property from ITU-T Recommendation H.248.53 [7]. |
| Port | Local Descriptor or Remote Descriptor | <port> in SDP m-line. |
| Priority Information | NA | Priority Indicator (subclause 6.1.1 of ITU-T Recommendation H.248.1 [10]) Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex A "priority" context attribute Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex B "priority" context attribute |
| Realm Availability Change | Events, Observed Events | According to Available Realms Changed event in ITU-T Recommendation H.248.41 [8]. |
| Reduction | ObservedEvent Descriptor | As for the ObserverdEventDescriptor in subclause 4.2.1/ ITU-T Recommendation H.248.10 [14] "MGCongestion". |
| Release (D)TLS session | Signals | Defined according to the Release BNC signal (tlsbsc/RelBNC) in ITU-T Recommendation H.248.90 [48]. |
| Remote certificate fingerprint | Remote Descriptor | "fingerprint" attribute in SDP "a="-line as defined in IETF RFC 4572 [55], see table 5.16.1. |
| Remote Source Address Filtering | LocalControl | Defined according to Remote Source Address Filtering property in ITU-T Recommendation H.248.43 [6]. |
| Remote Source Address Mask | LocalControl | Defined according to Remote Source Address Mask property in ITU-T Recommendation H.248.43 [6]. |
| Remote Source Port Filtering | LocalControl | Defined according to Remote Source Port Filtering property in ITU-T Recommendation H.248.43 [6]. |
| Remote Source Port | LocalControl | Defined according to Remote Source Port property in ITU-T Recommendation H.248.43 [6]. |
| Remote Source Port Range | LocalControl | Defined according to Remote Source Port Range property in ITU-T Recommendation H.248.43 [6]. |
| Reserve_Value | LocalControl | ITU-T Recommendation H.248.1 [10] Reserve property. Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex A "reserveValue " Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex B "reservedValueMode". |
| ROOT Properties | Termination State | The properties in subclause E.2.1/ ITU-T Recommendation H.248.1 [10] |
| RTCP allocation | Local Control | Defined according to RTCP Allocation Specific Behaviour property in ITU-T Recommendation H.248.57 [5]. |
| RTP/RTCP transport multiplexing | Local Descriptor or Remote Descriptor | The SDP attribute "a=rtcp-mux" according to IETF RFC 5761 [x]. |
| RtcpbwRR | Local Descriptor or Remote Descriptor | <bandwidth> in SDP "b:RR"-line. see 5.15 |
| RtcpbwRS | Local Descriptor or Remote Descriptor | <bandwidth> in SDP "b:RS"-line. see 5.15 |
| Rtpbw | Local Descriptor or Remote Descriptor | <bandwidth> in SDP "b:AS"-line. see 5.15 |
| RTPpayload | Local Descriptor or Remote Descriptor | <fmt list> in SDP m-line. This may be set to CHOOSE ($) in a LD sent from the IMS-ALG toward the IMS-AGW. |
| Send TCP Connection Establishment Requests Indicator | Signals | Defined according to the Establish BNC signal (tcpbcc/EstBNC) in ITU-T Recommendation H.248.89 [47]. |
| Stream Number | Stream | Encoding as per ITU-T Recommendation H.248.1 [10] Annex B "Stream"/"ST". For a single stream, this may be omitted by the IMS-ALG. |
| STUN server request | LocalControl | Encoding as per ITU-T Recommendation H.248.50 [43] "MG Act-as STUN Server" (mgastuns) package "Act-as STUN Server" (astuns, 0x0001) property. |
| Sustainable Data Rate | LocalControl | This is the tman/sdr property from ITU-T Recommendation H.248.53 [7]. |
| TCP State-aware Handling Indicator and Setup Direction | Signals, Events or LocalControl | The "a=setup" SDP attribute as per subclause 13.5.1 of ITU-T Recommendation H.248.84 [46]. |
| Termination heartbeat | Events ObservedEvents | As per Termination Heartbeat defined in ITU-T Recommendation H.248.36 [9] Clause 5.2.1. |
| Termination ID | NA | Binary Encoding: As per ITU-T Recommendation H.248.1 [10] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [10] Annex B. |
| Transaction ID | NA | Binary Encoding: As per ITU-T Recommendation H.248.1 [10] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [10] Annex B. |
| Transport | Local Descriptor or Remote Descriptor | <transport> in SDP m-line, see 5.15 |

* * * Next Change * * * *

#### 5.17.2.2	Reserve AGW Connection Point

The IMS-ALG sends an ADD request command as in Table 5.17.2.2.1.

Table 5.17.2.2.1: Reserve AGW Connection Point Request

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| Local Descriptor { Port = $ IP Address = $ IP Version = IPv4 or IPv6 } | Transaction ID = x If Context Requested: Context ID= $ If Emergency Call: Emergency Call Indication If MPS call/session: Priority Indicator = x If Context Provided: Context ID = c1 Termination ID = $ If Stream Number specified:- Stream Number If Resources for multiple Codecs required: Reserve_Value If IP Interface Type: IP interface = "IP interface type" If indication on Bearer Released requested: NotificationRequested (Event ID = x, "BNC Release") If diffserv required:- Diffserv Code Point If tagging behaviour Diffserv Tagging Behaviour If Remote Source Address Filtering required:- Remote Source Address Filtering If Remote Source Address range required: Remote Source Address Mask If Remote Source Port Filtering required:- Remote Source Port Filtering If individual port: Remote Source Port If range of ports Remote Source Port Range NotificationRequested (Event ID = x, "termination heartbeat") If IP Realm specified:- IP Realm If Latching Required:- Latching If Sustainable Data Rate Policing Required:- Policing Required Sustainable Data Rate Maximum Burst Size If Peak Data Rate Policing Required: Policing Required Peak Data Rate If Delay Variation Required Delay Variation Tolerance If Media Inactivity Detection Required: NotificationRequested (Event ID = x, "Media Inactivity Detection( Media Inactivity Detection Time, Media Inactivity Detection Direction) ") (NOTE 1) If RTCP handling required: RTCP allocation If ECN transparent support required: ECN Enable = "True" Initiation Method = "inactive" If ECN Endpoint support required ECN Enable = "True" Initiation Method = "ECN Initiation Method" (NOTE 2) If notification of ECN Failure Report: NotificationRequested (Event ID = x,"ECN Failure") If ICE is applied: STUN server request If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If Discard Incoming TCP connection establishment request required: Discard Incoming TCP Connection Establishment Requests Indicator If Forward Incoming TCP connection establishment request required: Forward Incoming TCP Connection Establishment Requests Indicator If indication on TCP connection establishment failure requested: NotificationRequested (Event ID = x, "TCP connection establishment failure") If (D)TLS session establishment required: Establish (D)TLS session If indication on (D)TLS session establishment failure requested: NotificationRequested (Event ID = x, "(D)TLS session establishment failure") If media is "message": If B-ALG for MSRP required: Application-aware MSRP interworking request | Local Descriptor { If media is "audio" or "video": Codec List = Codec List RTP Payloads = RTP Payload Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE x1) If IMS media plane security required: Cryptographic SDES Attribute If media is "video": If CVO required: Extended Header For CVO (NOTE3) If imageattr negotiation: Generic Image Attribute (NOTE 4) If ICE is applied: ICE host candidate request ICE password request ICE Ufrag request If media is "message" or "application" or "-": If IMS media plane security required: Local certificate fingerprint Request } |
| NOTE 1: The event parameters "Media Inactivity Detection Time" and "Media Inactivity Detection Direction" are optional. NOTE 2: This shall be set to a value other than "inactive". See Table 5.14.3.15.1. NOTE 3: If the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information it shall pass any received extended RTP header with CVO bits on to outgoing RTP streams. If the IMS-AGW is transcoding between video payloads and it supports the extended RTP header with Coordination of Video Orientation information it shall convey received RTP header bytes on the outgoing RTP stream after transcoding associated packets as specified in 3GPP TS 26.114 [26], subclause 7.4.5. NOTE 4: The support of the generic image attributes is optional for the IMS-AGW. The list of image sizes per payload type supported by the IMS-AGW is preconfigured in the IMS-ALG. If none of the image sizes received within an SDP body on Mx/Mw interface is supported by the IMS-AGW then the IMS-ALG will not send the generic image attribute parameter to the IMS-AGW. NOTE x1: This element is optional. The RTCP port allocation rules are specified in tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5]. |  |  |

On reserving the termination, the IMS-AGW responds as in Table 5.17.2.2.2.

Table 5.17.2.2.2: Reserve AGW Connection Point Acknowledge

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| Local Descriptor { Port IP Address IP Version } | Transaction ID = x Context ID = C1 Termination ID = T1 Stream Number | Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute If ICE is applied: ICE host candidate ICE password ICE Ufrag If ICE lite implementation ICE lite indication If media is "message" or "application" or "-": If Local certificate fingerprint was requested: Local certificate fingerprint } |

* * * Next Change * * * *

#### 5.17.2.3	Configure AGW Connection Point

This procedure is used to configure the AGW connection point during session establishment or to reconfigure it during session establishment or after the session is established

The IMS-ALG sends a MODIFY request command as in Table 5.17.2.3.1.

Table 5.17.2.3.1: Configure AGW Connection Point Request

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| If local resources are modified: Local Descriptor { Port IP Address IP Version } If remote resources are modified: Remote Descriptor { Port IP Address IP Version } | Transaction ID = x Context ID = C1 Termination ID = T1 If MPS priority is modified: Priority Indicator = x (NOTE 4) If Stream Number specified: Stream Number If Resources for multiple Codecs required: Reserve_Value If diffserv required:- Diffserv Code Point If tagging behaviour Diffserv Tagging Behaviour If Remote Source Address Filtering required:- Remote Source Address Filtering If Remote Source Address range required: Remote Source Address Mask If Remote Source Port Filtering required:- Remote Source Port Filtering If individual port: Remote Source Port If range of ports Remote Source Port Range NotificationRequested (Event ID = x, "termination heartbeat") If IP Realm specified:- IP Realm (NOTE 1) If Latching Required:- Latching If Sustainable Data Rate Policing Required:- Policing Required Sustainable Data Rate Maximum Burst Size If Peak Data Rate Policing Required: Policing Required Peak Data Rate If Delay Variation Required Delay Variation Tolerance If Media Inactivity Detection Required: NotificationRequested (Event ID = x, "Media Inactivity Detection( Media Inactivity Detection Time, Media Inactivity Detection Direction)") (NOTE 2) If RTCP handling required: RTCP allocation If ECN transparent support required: ECN Enable = "True" Initiation Method = "inactive" If ECN Endpoint support required ECN Enable = "True" Initiation Method = "ECN Initiation Method" (NOTE 3) If notification of ECN Failure Report: NotificationRequested (Event ID = x,"ECN Failure") If full ICE is applied: Send Connectivity Check ("Control") If notification of ICE Connectivity Check Result Report: NotificationRequested (Event ID= xx, "Connectivity Check Result") If notification of New Peer Reflexive Candidate: NotificationRequested (Event ID = xy,"New Peer Reflexive Candidate") Send Additional Connectivity Check ("Control") If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If Discard Incoming TCP connection establishment request required: Discard Incoming TCP Connection Establishment Requests Indicator If Forward Incoming TCP connection establishment request required: Forward Incoming TCP Connection Establishment Requests Indicator If TCP connection establishment required: Send TCP Connection Establishment Request Indicator If indication on TCP connection establishment failure requested: NotificationRequested (Event ID = x, "TCP connection establishment failure") If (D)TLS session establishment required: Establish (D)TLS session If indication on (D)TLS session establishment failure requested: NotificationRequested (Event ID = x, "(D)TLS session establishment failure") If (D)TLS session release required: Release (D)TLS session If media is "message": If B-ALG for MSRP required: Application-aware MSRP interworking request | If local resources are modified: Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE x1) If IMS media plane security required: Cryptographic SDES Attribute If media is "video": If CVO required: Extended Header For CVO (NOTE 5) If imageattr negotiation: Generic Image Attribute (NOTE 6) } If remote resources are modified: Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE x1) If IMS media plane security required: Cryptographic SDES Attribute If media is "message" or "application" or "-": If IMS media plane security required: Remote certificate fingerprint If media is "video": If CVO required: Extended Header For CVO (NOTE 5) If imageattr negotiation: Generic Image Attribute (NOTE 6) If media is "message": If B-ALG for MSRP required: MSRP Path If ICE is applied: ICE received candidate ICE received password ICE received Ufrag (NOTE 7) } |
| NOTE 1: This can only be set to the same realm as at the reservation stage. If a different realm is specified, the IMS-AGW shall return error 501 “Not Implemented”. NOTE 2: The event parameters "Media Inactivity Detection Time" and "Media Inactivity Detection Direction" are optional. NOTE 3: This shall be set to a value other than "inactive". See Table 5.14.3.15.1. NOTE 4: The support of the modification of the Priority Indicator value is optional for the IMS-AGW and depends on implementation solution for Priority call/session authorisation (see 3GPP TS 23.334 [23]). NOTE 5: If the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information it shall pass any received extended RTP header with CVO bits on to outgoing RTP streams. If the IMS-AGW is transcoding between video payloads and it supports the extended RTP header with Coordination of Video Orientation information it shall convey received RTP header bytes on the outgoing RTP stream after transcoding associated packets as specified in 3GPP TS 26.114 [26], subclause 7.4.5. NOTE 6: The support of the generic image attributes is optional for the IMS-AGW. The list of image sizes per payload type supported by the IMS-AGW is preconfigured in the IMS-ALG. If none of the image sizes received within an SDP body on Mx/Mw interface is supported by the IMS-AGW then the IMS-ALG will not send the generic image attribute parameter to the IMS-AGW. NOTE 7: The support of ICE received candidate, ICE received password, ICE received Ufrag are optional for ICE lite, as specified in 3GPP TS 23.334 [23]. NOTE x1: This element is optional. The RTCP port allocation rules are specified in tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5]. |  |  |

The IMS-AGW responds as in Table 5.17.2.3.2.

Table 5.17.2.3.2: Configure AGW Connection Point Request Acknowledge

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| If local resources were provided in request: Local Descriptor { Port IP Address IP Version } If remote resources are provided in request: Remote Descriptor { Port IP Address IP Version } NOTE | Transaction ID = x Context ID = C1 Termination ID = T1 If Stream Number Specified: Stream Number | If local resources were provided in request: Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in request: Cryptographic SDES Attribute If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute } If remote resources are provided in request: Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute } NOTE |
| NOTE: Sending of the Remote Descriptor is optional. |  |  |

* * * Next Change * * * *

#### 5.17.2.4	Reserve and Configure AGW Connection Point

The IMS-ALG sends an ADD request command as in Table 5.17.2.4.1.

Table 5.17.2.4.1: Reserve and Configure AGW Connection Point Request

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| Local Descriptor { Port = $ IP Address = $ IP Version = IPv4 or IPv6 } Remote Descriptor { Port IP Address IP Version } | Transaction ID = x If Context Requested: Context ID = $ If Emergency Call: Emergency Call Indication If MPS call/session: Priority Indicator = x If Context Provided: Context ID = c1 Termination ID = $ If Stream Number Specified: Stream Number If Resources for multiple Codecs shall be reserved: Reserve_Value If IP Interface Type: IP interface = "IP interface type" If indication on Bearer Released requested: NotificationRequested (Event ID = x, "BNC Release") If diffserv required:- Diffserv Code Point If tagging behaviour Diffserv Tagging Behaviour If Remote Source Address Filtering required:- Remote Source Address Filtering If Remote Source Address range required: Remote Source Address Mask If Remote Source Port Filtering required:- Remote Source Port Filtering If individual port: Remote Source Port If range of ports Remote Source Port Range NotificationRequested (Event ID = x, "termination heartbeat") If IP Realm specified:- IP Realm If Latching Required:- Latching If Sustainable Data Rate Policing Required:- Policing Required Sustainable Data Rate Maximum Burst Size If Peak Data Rate Policing Required: Policing Required Peak Data Rate If Delay Variation Required Delay Variation Tolerance If Media Inactivity Detection Required: NotificationRequested (Event ID = x, "Media Inactivity Detection( Media Inactivity Detection Time, Media Inactivity Detection Direction)") (NOTE 1) If RTCP handling required: RTCP allocation If ECN transparent support required: ECN Enable = "True" Initiation Method = "inactive" If ECN Endpoint support required ECN Enable = "True" Initiation Method = "ECN Initiation Method" (NOTE 2) If notification of ECN Failure Report: NotificationRequested (Event ID = x,"ECN Failure") If ICE is applied: STUN server request If full ICE is applied Send Connectivity Check ("Control") If notification of ICE Connectivity Check Result Report: NotificationRequested (Event ID = xx, "Connectivity Check Result") If notification of New Peer Reflexive Candidate: NotificationRequested (Event ID = xy,"New Peer Reflexive Candidate") If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If Discard Incoming TCP connection establishment request required: Discard Incoming TCP Connection Establishment Requests Indicator If Forward Incoming TCP connection establishment request required: Forward Incoming TCP Connection Establishment Requests Indicator If indication on TCP connection establishment failure requested: NotificationRequested (Event ID = x, "TCP connection establishment failure") If (D)TLS session establishment required: Establish (D)TLS session If indication on (D)TLS session establishment failure requested: NotificationRequested (Event ID = x, "(D)TLS session establishment failure") If media is "message": If B-ALG for MSRP required: Application-aware MSRP interworking request | Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE x1) If IMS media plane security required: Cryptographic SDES Attribute If media is "video": If CVO required: Extended Header For CVO (NOTE 3) If imageattr negotiation: Generic Image Attribute (NOTE 4) If ICE is applied: ICE host candidate request ICE password request ICE Ufrag request If media is "message" or "application" or "-": If IMS media plane security required: Local certificate fingerprint Request } Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE x1) If IMS media plane security required: Cryptographic SDES Attribute If media is "video": If CVO required: Extended Header For CVO (NOTE 3) If imageattr negotiation: Generic Image Attribute (NOTE 4) If media is "message": If B-ALG for MSRP required: MSRP Path If ICE is applied: ICE received candidate ICE received password ICE received Ufrag (NOTE 5) If media is "message" or "application" or "-": If IMS media plane security required: Remote certificate fingerprint } |
| NOTE 1: The event parameters "Media Inactivity Detection Time" and "Media Inactivity Detection Direction" are optional. NOTE 2: This shall be set to a value other than "inactive". See Table 5.14.3.15.1. NOTE 3: If the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information it shall pass any received extended RTP header with CVO bits on to outgoing RTP streams. If the IMS-AGW is transcoding between video payloads and it supports the extended RTP header with Coordination of Video Orientation information it shall convey received RTP header bytes on the outgoing RTP stream after transcoding associated packets as specified in 3GPP TS 26.114 [26], subclause 7.4.5. NOTE 4: The support of the generic image attributes is optional for the IMS-AGW. The list of image sizes per payload type supported by the IMS-AGW is preconfigured in the IMS-ALG. If none of the image sizes received within an SDP body on Mx/Mw interface is supported by the IMS-AGW then the IMS-ALG will not send the generic image attribute parameter to the IMS-AGW. NOTE 5: The support of ICE received candidate, ICE received password, ICE received Ufrag are optional for ICE lite, as specified in 3GPP TS 23.334 [23]. NOTE x1: This element is optional. The RTCP port allocation rules are specified in tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5]. |  |  |

The IMS-AGW  responds as in Table 5.17.2.4.2.

Table 5.17.2.4.2: Reserve and Configure AGW Connection Point Request Acknowledge

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| Local Descriptor { Port IP Address IP Version } Remote Descriptor { Port IP Address IP Version } NOTE | Transaction ID = x Context ID = C1 Termination ID = T1 Stream Number | Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute If ICE is applied: ICE host candidate ICE password ICE Ufrag If ICE lite implementation ICE lite indication If media is "message" or "application" or "-": If Local certificate fingerprint was requested: Local certificate fingerprint } Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute } NOTE |
| NOTE: Sending of the Remote Descriptor is optional. |  |  |