3GPP TSG CT WG4 Meeting #85bis	C4-185487

Sophia Antipolis, France, 09st – 13th July 2018	was C4-185429

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0117 | rev | 2 | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Essential correction on UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | CUPS-CT |  |  |  |  |  |  | Date: |  | 2018-07-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IPv6 prefix delegation as specified 5.3.1.2.6 in TS 23.401 should be supported in Sx interface. The /64 default prefix is normally allocated via IPv6 stateless autoconfiguration, while when IPv6 prefix delegation is supported and used, a single network prefix shorter than the default /64 prefix may be assigned to a PDN connection, the remaining address space from the network prefix can be delegated to the PDN connection using prefix delegation after the default bearer establishment and IPv6 prefix allocation via IPv6 stateless address autoconfiguration. So when providing UE IP address in PDI, how many bits used for IPv6 delegation should be indicated. In addition, it should be clarified that when the UE IP address in a PDI is an IPv6 address, the UP function shall only use IPv6 Prefix of this IP address to match the packets and ignore the interface identifier part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an indication in UE IP address IE to indicate how many bits are used for IPv6 prefix delegation, and the UP function shall use IPv6 Prefix to match the packets when UE IP Address is present in a PDI. Add a new subclause in chapter 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous requirement may lead incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x, 8.2.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.x	Support IPv6 Prefix Delegation

Subclause 5.3.1.2.6 of 3GPP TS 23.401 [14] specifies the requirements for IPv6 Prefix Delegation via DHCPv6, that allow assigning a single network prefix shorter than the default /64 prefix to a PDN connection.

When assigning additional IPv6 prefixes (i.e. prefixes in addition to the default prefix) to a UE, the CP function shall provision/update the UE IP Address IE in the UP function with the IPv6D flag set to "1" and IPv6 Prefix Delegation Bits field to indicating the length of IPv6 Prefix for delegation.

* * * Next Change * * * *

### 8.2.62	UE IP Address

The UE IP Address IE type shall be encoded as shown in Figure 8.2.62-1. It contains a source or destination IP address.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 93 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  | IPv6D | S/D | V4 | V6 |  |
|  | m to (m+3) | IPv4 address |  |  |  |  |  |  |  |  |
|  | p to (p+15) | IPv6 address |  |  |  |  |  |  |  |  |
|  | r | IPv6 Prefix Delegation Bits |  |  |  |  |  |  |  |  |
|  | k to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.62-1: UE IP Address

The following flags are coded within Octet 5:

-	Bit 1 – V6: If this bit is set to "1", then the IPv6 address field shall be present in the UE IP Address, otherwise the IPv6 address field shall not be present.

-	Bit 2 – V4: If this bit is set to "1", then the IPv4 address field shall be present in the UE IP Address, otherwise the IPv4 address field shall not be present.

-	Bit 3 – S/D: This bit is only applicable to the UE IP Address IE in the PDI IE. It shall be set to "0" and ignored by the receiver in IEs other than PDI IE. In the PDI IE, if this bit is set to "0", this indicates a Source IP address; if this bit is set to "1", this indicates a Destination IP address.

-	Bit 4 – IPv6D: This bit is only applicable to the UE IP address IE in the PDI IE and when V6 bit is set to "1". If this bit is set to "1", then the IPv6 Prefix Delegation Bits field shall be present, otherwise the UP function shall consider IPv6 prefix is default /64.

-	Bit 5 to 8 Spare, for future use and set to 0.

Octets "m to (m+3)" or "p to (p+15)" (IPv4 address / IPv6 address fields), if present, shall contain the address value.

Octet r, if present, shall contain the number of bits is allocated for IPv6 prefix delegation, e.g. if /60 prefix is used, the value is set to "4". When using UE IP address IE in a PDI to match the packets, the UP function shall only use the IPv6 prefix part and ignore the interface identifier part.

* * * End of Changes * * * *