3GPP TSG-SA WG3 Meeting #91 	S3-181962

# Belgrade (Serbia), 16 - 20 April 2018        revison of 181487, merger of S3-181646 and S3-182034

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0046 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The granularity of NF service discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Nokia, Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-04-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | First, as described in 7.1.4 of TS 23.501: Due to roaming agreements and operator policies, a NF Service Consumer shall be authorised based on UE/subscriber/roaming information and NF type, the Service authorization may entail two steps: - Check whether the NF Service Consumer is permitted to discover the requested NF Service Producer instance during the NF service discovery procedure. This is performed on a per NF granularity by NRF. So the granularity of service discovery authorization should be per NF based. There is an Editor’s Note: Whether per UE subscription level authorization in NRF is needed is FFS. This contribution resovles this Editor’s Note by adding the sentence “The granularity of service discovery authorization is per NF based.” In additon, some reference number is missing in clause 13.4.1. The steps of authorization the client and generating the access token are combined at one step. What is more, it should implement recommendation from CT4 in S3-182012 At last, it should clarify the procedure of NF service consumer registration with NRF should be oauth2.0 client registration with authorization server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.The granularity of service discovery authorization is per NF based. The reference number missing is also added. 2.Remove the "Access token request during NF discovery" procedure, resulting additions to the independent token request procedure 3.Add oauth2.0 client after NF service consumer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The granularity of service discovery authorization is not determined. Some reference number is missing. CT4 recommendation will not be followed. It may cause confusion that NF as consumer needs to be registered with NRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.0, 13.4.1.1, 13.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start of Change 1 ****************

13.4	Authorization of NF service access

13.4.1	OAuth 2.0 based authorization of Network Function service access

13.4.1.0	General

The authorization framework uses the OAuth 2.0 framework as specified in [43]. Grants shall be of the type Client Credentials Grant, as described in clause 4.4. of [43]. Access tokens shall be JSON Web Tokens as described in [44] and are secured with digital signatures or Message Authentication Codes (MAC) based on JSON Web Signature (JWS) as described in [45].

The authorization framework described in clause 13.4.1 is mandatory to support for NRF and NF.

13.4.1.1	Service access authorization within the PLMN

OAuth 2.0 roles, as defined in clause 1.1 of [43], are as follows:

a.	The Network Resource Function (NRF) shall be the OAuth 2.0 Authorization server.

b.	The NF service consumer shall be the OAuth 2.0 client.

c.	The NF service producer shall be the OAuth 2.0 resource server.

OAuth 2.0 client (NF service consumer) registration with the OAuth 2.0 authorization server (NRF)

The NF service registration procedure, as defined in clause 4.17.1 of TS 23.502 [8], shall be used to register the OAuth 2.0 client (NF service consumer) with the OAuth 2.0 Authorization server (NRF), as described in clause 2.0 of [43]. The client id, used during OAuth 2.0 registration, shall be the NF Instance Id of the NF.

Access token request before service access

The following procedure describes how the NF service consumer obtains an access token before service access.

Pre-requisite:

a.	The NF Service consumer (OAuth2.0 client) is registered with the NRF (Authorization Server).

b.	The NRF and NF service producer share the required credentials when token validation is performed by the NF service producer.

c. The NRF and NF have mutually authenticated each other.

Figure 13.4.1.1-1: NF service consumer obtaining access token before NF service access

1. The NF service consumer requests an access token from the NRF in the same PLMN. The message it includes the NF Instance Id of the NF service consumer, expected NF service name, NF types of the expected NF instance and NF consumer.

2. If the NRF decides that NF service consumer can be authorized (based on the information sent by the NF service consumer in the access token request), the NRF shall generate an access token with appropriate claims included. The NRF shall digitally sign the generated access token based on a shared secret or private key as described in [45].

The claims in the token shall include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service consumer (subject), NF Instance Id of the NF Service Producer (audience), authorized services (scope) and expiration time (expiration).

Editor’s Note: Additional claims in the token are FFS.

3. The token is included in the response sent to the NF service consumer.

Service access request based on token verification

The following figure and procedure describe how authorization is performed during Service request of the NF service consumer.

Figure 13.4.1.1-2: NF service consumer requesting service access with an access token

Pre-requisite: The NF service consumer is in possession of a valid access token before requesting service access from the NF Service producer.

1.	The NF Service consumer requests service from the NF service producer. The NF Service Consumer includes the access token.

Editor’s Note: Additional parameters needed in step 1 are based on the included claims in the token and are FFS.

The NF Service consumer and NF service producer authenticate each other following clause 13.3.

2.	The NF Service producer verifies the token in one of the following ways:

-	Request the NRF to verify the token. The NRF verifies the token and responds back to the NF service producer.

-	The NF Service producer verifies the integrity of the token by checking the signature using the shared secret or NRF’s public key. If integrity check is successful, the NF Service producer verifies the claims in the token.

Editor’s Note: Details of verification of claims in the token are FFS.

3.	If the verification is successful, the NF Service producer executes the requested service and responds back to the NF Service consumer.

13.4.1.2	Service access authorization in roaming scenarios

In the roaming scenario, OAuth 2.0 roles are as follows:

a.	The visiting Network Resource Function (vNRF) shall be the OAuth 2.0 Authorization server for vPLMN and authenticates the NF service consumer.

b.	The home Network Resource Function (hNRF) shall be OAuth 2.0 Authorization server for hPLMN and generates the access token.

c.	The NF service consumer in the visiting PLMN shall be the OAuth 2.0 client.

d.	The NF service producer in the home PLMN shall be the OAuth 2.0 resource server.

OAuth 2.0 client (NF service consumer) registration with the OAuth 2.0 authorization server (NRF)

Same as in the non-roaming scenario in 13.4.1.1.

Obtaining access token independently before NF service access

The following procedure describes how the NF service consumer obtains an access token for use in the roaming scenario.

Figure 13.4.1.2-1: NF service consumer obtaining access token before NF service access (roaming)

Pre-requisite:

a.	The NF Service consumer (OAuth2.0 client) is registered with the NRF (Authorization Server).

b.	The NRF and NF service producer share the required credentials when token validation is performed by the NF service producer.

c.	The two NRFs have mutually authenticated each other.

1.	The NF service consumer invokes Access Token Request (NF Instance Id of the NF service consumer, Expected NF service Name, NF Type of the expected NF instance, NF type of the NF consumer, home and serving PLMN IDs) from NRF in the same PLMN.

2.	After successfully authentication of the client, the NRF in serving PLMN identifies the NRF in home PLMN (hNRF) based on the home PLMN ID, and requests token from hNRF as described in clause 4.17.5 of [8]. The vNRF forwards the parameters it obtained from the NF service consumer, including NF service consumer type, to the hNRF.

3.	If the hNRF decides that NF service consumer can be authorized, the hNRF shall generate an access token with appropriate claims included. The hNRF shall digitally sign the generated access token based on a shared secret or private key as described in [45].

The claims in the token shall include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service consumer (subject), NF Instance Id of the NF Service Producer (audience), authorized services (scope) and expiration time (expiration).

4.	The access token is included in Access Token Response message to the hNRF.

5.	The vNRF forwards the Access Token Response message to the NF service consumer.

Service access request based on token verification

Same as in the non-roaming scenario in 13.4.1.1.

*************** End of Change 1 **************