3GPP TSG-CT WG6 Meeting #110e	C6-220185

# E-Meeting, 22nd – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0939 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SOR-CMCI parameters without rule storage in USIM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales DIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCPSOR_CON |  |  |  |  |  | Date: |  |  | 2022-02-23 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CT1’s CR (C1-217162) on 3GPP TS 23.122 clarified the case for SOR-CMCI without rule. The HPLMN may update the SOR-CMCI in the USIM such that it contains no SOR-CMCI rules, in which case the UE behaviour described in subclause C.4.2 applies. Also the HPLMN may make the SOR-CMCI file in the USIM unavailable (see 3GPP TS 31.102 [40]). |  |  |  |  |  |  |  |  |
| Summary of change: |  | EFSOR-CMCI file content is now a data object with possibility to have : - SOR-CMCI wihout rule (zero length parameter stored) - SOR-CMCI rules up to 64Kb - no parameter stored |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No compliancy with 3GPP TS 23.112 on SOR-CMCI without rule |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.11.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | was C6-220125 |  |  |  |  |  |  |  |  |

***** Next change *****

#### 4.4.11.15	EFSOR-CMCI (Steering Of Roaming - Connected Mode Control Information)

If service n°138 is "available" in EFUST, this file shall be present.

This EF contains SOR-CMCI parameters as specified in 3GPP TS 24.501 [104]

| Identifier: '4F0E' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '0E' |  |  |  |  |  |  |
| File size: Z bytes (Z ≥ X) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | SOR-CMCI data object |  |  | O |  | X bytes |

-	SOR-CMCI data object

Coding of the SOR-CMCI data object

| Length | Description | Coding | Status |
| --- | --- | --- | --- |
| 1 | SOR-CMCI data object tag | ‘80’ | O |
| 1 to L (L ≤ 4) | SOR-CMCI data object length | As defined in TS 31.101 [11] for BER-TLV structured files | O |
| X-1-L bytes | SOR-CMCI parameters | As specified in TS 24.501 [104] clause 9.11.3.51 and start from octet (o+3) of SOR-CMCI as defined in Figure 9.11.3.51.7 of TS 24.501 [104]. | O |

As defined in 3GPP TS 23.122 [31] clause C.4.1, 'no SOR-CMCI rule' is defines when:

- SOR-CMCI data object is not present; or

- SOR-CMCI data object is zero length; or

- EFSOR-CMCI is deactivated

Then UE behaviour described in 3GPP TS 23.122 [31] clause C.4.2 applies.

***** End of changes *****