3GPP TSG-CT WG6 Meeting #123	C6-250440

# Göteborg, Sweden; 26th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.213 | CR | 0026 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GBA_U API additionnal test cases on GBAUCipher and GBAUSignature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEST_GBA_U_APIs |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During CT#100 was approved the new WID GBA_U Based APIs (UID 1000003) (CP-231107). During CT6#119 meeting was agreed the CR0102r2 on TS 31.130 (TDoc: C6-240313) defining the new GBA_U API, uicc.usim.gba_u package. During CT#105 was approved the new WID TEST_GBA_U_APIs (UID 1050008) (CP-242028) to complete the test coverages on on TS 31.130 uicc.usim.gba_u package. During CT6#121 was agreed the CR0022r5 on 3GPP TS 31.213 (C6-250107) including the first test cases to cover GBAUCipher class from uicc.usim.gba_u package on following methods: - getInstance (byte, Boolean) - doFinal(byte[], short, short, byte[], short) But clause 5.4.1.1 was not correct on source code name and Test coverage table. But method prototype in clause 5.4.2.1 includes type and parameters name which is not consistent with clause 5.4.1.1 which includes parameters type only: - The clause 5.4.2.1 shall be simplified to be aligned with 5.4.1.1, the parameters names shall be removed. But the clause numbering was not correct on 5.4.2.1.x and sub-clauses. - The numbering shall be changed to 5.4.1.2.x. But reference [13] is not properly voided in version 18.1.0 when requested in CR0022r5 Then reference [13] shall be voided. During CT6#121 was also agreed evolution in uicc.usim.gba_u package defined in 3GPP TS 31.130 (C6-250109) on GBAUCipher class defining two new methods (with new key length parameter): - init(byte theMode, byte[] adfAID, short adfAIDOff, short adfAIDLen, byte[] nafID, short nafIDOff, short nafIDLen, short keyLength) - init(byte theMode, byte[] adfAID, short adfAIDOff, short adfAIDLen, byte[] nafID, short nafIDOff, short nafIDLen, byte[] bArray, short bOff, short bLen, short keyLength) - on GBAUSignature class defining new methods with key length: - init(byte theMode, byte[] adfAID, short adfAIDOff, short adfAIDLen, byte[] nafID, short nafIDOff, short nafIDLen, short keyLength) - init(byte theMode, byte[] adfAID, short adfAIDOff, short adfAIDLen, byte[] nafID, short nafIDOff, short nafIDLen, byte[] bArray, short bOff, short bLen, short keyLength) These new methods shall be added in GBA_API acronyms list. Test cases shall be extended to cover the two new methods from GBAUCipher: - init(byte, byte[], short, short, byte[], short, short, short) - init(byte, byte[], short, short, byte[], short, short, byte[], short, short, short) Finally, 3GPP TS 31.213 test cases shall be extended to cover GBAUCipher addionnal following methods: - getInstance(byte, byte, boolean) - init(byte,byte[],short,short,byte[],short,short) - init(byte,byte[],short,short,byte[],short,short, byte[],short,short) Some test cases are conditional to support of: - CIPHER_SM4_CBC - CIPHER_SM4_ECB |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR includes correction on 5.4.2.1 method prototype by keeping only parameters type and removing parameters name. This CR includes correction on 5.4.2.1 and 5.4.2.1.x sub-clauses on numbering which shall be changed to 5.4.1.2 and 5.4.1.2.x. This CR includes reference [13] voiding. This CR includes test coverage on additonnal GBAUCipher methods on following new clauses: - 5.4.1.a: getInstance(byte, byte, boolean) - 5.4.b: init(byte, byte[], short, short, byte[], short, short) - 5.4.1.c: init(byte, byte[], short, short, byte[], short, short, byte[], short, short) - 5.4.1.d: init(byte, byte[], short, short, byte[], short, short, short) - 5.4.1.e: init(byte, byte[], short, short, byte[], short, short, byte[], short, short, short) Annex A.3.1 adds GBAUCipher new acronyms Ini3 and Ini4. Annex A.3.2 adds GBAUSignature new acronyms Ini3 and Ini4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test cases covering the GBA_U API (uicc.usim.gba_u package) from 3GPP TS 31.130, inconsistency on clauses numbering, missed voided reference [13]. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.1.1.x, 5.4.2.1.x renumbered to 5.4.1.2.x, 5.4.1.a (new) and sub-clauses, 5.4.1.b (new) and sub-clauses, 5.4.1.c (new) and sub-clauses, 5.4.1.d (new) and sub-clauses, 5.4.1.e (new) and sub-clauses, A.3.1, A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | was C6-250439 (CT6#123), was C6-250335 (CT6#122), was C6-250229 (CT6#122) |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 220: "Integrated Circuit Cards (ICC); ETSI numbering system for telecommunication; Application providers (AID)".

[2]	3GPP TS 31 130: "(U)SIM API for Java Card™"

[3]	Void.

[4]	3GPP TS 31.102: "Characteristics of the USIM Application".

[5]	3GPP TS 51.011 Release 4: "Specification of the Subscriber Identity Module- Mobile Equipment (SIM – ME) interface".

[6]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[7]	Void.

[8]	3GPP TS 31.111: "USIM Application Toolkit (USAT)".

[9]	Void.

[10]	3GPP TS 31.115: "Secured packet structure for the (U)SIM Toolkit applications".

[11]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[12]	void

[13]	void

[14]	void

[15]	ETSI TS 102 268 V7.3.0: "Test specification for UICC Application Programming Interface for Java Card (TM) ".

[16]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[17]	3GPP TS 31.101: "UICC-Terminal Interface, Physical and Logical Characteristics".

[18]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[19]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping (GBA)".

[20]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for subscriber certificates".

* * * Next Change * * * *

### 5.4.1	Class GBAUCipher

#### 5.4.1.1	Method 	getInstance (byte, boolean)

Test Area Reference: Api_3_Gci_Gin1

5.4.1.1.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public static GBAUCipher getInstance(byte algorithm,

boolean externalAccess)

throws javacard.security.CryptoException

5.4.4.1.1.1	Normal execution

-	CRRN1: The method shall return the single system instance of the GBAUCipher class.

-	CRRN2: The GBAUCipher is a Temporary JCRE Entry Point Object.

5.4.4.1.1.2	Parameter Errors

-	CRRP1: The method throws javacard.security.CryptoException with reason CryptoException.NO_SUCH_ALGORITHM when the requested algorithm is an asymmetric algorithm.

5.4.4.1.1.3	Context errors

No requirements.

5.4.1.1.2	Test area files

Test Source:	Test_Api_3_Gci_Gin1.java

Test Applet:	Api_3_Gci_Gin1.java

Cap File:	Api_3_Gci_Gin1.cap

5.4.1.1.3	Test coverage

| CRR number | Test case number |
| --- | --- |
| N1 | 1 |
| N2 | 1,2,3 |
| P1 | 4 |

* * * Next Change * * * *

#### 5.4.1.2	Method doFinal(byte[], short, short, byte[], short)

Test Area Reference: Api_3_Gci_Dfin

5.4.1.2.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public abstract short doFinal(byte[] inBuff,

short inOffset,

short inLength,

byte[] outBuff,

short outOffset)

throws javacard.security.CryptoException

5.4.1.2.1.1	Normal execution

-	CRRN1: The method shall return encrypted/decrypted output from all/last input data using Ks_int_NAF linked to NAF ID given in init()..

-	CRRN2: This method shall process any remaining input data buffered by one or more calls to the update() method as well as input data supplied in the inBuff parameter.

-	CRRN3: After a call to this method, this Cipher object shall be reset  to the state it was in when previously initialized via a call to init() methods. The initial vector(IV) used in AES, DES, Korean SEED and SM4 algorithms will be reset to 0.

-	CRRN4: When using block-aligned data (multiple of block size), if the input buffer, inBuff and the output buffer, outBuff are the same array, then the output data area must not partially overlap the input data area such that the input data is modified before it is used; if inBuff==outBuff and inOffset < outOffset < inOffset+inLength, incorrect output may result.

5.4.1.2.1.2	Parameter errors

-	CRRP1: The method throws java.lang.NullPointerException if inBuff or outOffset is null.

-	CRRP2: The method throws java.lang.ArrayIndexOutOfBoundsException if the check operation on inBuff or outOffset would cause access of data outside array bounds.

5.4.1.2.1.3	Context errors

-	CRRC1: The method throws javacard.security.CryptoException with the reason code CryptoException.UNINITIALIZED_KEY if the key (Ks_int_NAF key) is uninitialized.

-	CRRC2: The method throws javacard.security.CryptoException with the reason code CryptoException.INVALID_INIT if this GBAUCipher object is not initialized.

-	CRRC3: The method throws javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE if this GBAUCipher algorithm does not pad the message and the message is not block aligned.

-	CRRC4: The method throws javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE if this GBAUCipher algorithm does not pad the message and no input data has been provided in inBuff or via the update() method.

-	CRRC5: The method throws javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE if the decrypted data is not bounded by appropriate padding bytes.

5.4.1.2.2	Test area files

Test Source:	Test_Api_3_Gci_Dfin.java

Testcase_Api_3_Gci_Dfin_1.java

Test Applet:	Api_3_Gci_Dfin_1.java

Cap File:	Api_3_Gci_Dfin_1.cap

5.4.1.2.3	Test coverage

| CRR number | Test case number |
| --- | --- |
| N1 | 10 |
| N2 | 12 |
| N3 | 10 |
| N4 | 11 |
| P1 | 1,2 |
| P2 | 3,4 |
| C1 | 5 |
| C2 | 6 |
| C3 | 7 |
| C4 | 8 |
| C5 | 9 |

5.4.1.2.4	Test procedure

| Id | Description | API Expectation | APDU Expectation |
| --- | --- | --- | --- |
| 0 | The content of EF for Access Control to GBA_U_APIs include NAF_ID1 and the AID of app1, which means NAF_ID1 is corresponding to app1. |  |  |
|  | Parameter errors |  |  |
| 1 | - Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode of MODE_ENCRYPT. 5- Call doFinal() method, the inBuff is set to null, and the other parameters are correct. 6- Call init() method with theMode of MODE_DECRYPT. 7- Call doFinal() method, the inBuff is set to null, and the other parameters are correct.. | 5- Throw java.lang.NullPointerException 7- Throw java.lang.NullPointerException |  |
| 2 | - Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode of MODE_ENCRYPT. 5- Call doFinal() method, the outOffset is set to null, and the other parameters are correct. 6- Call init() method with theMode of MODE_DECRYPT. 7- Call doFinal() method, the outOffset is set to null, and the other parameters are correct. | 5- Throw java.lang.NullPointerException 7- Throw java.lang.NullPointerException |  |
| 3 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode of MODE_ENCRYPT. 5- Call doFinal() method, inOffset is equal to inbuff.length, and the other parameters are correct. 6- Call doFinal() method,inOffset is equal to -1, and the other parameters correct. 7- Call doFinal() method,inLength is equal to inbuff.length+1, and the other parameters are correct. 8- Call doFinal() method, inOffset is equal to 1, inLength is equal to inbuff.length, and the other parameters correct. 9- Call init() method with theMode of MODE_DECRYPT. 10- Call doFinal() method, inOffset is equal to the length of inbuff, and the other parameters are correct. 11- Call doFinal() method, inOffset is equal to -1, and other parameters are correct. 12- Call doFinal() method, inLength is equal to inbuff.length+1, and the other parameters are correct. 13- Call doFinal() method, inOffset is equal to 1, inLength is equal to inbuff.length, and the other parameters are correct. | Step 5 to 8 - Throw java.lang.ArrayIndexOutOfBoundsException Step 11 to 13 - Throw java.lang.ArrayIndexOutOfBoundsException |  |
| 4 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode of MODE_ENCRYPT. 5- Call doFinal() method, outOffset is equal to outbuff.length, and the other parameters are correct. 6- Call doFinal() method, outOffset is equal to -1, and the other parameters are correct. 7- Call doFinal() method, the length of the return value is equal to outbuff.length+1, and the other parameters are correct. 8- Call doFinal() method, outOffset is equal to 1, the length of the return value is equal to outbuff.length, and the other parameters are correct. 9- Call init() method with theMode of MODE_DECRYPT. 10- Call doFinal() method, the outOffset is equal to outbuff.length, and the other parameters are correct. 11- Call doFinal() method, the outOffset is equal to -1, and the other parameters are correct. 12- Call doFinal() method, the length of the return value is equal to outbuff.length+1, and the other parameters are correct. 13- Call doFinal() method, the outOffset is equal to 1, the length of the return value is equal to inbuff.length, and the other parameters are correct. | Step 5 to 8 - Throw java.lang.ArrayIndexOutOfBoundsException Step 11 to 13 - Throw java.lang.ArrayIndexOutOfBoundsException |  |
|  | Context errors |  |  |
| 5 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAFkey and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode of MODE_ENCRYPT. 5- Clear the NAFkey value. 6- Call doFinal() method. 7- Set the NAFkey value. 8- Call init() method with theMode of MODE_DECRYPT. 9- Call doFinal() method. | 6 - Throw javacard.security.CryptoException with the reason code CryptoException.UNINITIALIZED_KEY 9 - Throw javacard.security.CryptoException with the reason code CryptoException.UNINITIALIZED_KEY |  |
| 6 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create a GBAUCipher object instance. 3- Call doFinal() method. 4- Call init() method to initialize the GBAUCipher object with the key object with theMode value 0xFF. 5- Call doFinal() method. | 3-Throw javacard.security.CryptoException with the reason code CryptoException.INVALID_INIT. 5- Throw javacard.security.CryptoException with the reason code CryptoException.INVALID_INIT |  |
| 7 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method to initialize GBAUCipher object with key object, and theMode value is set to MODE_ENCRYPT. 5- Call doFinal() method, and the value of inLength is set to10. | 5- Throw javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE |  |
| 8 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method to initialize GBAUCipher object with key object, and theMode value is set to MODE_ENCRYPT. 5- Call doFinal() method, and the value of inLength is set to 0. 6- Call updata() method, and the value of inLength is set to 0. 7- Call doFinal() method, and the value of inLength is set to 0. | 5- Throw javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE 7- Throw javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE |  |
| 9 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method to initialize GBAUCipher object with key object, and theMode value is set to MODE_DECRYPT. 5- Call doFinal() method, and the value of inLength is equal to the value of the correct length minus one. 6- Call doFinal() method, and the value of inLength is equal to the value of the correct length plus one. 7-Call doFinal() method, and the value of inLength is set to 0. | 5- Throw javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE 6- Throw javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE 7- Throw javacard.security.CryptoException with the reason code CryptoException.ILLEGAL_USE |  |
|  | Normal execution |  |  |
| 10 | Note:The test of symmetric algorithm supports all key types, and when the algorithm supports padding, the value of input plaintext n is 1, and when the algorithm does not support padding, the block size of the input plaintext is n. 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode value of MODE_ENCRYPT,call dofinal() method to encrypt n-byte plaintext. Call init() method with theMode value of MODE_DECRYPT, call dofinal method to decrypt the encryption datas. 5- Call init() method with theMode value of MODE_ENCRYPT, call dofinal() method to encrypt n bytes plaintext, Call init() method with theMode value of MODE_ENCRYPT, call dofinal() method to encrypt n+6 bytes plaintext. 6- Call init() method with theMode value of MODE_ENCRYPT, call dofinal() method to encrypt n byte plaintext, call init() method with theMode value of MODE_ENCRYPT, call dofinal() method to encrypt the same n byte plaintext. | 4- The return value of dofinal is n, and the decryption result is consistent with the input plaintext. 5- Two return values of dofinal() method should be correct. 6- Two return values of dofinal() method should be same and correct. |  |
| 11 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode value of MODE_ENCRYPT(using block-aligned data), call dofinal() method to encrypt 8 bytes plaintext, inoffset=5, outoffset=1, inbuff=outbuff, call init() method with theMode value of MODE_DECRYPT, call dofinal() method to decrypt the encryption result, inoffset=5, outoffset=4, inbuff=outbuff. 5- Call init() method with theMode value of MODE_ENCRYPT(using block-aligned data), call dofinal() method to encrypt 8 bytes plaintext, inoffset=1, outoffset=18, inbuff=outbuff, call init() method with theMode value of MODE_DECRYPT, call dofinal() method to decrypt the encrypted result, inoffset=1, outoffset=18, inbuff=outbuff | 4- The decryption result of dofinal() method should be consistent with the plaintext. 5- The decryption result of dofinal() method should be consistent with the plaintext. |  |
| 12 | 1- Execute GBA_U Procedure with NAF_ID1 and select app1. 2- Create the NAF key and set the key value. 3- Create a GBAUCipher object instance. 4- Call init() method with theMode value of MODE_ENCRYPT, call update() method to accumulate 5 bytes of plaintext, call dofinal() method to encrypt n bytes of plaintext. 5- Call init() method with theMode value of MODE_DECRYPT, call update() method to accumulate 5 bytes of ciphertext, call dofinal() method to decrypt the encryption result. | 4- The return of dofinal() method should be correct. 5- The return value’ length of dofinal() method is n+5, and the decryption result should be consistent with the plaintext. |  |

#### 5.4.1.a	Method 	getInstance(byte,byte,boolean)

Test Area Reference: Api_3_Gci_Gin2

5.4.1.a.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public static GBAUCipher getInstance(byte cipherAlgorithm,

byte paddingAlgorithm,

boolean externalAccess)

throws javacard.security.CryptoException

5.4.1.a.1.1	Normal execution

-	CRRN1: The method shall return the single system instance of the GBAUCipher class.

-	CRRN2: The GBAUCipher is a Temporary JCRE Entry Point Object.

5.4.1.a.1.2	Parameter Errors

-	CRRP1: The method throws javacard.security.CryptoException with reason CryptoException.NO_SUCH_ALGORITHM if the at least one requested algorithm is not supported (e.g when the requested algorithm is an asymmetric algorithm).

5.4.1.a.1.3	Context errors

No requirements.

5.4.1.a..2	Test area files

Test Source:	Test_Api_3_Gci_Gin2.java

Test Applets:

Api_3_Gci_Gin2_CIPHER_AES_CBC.java (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Gin2_CIPHER_SM4_CBC.java (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Gin2_CIPHER_SM4_ECB.java (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Cap File:	Api_3_Gci_Gin2.cap

5.4.1.a.3	Test coverage

| CRR number | Test case number | Condition |
| --- | --- | --- |
| N1 | 1 |  |
| N2 | 1,2,3 |  |
| P1 | 4 |  |
| N1 | 5 | CIPHER_SM4_CBC algorithm is supported |
| N2 | 5,6,7 | CIPHER_SM4_CBC algorithm is supported |
| P1 | 8 | CIPHER_SM4_CBC algorithm is not supported |
| N1 | 9 | CIPHER_SM4_ECB algorithm is supported |
| N2 | 9,10,11 | CIPHER_SM4_ECB algorithm is supported |
| P1 | 12 | CIPHER_SM4_ECB algorithm is not supported |

5.4.1.a.4	Test procedure

| Id | Description | API Expectation | APDU Expectation |
| --- | --- | --- | --- |
| 1 | Call getInstance() method twice Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The returned objects shall be the same, |  |
| 2 | Call getInstance() mehod and verify returned object is an instance of GBAUCipher class. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The reference returned shall be an object instance of GBAUCipher class (check cast), |  |
| 3 | Call getInstance() mehod and verify the returned value is not null. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The reference returned shall not be null, |  |
| 4 | Call getInstance() mehod and verify that method throws javacard.security.CryptoException with reason CryptoException.NO_SUCH_ALGORITHM. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_RSA paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | CryptoException.NO_SUCH_ALGORITHM well throwns. |  |
| 5 | Call getInstance() method twice Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The returned objects shall be the same, |  |
| 6 | Call getInstance() mehod and verify returned object is an instance of GBAUCipher class. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The reference returned shall be an object instance of GBAUCipher class (check cast), |  |
| 7 | Call getInstance() mehod and verify the returned value is not null. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The reference returned shall not be null, |  |
| 8 | Call getInstance() mehod and verify that method throws javacard.security.CryptoException with reason CryptoException.NO_SUCH_ALGORITHM. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | CryptoException.NO_SUCH_ALGORITHM well throwns. |  |
| 9 | Call getInstance() method twice Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The returned objects shall be the same, |  |
| 10 | Call getInstance() mehod and verify returned object is an instance of GBAUCipher class. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The reference returned shall be an object instance of GBAUCipher class (check cast), |  |
| 11 | Call getInstance() mehod and verify the returned value is not null. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | The reference returned shall not be null, |  |
| 12 | Call getInstance() mehod and verify that method throws javacard.security.CryptoException with reason CryptoException.NO_SUCH_ALGORITHM. Parameters are set to: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL externalAccess = false | CryptoException.NO_SUCH_ALGORITHM well throwns. |  |

#### 5.4.1.b	Method 	init(byte,byte[],short,short,byte[],short,short)

Test Area Reference: Api_3_Gci_Ini1

5.4.1.b.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public abstract void init(byte theMode,

byte[] adfAID,

short adfAIDOff,

short adfAIDLen,

byte[] nafID,

short nafIDOff,

short nafIDLen)

throws GBAUException, javacard.security.CryptoException

5.4.1.b.1.1	Normal execution

-	CRRN1: The method shall not return Exception if adfAID exist and nafID is allowed for the applet AID

5.4.1.b.1.2	Parameter Errors

-	CRRP1: The method throws CryptoException.ILLEGAL_USE if theMode option is an undefined value

-	CRRP2: The method throws NullPointerException  if adfAID or nafID is null

-	CRRP3: The method throws ArrayIndexOutOfBoundsException if the check operation on adfAIDOff/adfAIDLen would cause access of data outside adfAID array bounds or the check operation on nafIDOff/nafIDLen would cause access of data outside nafID array bound

5.4.1.b.1.3	Context errors

-	CRRC1: The method throws GBAUException.GBA_U_BOOTSTRAP_NOT_DONE if GBA_U Bootstrap procedure was not performed

-	CRRC2: The method throws GBAUException.GBA_U_NAF_DERIVATION_NOT_DONE if GBA_U NAF derivation procedure was not performed

-	CRRC3: The method throws GBAUException.GBA_U_UNALLOWED_ACCESS if applet is not allowed to use API

-	CRRC4: The method throws GBAUException.GBA_U_INCORRECT_NAF_ID if applet provides a NAF ID (through nafID, nafIDOff and nafIDLen) which is not defined in its access condition

-	CRRC5: The method throws GBAUException.GBA_U_INCORRECT_ADF_AID if applet provides a ADF AID (through adfAID, adfAIDOff and adfAIDLen) which does not support GBA_U computation

5.4.1.b.2	Test area files

Test Source:	Test_Api_3_Gci_Ini1.java

Test Applet:

Api_3_Gci_Ini1_1_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini1_2_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini1_1_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini1_2_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini1_1_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Api_3_Gci_Ini1_2_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Cap File:	Api_3_Gci_Ini1.cap

5.4.1.b.3	Test coverage

| CRR number | Test case number | Condition |
| --- | --- | --- |
| N1 | 10 |  |
| P1 | 1 |  |
| P2 | 2, 3 |  |
| P3 | 4, 5 |  |
| C1 | 6 |  |
| C2 | 7 |  |
| C3 | 11 |  |
| C4 | 9 |  |
| C5 | 8 |  |
| N1 | 110 | CIPHER_SM4_CBC algorithm is supported |
| P1 | 101 | CIPHER_SM4_CBC algorithm is supported |
| P2 | 102, 103 | CIPHER_SM4_CBC algorithm is supported |
| P3 | 104, 105 | CIPHER_SM4_CBC algorithm is supported |
| C1 | 106 | CIPHER_SM4_CBC algorithm is supported |
| C2 | 107 | CIPHER_SM4_CBC algorithm is supported |
| C3 | 111 | CIPHER_SM4_CBC algorithm is supported |
| C4 | 109 | CIPHER_SM4_CBC algorithm is supported |
| C5 | 108 | CIPHER_SM4_CBC algorithm is supported |
| N1 | 210 | CIPHER_SM4_ECB algorithm is supported |
| P1 | 201 | CIPHER_SM4_ECB algorithm is supported |
| P2 | 202, 203 | CIPHER_SM4_ECB algorithm is supported |
| P3 | 204, 205 | CIPHER_SM4_ECB algorithm is supported |
| C1 | 206 | CIPHER_SM4_ECB algorithm is supported |
| C2 | 207 | CIPHER_SM4_ECB algorithm is supported |
| C3 | 211 | CIPHER_SM4_ECB algorithm is supported |
| C4 | 209 | CIPHER_SM4_ECB algorithm is supported |
| C5 | 208 | CIPHER_SM4_ECB algorithm is supported |

5.4.1.b.4	Test procedure

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 1 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode different from MODE_ENCRYPT and MODE_DECRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 2 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = null - adfAIDOff = 0 - adfAIDLen= 0 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 3 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = null - nafIDOff = 0 - nafIDLen = 0 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 4 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 + 1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 5 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 + 1 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 6 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 7 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF derivation not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 8 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID2 - adfAIDOff = 0 - adfAIDLen= length of adfAID2 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 9 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID2 - nafIDOff = 0 - nafIDLen = length of nafID2 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 10 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 11 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() from unauthorized applet instance. Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_CBC is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). |  |  |
| 101 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 102 | Same sequence as Test Case 2 but with step a using: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 103 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 104 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 105 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 106 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 107 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 108 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 109 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 110 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 111 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_ECB is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). |  |  |
| 201 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 202 | Same sequence as Test Case 2 but with step a using: cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 203 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 204 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 205 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 206 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 207 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 208 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 209 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 210 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 211 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

#### 5.4.1.c	Method 	init(byte,byte[],short,short,byte[],short,short, byte[],short,short)

Test Area Reference: Api_3_Gci_Ini2

5.4.1.c.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public abstract void init(byte theMode,

byte[] adfAID,

short adfAIDOff,

short adfAIDLen,

byte[] nafID,

short nafIDOff,

short nafIDLen,

byte[] bArray,

short bOff,

short bLen)

throws GBAUException, javacard.security.CryptoException

5.4.1.c.1.1	Normal execution

-	CRRN1: The method shall not return Exception if adfAID exist and nafID is allowed for the applet AID

5.4.1.c.1.2	Parameter Errors

-	CRRP1: The method throws CryptoException.ILLEGAL_USE if theMode option is an undefined value

-	CRRP2: The method throws NullPointerException  if adfAID or nafID or bArray is null

-	CRRP3: The method throws ArrayIndexOutOfBoundsException if the check operation on adfAIDOff/adfAIDLen would cause access of data outside adfAID array bounds or the check operation on nafIDOff/nafIDLen would cause access of data outside nafID array bound or the check operation on bOff/bLen would cause access of data outside bArray array bound

5.4.1.c.1.3	Context errors

-	CRRC1: The method throws GBAUException.GBA_U_BOOTSTRAP_NOT_DONE if GBA_U Bootstrap procedure was not performed

-	CRRC2: The method throws GBAUException.GBA_U_NAF_DERIVATION_NOT_DONE if GBA_U NAF derivation procedure was not performed

-	CRRC3: The method throws GBAUException.GBA_U_UNALLOWED_ACCESS if applet is not allowed to use API

-	CRRC4: The method throws GBAUException.GBA_U_INCORRECT_NAF_ID if applet provides a NAF ID (through nafID, nafIDOff and nafIDLen) which is not defined in its access condition

-	CRRC5: The method throws GBAUException.GBA_U_INCORRECT_ADF_AID if applet provides a ADF AID (through adfAID, adfAIDOff and adfAIDLen) which does not supported GBA_U computation

5.4.1.c.2	Test area files

Test Source:	Test_Api_3_Gci_Ini2.java

Test Applet:

Api_3_Gci_Ini2_1_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini2_2_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini2_1_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini2_2_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini2_1_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Api_3_Gci_Ini2_2_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Cap File:	Api_3_Gci_Ini2.cap

5.4.1.c.3	Test coverage

| CRR number | Test case number | Condition |
| --- | --- | --- |
| N1 | 12 |  |
| P1 | 1 |  |
| P2 | 2, 3, 4 |  |
| P3 | 5, 6, 7 |  |
| C1 | 8 |  |
| C2 | 9 |  |
| C3 | 13 |  |
| C4 | 10 |  |
| C5 | 11 |  |
| N1 | 112 | CIPHER_SM4_CBC algorithm is supported |
| P1 | 101 | CIPHER_SM4_CBC algorithm is supported |
| P2 | 102, 103, 104 | CIPHER_SM4_CBC algorithm is supported |
| P3 | 105, 106, 107 | CIPHER_SM4_CBC algorithm is supported |
| C1 | 108 | CIPHER_SM4_CBC algorithm is supported |
| C2 | 109 | CIPHER_SM4_CBC algorithm is supported |
| C3 | 113 | CIPHER_SM4_CBC algorithm is supported |
| C4 | 110 | CIPHER_SM4_CBC algorithm is supported |
| C5 | 111 | CIPHER_SM4_CBC algorithm is supported |
| N1 | 212 | CIPHER_SM4_ECB algorithm is supported |
| P1 | 201 | CIPHER_SM4_ECB algorithm is supported |
| P2 | 202, 203, 204 | CIPHER_SM4_ECB algorithm is supported |
| P3 | 205, 206, 207 | CIPHER_SM4_ECB algorithm is supported |
| C1 | 208 | CIPHER_SM4_ECB algorithm is supported |
| C2 | 209 | CIPHER_SM4_ECB algorithm is supported |
| C3 | 213 | CIPHER_SM4_ECB algorithm is supported |
| C4 | 210 | CIPHER_SM4_ECB algorithm is supported |
| C5 | 211 | CIPHER_SM4_ECB algorithm is supported |

5.4.1.c.4	Test procedure

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 1 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode different from MODE_ENCRYPT and MODE_DECRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 2 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = null - adfAIDOff = 0 - adfAIDLen= 0 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 3 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = null - nafIDOff = 0 - nafIDLen = 0 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 4 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = 0 - bArray = null - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 5 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 + 1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 6 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 + 1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 7 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector + 1 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 8 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 9 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF derivation not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 10 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID2 - adfAIDOff = 0 - adfAIDLen= length of adfAID2 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 11 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID2 - nafIDOff = 0 - nafIDLen = length of nafID2 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 12 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 13 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() from unauthorized applet instance. Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_CBC is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 101 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 102 | Same sequence as Test Case 2 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 103 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 104 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 105 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 106 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 107 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 108 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 109 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 110 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 111 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 112 | Same sequence as Test Case 12 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 113 | Same sequence as Test Case 13 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_ECB is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 201 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 202 | Same sequence as Test Case 2 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 203 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 204 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 205 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 206 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 207 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 208 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 209 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 210 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 211 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 212 | Same sequence as Test Case 12 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 213 | Same sequence as Test Case 13 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

#### 5.4.1.d	Method 	init(byte,byte[],short,short,byte[],short,short,short)

Test Area Reference: Api_3_Gci_Ini3

5.4.1.d.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public abstract void init(byte theMode,

byte[] adfAID,

short adfAIDOff,

short adfAIDLen,

byte[] nafID,

short nafIDOff,

short nafIDLen,

short keyLength)

throws GBAUException, javacard.security.CryptoException

5.4.1.d.1.1	Normal execution

-	CRRN1: The method shall not return Exception if adfAID exist and nafID is allowed for the applet AID

5.4.1.d.1.2	Parameter Errors

-	CRRP1: The method throws CryptoException.ILLEGAL_USE if theMode option is an undefined value

-	CRRP2: The method throws NullPointerException  if adfAID or nafID is null

-	CRRP3: The method throws ArrayIndexOutOfBoundsException if the check operation on adfAIDOff/adfAIDLen would cause access of data outside adfAID array bounds or the check operation on nafIDOff/nafIDLen would cause access of data outside nafID array bound

-	CRRP4: The method throws CryptoException.ILLEGAL_USE if keyLength is incompatible with algorithm defined in getInstance()

5.4.1.d.1.3	Context errors

-	CRRC1: The method throws GBAUException.GBA_U_BOOTSTRAP_NOT_DONE if GBA_U Bootstrap procedure was not performed

-	CRRC2: The method throws GBAUException.GBA_U_NAF_DERIVATION_NOT_DONE if GBA_U NAF derivation procedure was not performed

-	CRRC3: The method throws GBAUException.GBA_U_UNALLOWED_ACCESS if applet is not allowed to use API

-	CRRC4: The method throws GBAUException.GBA_U_INCORRECT_NAF_ID if applet provides a NAF ID (through nafID, nafIDOff and nafIDLen) which is not defined in its access condition

-	CRRC5: The method throws GBAUException.GBA_U_INCORRECT_ADF_AID if applet provides a ADF AID (through adfAID, adfAIDOff and adfAIDLen) which does not supported GBA_U computation

5.4.1.d.2	Test area files

Test Source:	Test_Api_3_Gci_Ini3.java

Test Applet:

Api_3_Gci_Ini3_1_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini3_2_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini3_1_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini3_2_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini3_1_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Api_3_Gci_Ini3_2_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Cap File:	Api_3_Gci_Ini3.cap

5.4.1.d.3	Test coverage

| CRR number | Test case number | Condition |
| --- | --- | --- |
| N1 | 11 |  |
| P1 | 1 |  |
| P2 | 2, 3 |  |
| P3 | 4, 5 |  |
| P4 | 6 |  |
| C1 | 7 |  |
| C2 | 8 |  |
| C3 | 12 |  |
| C4 | 10 |  |
| C5 | 9 |  |
| N1 | 111 | CIPHER_SM4_CBC algorithm is supported |
| P1 | 101 | CIPHER_SM4_CBC algorithm is supported |
| P2 | 102, 103 | CIPHER_SM4_CBC algorithm is supported |
| P3 | 104, 105 | CIPHER_SM4_CBC algorithm is supported |
| P4 | 106 | CIPHER_SM4_CBC algorithm is supported |
| C1 | 107 | CIPHER_SM4_CBC algorithm is supported |
| C2 | 108 | CIPHER_SM4_CBC algorithm is supported |
| C3 | 112 | CIPHER_SM4_CBC algorithm is supported |
| C4 | 110 | CIPHER_SM4_CBC algorithm is supported |
| C5 | 109 | CIPHER_SM4_CBC algorithm is supported |
| N1 | 211 | CIPHER_SM4_ECB algorithm is supported |
| P1 | 201 | CIPHER_SM4_ECB algorithm is supported |
| P2 | 202, 203 | CIPHER_SM4_ECB algorithm is supported |
| P3 | 204, 205 | CIPHER_SM4_ECB algorithm is supported |
| P4 | 206 | CIPHER_SM4_ECB algorithm is supported |
| C1 | 207 | CIPHER_SM4_ECB algorithm is supported |
| C2 | 208 | CIPHER_SM4_ECB algorithm is supported |
| C3 | 212 | CIPHER_SM4_ECB algorithm is supported |
| C4 | 210 | CIPHER_SM4_ECB algorithm is supported |
| C5 | 209 | CIPHER_SM4_ECB algorithm is supported |

5.4.1.d.4	Test procedure

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 1 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode different from MODE_ENCRYPT and MODE_DECRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 2 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = null - adfAIDOff = 0 - adfAIDLen= 0 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 3 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = null - nafIDOff = 0 - nafIDLen = 0 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 4 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 + 1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 5 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 + 1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 6 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode different from MODE_ENCRYPT and MODE_DECRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_DES | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 7 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 8 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF derivation not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 9 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID2 - adfAIDOff = 0 - adfAIDLen= length of adfAID2 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 10 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID2 - nafIDOff = 0 - nafIDLen = length of nafID2 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 11 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 12 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() from unauthorized applet instance. Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_CBC is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 101 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 102 | Same sequence as Test Case 2 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 103 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 104 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 105 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 106 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 107 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 108 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 109 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 110 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 111 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 112 | Same sequence as Test Case 12 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_ECB is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 201 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 202 | Same sequence as Test Case 2 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 203 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 204 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 205 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 206 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 207 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 208 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 209 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 210 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 211 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 212 | Same sequence as Test Case 12 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

#### 5.4.1.e	Method 	init(byte,byte[],short,short,byte[],short,short,byte[],short,short,short)

Test Area Reference: Api_3_Gci_Ini4

5.4.1.e.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public abstract void init(byte theMode,

byte[] adfAID,

short adfAIDOff,

short adfAIDLen,

byte[] nafID,

short nafIDOff,

short nafIDLen,

byte[] bArray,

short bOff,

short bLen,

short keyLength)

throws GBAUException, javacard.security.CryptoException

5.4.1.e.1.1	Normal execution

-	CRRN1: The method shall not return Exception if adfAID exist and nafID is allowed for the applet AID

5.4.1.e.1.2	Parameter Errors

-	CRRP1: The method throws CryptoException.ILLEGAL_USE if theMode option is an undefined value

-	CRRP2: The method throws NullPointerException  if adfAID or nafID or bArray is null

-	CRRP3: The method throws ArrayIndexOutOfBoundsException if the check operation on adfAIDOff/adfAIDLen would cause access of data outside adfAID array bounds or the check operation on nafIDOff/nafIDLen would cause access of data outside nafID array bound or the check operation on bOff/bLen would cause access of data outside bArray array bound

-	CRRP4: The method throws CryptoException.ILLEGAL_USE if keyLength is incompatible with algorithm defined in getInstance()

5.4.1.e.1.3	Context errors

-	CRRC1: The method throws GBAUException.GBA_U_BOOTSTRAP_NOT_DONE if GBA_U Bootstrap procedure was not performed

-	CRRC2: The method throws GBAUException.GBA_U_NAF_DERIVATION_NOT_DONE if GBA_U NAF derivation procedure was not performed

-	CRRC3: The method throws GBAUException.GBA_U_UNALLOWED_ACCESS if applet is not allowed to use API

-	CRRC4: The method throws GBAUException.GBA_U_INCORRECT_NAF_ID if applet provides a NAF ID (through nafID, nafIDOff and nafIDLen) which is not defined in its access condition

-	CRRC5: The method throws GBAUException.GBA_U_INCORRECT_ADF_AID if applet provides a ADF AID (through adfAID, adfAIDOff and adfAIDLen) which does not supported GBA_U computation

5.4.1.e.2	Test area files

Test Source:	Test_Api_3_Gci_Ini4.java

Test Applet:

Api_3_Gci_Ini4_1_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini4_2_CIPHER_AES_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_AES_CBC)

Api_3_Gci_Ini4_1_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini4_2_CIPHER_SM4_CBC.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_CBC)

Api_3_Gci_Ini4_1_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Api_3_Gci_Ini4_2_CIPHER_SM4_ECB.java  (cipherAlgorithm  paremeter set to Cipher.CIPHER_SM4_ECB)

Cap File:	Api_3_Gci_Ini4.cap

5.4.1.e.3	Test coverage

| CRR number | Test case number | Condition |
| --- | --- | --- |
| N1 | 13 |  |
| P1 | 1 |  |
| P2 | 2, 3, 4 |  |
| P3 | 5, 6, 7 |  |
| P4 | 8 |  |
| C1 | 9 |  |
| C2 | 10 |  |
| C3 | 14 |  |
| C4 | 11 |  |
| C5 | 12 |  |
| N1 | 113 | CIPHER_SM4_CBC algorithm is supported |
| P1 | 101 | CIPHER_SM4_CBC algorithm is supported |
| P2 | 102, 103, 104 | CIPHER_SM4_CBC algorithm is supported |
| P3 | 105, 106, 107 | CIPHER_SM4_CBC algorithm is supported |
| P4 | 108 | CIPHER_SM4_CBC algorithm is supported |
| C1 | 109 | CIPHER_SM4_CBC algorithm is supported |
| C2 | 110 | CIPHER_SM4_CBC algorithm is supported |
| C3 | 114 | CIPHER_SM4_CBC algorithm is supported |
| C4 | 111 | CIPHER_SM4_CBC algorithm is supported |
| C5 | 112 | CIPHER_SM4_CBC algorithm is supported |
| N1 | 213 | CIPHER_SM4_ECB algorithm is supported |
| P1 | 201 | CIPHER_SM4_ECB algorithm is supported |
| P2 | 202, 203, 204 | CIPHER_SM4_ECB algorithm is supported |
| P3 | 205, 206, 207 | CIPHER_SM4_ECB algorithm is supported |
| P4 | 208 | CIPHER_SM4_ECB algorithm is supported |
| C1 | 209 | CIPHER_SM4_ECB algorithm is supported |
| C2 | 210 | CIPHER_SM4_ECB algorithm is supported |
| C3 | 214 | CIPHER_SM4_ECB algorithm is supported |
| C4 | 211 | CIPHER_SM4_ECB algorithm is supported |
| C5 | 212 | CIPHER_SM4_ECB algorithm is supported |

5.4.1.e.4	Test procedure

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 1 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode different from MODE_ENCRYPT and MODE_DECRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 2 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacard.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = null - adfAIDOff = 0 - adfAIDLen= 0 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 3 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = null - nafIDOff = 0 - nafIDLen = 0 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 4 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = 0 - bArray = null - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 5 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 + 1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 6 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 + 1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 7 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector + 1 - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 8 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUCipher1.init() Parameters are set to: - theMode different from MODE_ENCRYPT and MODE_DECRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - keyLength = KeyBuilder.LENGTH_DES | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 9 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 10 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF derivation not yet performed - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 11 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID2 - adfAIDOff = 0 - adfAIDLen= length of adfAID2 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 12 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID2 - nafIDOff = 0 - nafIDLen = length of nafID2 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 13 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 14 | - Set GBAUCipher1 object to returned getInstance(). Parameters are set to: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_AES_CBC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - GBA_U bootstrap performed successfully - GBA_U NAF performed successfully with NAF_ID using nafID1 value - Call GBAUCipher1.init() from unauthorized applet instance. Parameters are set to: - theMode = MODE_ENCRYPT - adfAID = adfAID1 - adfAIDOff = 0 - adfAIDLen= length of adfAID1 - nafID = nafID1 - nafIDOff = 0 - nafIDLen = length of nafID1 - bArray = initVector - bOff = 0 - bLen = length of initVector - keyLength = KeyBuilder.LENGTH_AES_128 | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_CBC is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 101 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 102 | Same sequence as Test Case 2 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 103 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 104 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 105 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 106 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 107 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 108 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 109 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 110 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 111 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 112 | Same sequence as Test Case 12 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| &13 | Same sequence as Test Case 13 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 114 | Same sequence as Test Case 14 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_CBC | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

Following test sequences to be performed only if Cipher.CIPHER_SM4_ECB is supported.

| Id | Description | API Expectation | APDU Exp. |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 201 | Same sequence as Test Case 1 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 202 | Same sequence as Test Case 2 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 203 | Same sequence as Test Case 3 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 204 | Same sequence as Test Case 4 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. NullPointerException well throwns |  |
| 205 | Same sequence as Test Case 5 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 206 | Same sequence as Test Case 6 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 207 | Same sequence as Test Case 7 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. ArrayIndexOutOfBoundsException well throwns |  |
| 208 | Same sequence as Test Case 8 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. CryptoException.ILLEGAL_USE well throwns |  |
| 209 | Same sequence as Test Case 9 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_BOOTSTRAP_NOT_DONE |  |
| 210 | Same sequence as Test Case 10 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_NAF_DERIVATION_NOT_DONE |  |
| 211 | Same sequence as Test Case 11 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_ADF_AID |  |
| 212 | Same sequence as Test Case 12 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_INCORRECT_NAF_ID |  |
| 213 | Same sequence as Test Case 13 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. No exception thrown |  |
| 214 | Same sequence as Test Case 14 but with step a using: - cipherAlgorithm = javacardx.crypto.Cipher.CIPHER_SM4_ECB | GBAUCipher1 is a GBAUCipher instance. GBAUException well throwns with reason GBA_U_UNALLOWED_ACCESS |  |

* * * Next Change * * * *

## A.3.1	GBAUCipher method

| Method name | Acronyms |
| --- | --- |
| GBAUCipher getInstance(byte,boolean) | Gin1 |
| GBAUCipher getInstance(byte,byte,boolean) | Gin2 |
| void init(byte,byte[],short,short,byte[],short,short) | Ini1 |
| void init(byte,byte[],short,short,byte[],short,short, byte[],short,short) | Ini2 |
| void init(byte,byte[],short,short,byte[],short,short,short) | Ini3 |
| void init(byte,byte[],short,short,byte[],short,short,byte[],short,short,short) | Ini4 |
| short update(byte[],short,short,byte[],short) | Updt |
| short doFinal(byte[],short,short,byte[],short) | Dfin |
| byte getAlgorithm() | Galg |
| byte getCipherAlgorithm() | Gcal |
| byte getPaddingAlgorithm() | Gpal |

## A.3.2	GBAUSignature method

| Method name | Acronyms |
| --- | --- |
| GBAUSignature getInstance(byte,boolean) | Gin1 |
| GBAUSignature getInstance(byte,byte,boolean) | Gin2 |
| void init(byte,byte[],short,short,byte[],short,short) | Ini1 |
| void init(byte,byte[],short,short,byte[],short,short, byte[],short,short) | Ini2 |
| void init(byte,byte[],short,short,byte[],short,short,short) | Ini3 |
| void init(byte,byte[],short,short,byte[],short,short,byte[],short,short,short) | Ini4 |
| short update(byte[],short,short) | Updt |
| short sign(byte[],short,short,byte[],short) | Sign |
| boolean verify(byte[],short,short,byte[],short,short) | Veri |
| byte getAlgorithm() | Galg |
| byte getCipherAlgorithm() | Gcal |
| short getLength() | Glen |
| byte getMessageDigestAlgorithm() | Gmda |
| byte getPaddingAlgorithm() | Gpal |
| void setInitialDigest(byte[],short,short,byte[],short,short) | Sidi |
| short signPreComputedHash(byte[],short,short,byte[],short) | Spch |
| boolean verifyPreComputedHash(byte[],short,short,byte[],short,short) | Vpch |

* * * End of Changes * * * *