3GPP TSG-CT6 Meeting #123	C6-250464

# Stor-Göteborg, Sweden, 26th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0823 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TC 27.22.4.15; Expected Sequence 1.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 27.22.4.15 needs correction as in the expected sequence 1.30 on ”(PROVIDE LOCAL INFORMATION, CAG information list)” the step 9 in unnecessary for the purpose of the test case. Quote of 27.22.4.15.4.1 Expected Sequence 1.30: As seen in the picture above, the step 9 has the purpose of, i.e., ”[..] such the UE does not perform reselection to Cell 2”. However, the after step 8, the UE is still in RRC CONNECTED mode and the ”Cell 2” belongs to another PLMN, so the UE does not perform reselection regardless the power level is low or high. Even more, the UE may not even start intra/inter-frequency measurements in RRC CONNECTED mode without measurement configuration from the SS. Hence, camping on CAG Cell (i.e., ”Cell 1”) and receiving SIB1 (including CAG information list) and SIB10 (including CAG Human-readable network name list) from ”Cell 1” are enough for the UE to correctly send TERMINAL RESPONSE in the Step13 of the Expected Sequence 1.30. All this means, that the UE can pass test case even without the Step 9 which means that this step is unnecessary for the purpose of the test case. In short, the Step 9 should either be removed or at minimum, the text indicating the ”such the UE does not perform reselection to Cell 2” be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the Expected Sequence 1.30, the Step 9 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the test case 27.22.4.15.4.1 the Step 9 is unnecessary and the Comments text indicating that ”[..] such the UE does not perform reselection to Cell 2” is in fact incorrect. Hence, a conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 27.22.4.15.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

27.22.4.15.4.1	Initial conditions

*****skip unchanged part change *****

Expected Sequence 1.30 (PROVIDE LOCAL INFORMATION, CAG information list)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | Cell 1 and Cell 2 are disabled. |  |
| 2 | USER ME | ME is switched on | ME does not register to any cell and remains in no service state. |
| 3 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.30.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.30.1 |  |
| 6 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.30.1 | [Command performed successfully] |
| 7 | NG-SS | Cell 1 is enabled. |  |
| 8 | ME NG-SS | The ME successfully registers to Cell 1 |  |
| 9 | NG-SS | Cell 2 is enabled. | Cell 2 is powered up at lower power level. |
| 10 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.30.1 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.30.1 |  |
| 13 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.30.2 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.30.1

Logically:

Command details:

Command number:	1

Command type:		PROVIDE LOCAL INFORMATION

Command qualifier:	"16" CAG information list and the corresponding human-readable network name per

PLMN

Device identities:

Source device:		UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 16 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.30.1

Logically:

Command details:

Command number:	1

Command type:		PROVIDE LOCAL INFORMATION

Command qualifier:	"16" CAG information list and the corresponding human-readable network name per

CAG ID

Device identities:

Source device:		ME

Destination device:	UICC

Result:

General result:		ME currently unable to process command

Additional information on result:	No service

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 16 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.30.2

Logically:

Command details:

Command number:	1

Command type:		PROVIDE LOCAL INFORMATION

Command qualifier:	"16" CAG information list and the corresponding human-readable network name per

CAG ID

Device identities:

Source device:		ME

Destination device:	UICC

Result:

General Result:		Command performed successfully

CAG Cell selection status:

Byte 1, general information:		'01' (camped on a CAG cell)

Byte 2, additional information:	'81' (Result of another CAG selection type)

CAG information list:

Two sets of CAG information in the list

1st CAG information:

MCC = 244, MNC = 083

Ignore 'CAG only' bit = false

'CAG only' bit = true

1st CAG ID:

CAG ID: 00 00 00 01

2nd CAG information:

MCC = 244, MNC = 084

Ignore 'CAG only' bit = false

'CAG only' bit = true

2nd CAG ID:

CAG ID: 00 00 00 02

CAG Human-readable network name list:

Two CAG Human-readable network names in the list:

1st CAG Human-readable network name: empty

2nd CAG Human-readable network name: ′CAG-00000002′

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 16 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 55 | 02 | 01 | 81 | 56 | 12 | 08 | 42 | 34 | 80 | 01 | 00 |
|  | 00 | 00 | 01 | 08 | 42 | 44 | 80 | 01 | 00 | 00 | 00 | 02 |
|  | 57 | 10 | 80 | 00 | 80 | 0C | 43 | 41 | 47 | 2D | 30 | 30 |
|  | 30 | 30 | 30 | 30 | 30 | 32 |  |  |  |  |  |  |

Expected Sequence 1.31 (PROVIDE LOCAL INFORMATION, Primary Timing advance in Satellite NG-RAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.27.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.27.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.31.1 | [Command performed successfully] |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.31.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"05" Timing Advance

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

NG-RAN/Satellite NG-RAN Primary Timing Advance:	 4 bytes

ME status:	"00" ME is in idle state

NG-RAN/Satellite NG-RAN Primary Timing Advance

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 05 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B1 | 04 | 00 | Note 1 | Note 1 | Note 1 |  |  |  |  |  |  |

Note 1:	This is the NG-RAN/Satellite NG-RAN Primary Timing Advance value. This byte shall not be verified.

Expected Sequence 1.32 (PROVIDE LOCAL INFORMATION, Access Technology, Satellite NG-RAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.32.1 | [Command performed successfully] |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.32.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Access Technology

Technology:	3GPP Satellite NG-RAN

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3F | 01 | 0C |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.33 (PROVIDE LOCAL INFORMATION, Rejected slice(s) information) - without S-NSSAI mapping

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER → ME | Internet PDU session using DNN "TestGp.rs" is configured in the terminal | DNN: "TestGp.rs" for internet PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. | Rejected S-NSSAI, is rejected with cause : S-NSSAI not available in the current registration area |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. | DNN=TestGp.rs, S-NSSAI = 01010103 |
| 4 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.33.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.33.1 |  |
| 7 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.33.1 | [Command performed successfully]. |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.33.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"17" rejected slices information

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 17 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.33.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"17" rejected slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Rejected slices information:

Length of rejected S-NSSAI elements: 5

Length of rejected S-NSSAI: 4

Cause value: 1 (S-NSSAI not available in the current registration area)

Rejected S-NSSAI:  '01 01 01 01' (SST: eMBB, SD: 010101)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 17 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 31 | 05 | 41 | 01 | 01 | 01 | 01 |  |  |  |  |  |

Expected Sequence 1.34 (PROVIDE LOCAL INFORMATION, Rejected slice(s) information) - with S-NSSAI mapping

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER → ME | Internet PDU session using DNN "TestGp.rs" is configured in the terminal | DNN: "TestGp.rs" for internet PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. | Rejected S-NSSAI, is rejected with cause : S-NSSAI not available in the current registration area |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. | DNN=TestGp.rs, S-NSSAI = 01010103 |
| 4 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.34.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.34.1 |  |
| 7 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.34.1 | [Command performed successfully]. |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.34.1

Same as PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.33.1 in expected sequence 1.33.1

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.34.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"17" rejected slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Rejected Slice information with S-NSSAI mapping:

Length of allowed S-NSSAI with mapping information elements: 08

Length of Rejected S-NSSAI: 8

Cause value: 1 (S-NSSAI not available in the current registration area)

Rejected S-NSSAI with mapping information element 1:  '01010102' (SST: eMBB, SD: 010102) '01010102' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010102)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 17 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 57 | 09 | 81 | 01 | 01 | 01 | 02 | 01 | 01 | 01 | 02 |  |

##### 27.22.4.15.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.34.

# *** End of Changes ***