3GPP Conference Call #1 on IMT-2030 	RP-25xxxx

April 29th, 2025

Source:	Indian Institute of Technology (Madras), CEWiT, IIT Kanpur, Tejas Networks, Indian Institute of Technology (Hyderabad), WiSig Networks

Title:	Views on the study of IMT-2030 requirements

Agenda Item:         16 – Study on 6G Scenarios and Requirements

Document for:       Discussion and Decision

# 1. Introduction

In RAN #107:

- Progress was made on the list and definitions of some of the TPRs, along with discussions on target values

- It was agreed to hold two conference calls to facilitate discussions between RAN #107 and RAN #108

In RAN #107, we provided our view in RP-250153. In this contribution, we provide our updated views for CC1 on:

- Possible target values for legacy TPRs

- Definition and possible evaluation methodology for the Coverage TPR

# 2. Target values for legacy TPRs

Our views on the target values for the legacy TPRs are shown in the table below.

Table 1: Our views on target values for legacy TPRs

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in this meeting | Moderator proposal | Our view |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | TBD | 1-1.5x compared to IMT-2020. The proponent shall report the bandwidth used in the calculation. |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | [2x] of IMT-2020 | 1-1.5x compared to IMT-2020 |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | DL: TBD UL: Revisit the evaluation methodology before setting values Further discuss applicable deployment scenarios | DL: 1-1.5x compared to IMT-2020 UL: 2x compared to IMT-2020 Note 1: For Ubiquitous Connectivity, lower values compared to IMT-2020 can be considered. For example: DL: 6 Mbit/s, UL: 4 Mbit/s Note 2: In general, we agree with the potential issues of bandwidth scaling, particularly in the uplink. Having said that, we think that bandwidth scaling can be applied in scenarios where the bands being aggregated are in the same frequency range. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | DL: 1.5x compared to IMT-2020 UL: 2x compared to IMT-2020 Note 1: For Ubiquitous Connectivity, lower values compared to IMT-2020 can be considered. |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | 1.5x compared to IMT-2020 for both UL and DL Note 1: For Ubiquitous Connectivity, lower values compared to IMT-2020 can be considered. |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [10] | 2-3x compared to IMT-2020 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | TBD | IC: 1-4 ms HRLLC: 0.5-1 ms |
|  | Control plane latency (ms) | 20 | [5~20] | TBD | 10-20 ms |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 | 5-10x compared to IMT-2020 |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] | 1-10−5 to 1-10−6 |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | Keep the velocities defined for IMT-2020 with one change. Divide Vehicular into two categories 10-60 kmph for city and 60-120 kmph for highway. |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] | 0 ms |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | 400MHz for system bandwidth (same definition as IMT-2020) | 200 MHz |

# 3. Coverage TPR – Definition and Evaluation Options

Our goal for coverage is increased and improved terrestrial coverage with larger cells. We envision terrestrial coverage to be an important component of Ubiquitous Connectivity, and it should evolve with every new generation. In IMT 2030, TN will be complemented by NTN wherever terrestrial coverage is missing. TN is expected to provide coverage to 95% of the population and with a higher user experienced data rate and cell edge data rate than NTN.

While NTN can be a big driver for serving under connected areas where there is not much habitation of people, we strongly believe that enhanced terrestrial coverage is still necessary to serve under connected areas which do have significant habitation of people (example: rural areas including villages and small towns such as those in the Indian subcontinent). NTN is not expected to handle the very large population density seen in rural environments of developing countries including India, and is also not expected to provide the above-mentioned user experienced data rate and cell edge data rate.

One type of deployment that we envision is one with sparse deployment of large cells without contiguous multi-cell coverage. We believe that this type of deployment is to offer broadband services and is therefore best suited for a new test environment, which we call Rural-UC. The evaluation parameters should be revisited (compared to Rural-eMBB in IMT-2020) to include different UE heights and different UE power classes.

This test environment can be used to evaluate the coverage TPR, which we define as follows:

Figure 1: Rural-UC test environment covering multiple small villages with a large standalone cell.

Coverage refers to the ability to provide access to communication services for users in a desired service area. In this context, communication services can be characterized by requirements such as Avg spectral efficiency, 5th percentile spectral efficiency or simply an SNR value. The desired service area is characterized by the cell edge distance of a single cell.

This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity Usage Scenario and Rural-UC test environment.

| Test Environment | Cell Edge Distance |
| --- | --- |
| Rural-UC (Large cells sparsely located) | >=7 km (corresponding to an ISD of 12 km) |

Note 1: Avg spectral efficiency, 5th percentile spectral efficiency or SNR are provided as evaluation configurations.

Note 2: Cell edge distances beyond the minimum requirement of 7 km, if supported, should be reported.

There are two options for the evaluation methodology of the Coverage TPR.

Option 1: System level simulations

- Run a single cell SLS with a cell radius of 7 km and/or more and with multiple drops.

- Obtain the Avg SE and 5th percentile SE values

- Compare against the configured expected values of Avg SE and 5th percentile SE.

- If the selected cell radius satisfies the expected Avg SE and 5th percentile SE, then the RIT satisfies the coverage TPR

Option 2: Link level simulations

- Using link budget analysis, obtain the SNR at cell edge distance of 7 km and/or more

- Compare against the configured expected SNR value

- If the selected cell radius satisfies expected the SNR, then the RIT satisfies the coverage TPR

# Conclusion

Proposal 1: Consider the target values presented in column 5 of Table 1, towards the discussion on legacy TPR values.

Proposal 2: For peak data rate calculations, the RIT proponent shall report the bandwidth used in the calculation.

Proposal 3: For test environments related to Ubiquitous Connectivity, lower values compared to IMT-2020 should be considered for the following TPRs: User Experienced Data Rate, Avg Spectral Efficiency and 5th Percentile Spectral Efficiency.

Proposal 4: Bandwidth scaling can be applied in scenarios where the bands being aggregated are in the same frequency range.

Proposal 5: For the Ubiquitous Connectivity usage scenario, the study should consider coverage, 5th percentile and average spectral efficiency as TPRs. A new test environment should be defined for Ubiquitous Connectivity which includes at least the Rural geographical area, with an emphasis on large cells.

Proposal 6: Rural geographies should be included in a new test environment called Rural-UC for the evaluation of TPRs related to Ubiquitous Connectivity. The Rural-UC test environment should include the sparse deployment of large cells without contiguous multi-cell coverage, as shown in Figure 1.

Proposal 7: The evaluation parameters for the Rural-UC test environment should be revisited (compared to Rural-eMBB in IMT-2020) to include different UE heights and different UE power classes.

Proposal 8: The evaluation of coverage in Rural-UC can be by system level simulation or link level simulation. The details are given below:

Option 1: System level simulations

- Run a single cell SLS with a cell radius of 7 km and/or more and with multiple drops.

- Obtain the Avg SE and 5th percentile SE values

- Compare against the configured expected values of Avg SE and 5th percentile SE.

- If the selected cell radius satisfies the expected Avg SE and 5th percentile SE, then the RIT satisfies the coverage TPR

Option 2: Link level simulations

- Using link budget analysis, obtain the SNR at cell edge distance of 7 km and/or more

- Compare against the configured expected SNR value

- If the selected cell radius satisfies expected the SNR, then the RIT satisfies the coverage TPR