3GPP TSG-RAN WG3 Meeting #120	R3-233385

Incheon, Republic of Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0105 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subcarrier Spacing correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, CMCC, China Telecom, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 9.2.28, the SCS values for FR2-2 (480 kHz, 960 kHz) are missing for the subcarreir spacing of Uplink Channel BW-PerSCS-List and Active UL BWP. Following is captured from TS 38.331 the field description for SCS-SpecificCarrier IE. To align with TS 38.331, the missing values should be added. subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. Similarly, the SCS values for FR2-2 (480 kHz, 960 kHz) are missing for the subcarreir spacing of the SSB Time/Frequency Configuration IE related to SSB-Configuration IE in TS 38.331. . |
| Summary of change: |  | - Add the SCS values of 480 kHz and 960 kHz to Uplink Channel BW-PerSCS-List and Active UL BWP. - Add the SCS values of 480 kHz and 960 kHz to the SSB Time/Frequency Configuration IE. - Indicate that the SSB Time/Frequency Configuration IE does not support SCS 60 kHz Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SSB and SRS Subcarrier Spacing configuration for positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SCS values are not aligned with TS 38.331 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.28, 9.2.55, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.455 CR 0104 TS/TR 38.473 CR 1178 TS/TR 38.473 CR 1179 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: correct the WI code, tick Core Network Box, cat. A to cat F as more feature impacted and add co-source |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.2.28	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1 |  |  |
| >SRS Carrier List Item |  | 1..<maxnoSRS-Carriers> |  |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >>Uplink Channel BW-PerSCS-List |  | 1 |  | Corresponds to sCS-SpecificCarrierList in TS 38.331 [13] |
| >>>SCS Specific Carrier |  | 1. .<maxnoSCSs> |  |  |
| >>>>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  |
| >>>>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  |
| >>Active UL BWP | M |  |  | Only the configuration in the active UL BWP is needed. |
| >>>Location And Bandwidth | M |  | INTEGER(0..37949,…) | Corresponds to information provided in locationAndBandwidth contained in BWP IE as defined in TS 38.331 [13] |
| >>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  |
| >>>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  |
| >>>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  |
| >>>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  |
| >>>SRS Config | M |  |  | Corresponds to information provided in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  |
| >>>>>SRS Resource | M |  | 9.2.29 | Corresponds to information provided in SRS-Resource contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  |
| >>>>>Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  |
| >>>>>SRS Resource Set | M |  | 9.2.31 | Corresponds to information provided in SRS-ResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  |
| >>>>>Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets per UL BWP. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.55	SSB Time/Frequency Configuration

This information element contains the time and frequency configuration of an SSB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB frequency | M |  | INTEGER (0..3279165) | ARFCN |
| SSB subcarrier spacing | M |  | ENUMERATED(15kHz, 30kHz, 60kHz, 120kHz, 240kHz,... , kHz480, kHz960) | The value 60kHz is not supported in this version of the specification. |
| SSB Transmit power | M |  | INTEGER (-60..50) | EPRE of SSS |
| SSB periodicity | M |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, 160ms, …) |  |
| SSB half frame index | M |  | INTEGER(0..1) |  |
| SSB SFN offset | M |  | INTEGER(0..15) |  |
| CHOICE SSB Position in Burst | O |  |  |  |
| >Short Bitmap |  |  | BIT STRING (SIZE(4)) |  |
| >Medium Bitmap |  |  | BIT STRING (SIZE(8)) |  |
| >Long Bitmap |  |  | BIT STRING (SIZE(64)) |  |
| SFN initialisation time | O |  | Relative Time 1900 9.2.36 |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

ActiveULBWP  ::= SEQUENCE {

locationAndBandwidth		INTEGER (0..37949,...),

subcarrierSpacing           ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

cyclicPrefix				ENUMERATED {normal, extended},

txDirectCurrentLocation		INTEGER (0..3301,...),

shift7dot5kHz				ENUMERATED {true, ...} OPTIONAL,

sRSConfig					SRSConfig,

iE-Extensions				ProtocolExtensionContainer { { ActiveULBWP-ExtIEs} } OPTIONAL,

...

}

ActiveULBWP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- S

SCS-SpecificCarrier ::= SEQUENCE {

offsetToCarrier                     INTEGER (0..2199,...),

subcarrierSpacing                   ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

carrierBandwidth                    INTEGER (1..275,...),

iE-Extensions					ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL,

...

}

SCS-SpecificCarrier-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

TF-Configuration ::= SEQUENCE {

sSB-frequency				INTEGER (0..3279165),

sSB-subcarrier-spacing		ENUMERATED {kHz15, kHz30, kHz120, kHz240, ..., kHz60, kHz480, kHz960},

-- The value kHz60 is not supported in this version of the specification.

sSB-Transmit-power			INTEGER (-60..50),

sSB-periodicity				ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},

sSB-half-frame-offset		INTEGER(0..1),

sSB-SFN-offset				INTEGER(0..15),

sSB-BurstPosition			SSBBurstPosition	OPTIONAL,

sFN-initialisation-time		RelativeTime1900	 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TF-Configuration-ExtIEs} }	OPTIONAL,

...

}

TF-Configuration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>