3GPP TSG RAN Meeting #100							RP-231203

Taipei, June 12th -14th, 2023

Agenda Item:	9.3.2.6

Source:	Ericsson

Title:	No-transmit zones for UAVs

Document for:	Discussion, Agreement

# Introduction

Rel-18 WI on NR support for UAVs has evolved in scope over time. In the latest update of the WID in RAN#99 [1], a RAN4-led objective was added to address out-of-band emission requirements set by the ECC Decision 22(07). The present contribution addresses another aspect in the ECC Decision 22(07), namely the no-transmit zone, which has not been covered in the UAV WI.

# No-transmit zone

In November 2022, CEPT made Decision 22(07) on Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in several bands harmonized for MFCN. The decision assumes multiple technical conditions and requirements to support aerial UEs in mobile systems (both LTE and NR). Two notable ones are no-transmit zone (NTZ) and out-of-band emission (OOBE) requirements, as shown in the following excerpt from the Decision (further details are in the Appendix):

In addition to the already harmonised technical conditions for MFCN bands and for spectrum compatibility purposes, there is the need to define some spectrum operational restrictions. This can be done using “no-transmit zones”, which should be defined at national level as a geographical area where aerial UE are not allowed to operate in a certain frequency band. Another measure to achieve coexistence is to define additional OOB emission limits specific to aerial UE (to avoid interference to other services in some other bands (e.g. to protect MetSat at 1675-1710 MHz) . The requirement may apply to aerial UE according to their operational frequency band, e.g. aerial UE operating in a specific band or specific channel (see no-fly zone definition set out in ECC Report 309, in this Decision referred to as “no-transmit zone”). In some cases, operation of aerial UE also requires respective cross-border coordination agreements. … # ECC Decision of 18 november 2022 on Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5g NR in the 703-733 MHz, 832-862 MHz, 880-915 MHz ,1710-1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570-2620 MHz MFCN harmonised bands (ECC decision (22)07) “The European Conference of Postal and Telecommunications Administrations, Considering … - that a no-transmit zone in this Decision is defined as a geographical area where aerial UE are not allowed to transmit for spectrum compatibility purposes in a given harmonised MFCN band or part of it; - that national studies are needed, as appropriate, to define no-transmit zones for spectrum compatibility purposes, for aerial UE operating in the relevant frequency bands; - that a mechanism is necessary to ensure that aerial UE respect no-transmit zones; … DECIDES …. that no-transmit zones as described in this Decision should be defined and implemented at national level and where necessary coordinated with neighbouring countries;

As can be seen in n) above, the ECC Decision asserts that a mechanism is necessary to ensure that aerial UE respect no-transmit zones.

As with additional OOBE requirements, no-transmit zones are important to protect incumbent radio systems from potential interference from aerial UEs.

ECC decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands” requires a mechanism to ensure that aerial UEs respect no-transmit zones.

To make 3GPP technology fully ready to support Aerials use cases, it is important that all relevant regulatory requirements are fulfilled. Given that Rel-18 UAV WI has already included objectives to fulfill the FAA requirements on broadcast remote ID and the additional OOBE requirements mandated by the Decision 22(07), we believe that the support of no-transmit zones should also be included. It is not advisable to postpone the support of no-transmit zones to a later release because it will delay the deployment of cellular-connected aerial UEs. Furthermore, since the ECC Decision covers both LTE and NR, no-transmit zones should be supported by the two RATs, as in the case of BRID and OOBE.

It is important that all regulatory requirements related to aerial UEs are fulfilled by the same release (Rel-18) of both LTE and NR.

Therefore, we propose to amend the Rel-18 WIDs on LTE and NR support of UAV with an objective of supporting no-transmit zones.

Amend Rel-18 UAV WIDs (both LTE and NR) with the following objective: specify mechanisms to support no-transmit zones.

# Mechanisms to support no-transmit zones

First thing to elaborate is what would be the impact of no-transmit zones on UAV communications in practice. Apparently, it would cover UL traffic from a UAV in connected mode which may include heavy user plane in UL but also UAV command and control, and control plane signaling needed to maintain the UAV in connected mode and supporting, e.g., mobility. If the no-transmit zone is to be understood in a very strict manner it would mean UE should not perform initial access to a cell in NTZ. In that case, a UAV UE may not even camp on such cell. In a less strict scenario, the UE could access the cell but is steered to another cell for further transmissions. Note that a flying UAV could see several cells and could potentially be served by a far- away cell even if the UAV itself would be in no-transmit zone or close to it.

Yet another aspect to consider is the potential interference to neighbor cells. In a case UAV is in no-transmit zone but connecting to another cell, UAV may cause unwanted interference. Also, in case UAV is close to non-transmit zone.

NTZs can affect aerial UE access to a cell and interference to neighbouring cells.

Given the analysis above, we foresee two types of mechanisms for an aerial UE to respect NTZ, which will be discussed next.

First, at cell-level granularity, given that a NTZ is a geographical area with static or semi-static location, the mobile network can be well-aware of the NTZ and knows which cells overlap with the NTZ. Therefore, the network can take certain measures to bar an aerial UE from connecting to a cell which have overlaps with the NTZ.

Cell-level measures can be imposed by RAN nodes to bar aerial UEs from accessing NTZ-affected cells.

Second, at a finer granularity, signaling can be defined for a connected drone to inform RAN nodes when the drone is entering a NTZ, allowing the network to be prepared to maintain the connectivity with the drone (e.g., to switch to another frequency band or carrier that is allowed in the NTZ). Vice versa, knowing when the drone is leaving a NTZ enables the network to recover to a previous mode of communication with the drones. This may be needed, e.g., to monitor interference caused by UAVs near the NTZ.

Connected aerial UEs can inform RAN nodes when the UEs are entering or leaving no-transmit zones.

The above may be needed in deployments that have large cells serving the UAVs.

In our view, the first mechanism (cell-level) has several advantages:

- It is applicable to UEs in both Connected mode and Idle mode,

- It requires less work in RAN2. Specifically, only one bit in the system information broadcasted by the network is needed to bar aerial UEs from accessing the affected cells.

Given these clear advantages, we propose to focus on the cell-level mechanism in the Rel-18 UAV WI.

RAN to task RAN2 to specify signalling that allows NTZ-affected cells to bar aerial UEs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	As with additional OOBE requirements, no-transmit zones are important to protect incumbent radio systems from potential interference from aerial UEs.

Observation 2	ECC decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands” requires a mechanism to ensure that aerial UEs respect no-transmit zones.

Observation 3	It is important that all regulatory requirements related to aerial UEs are fulfilled by the same release (Rel-18) of both LTE and NR.

Observation 4	NTZs can affect aerial UE access to a cell and interference to neighbouring cells.

Observation 5	Cell-level measures can be imposed by RAN nodes to bar aerial UEs from accessing NTZ-affected cells.

Observation 6	Connected aerial UEs can inform RAN nodes when the UEs are entering or leaving no-transmit zones.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Amend Rel-18 UAV WIDs (both LTE and NR) with the following objective: specify mechanisms to support no-transmit zones.

Proposal 2	RAN to task RAN2 to specify signalling that allows NTZ-affected cells to bar aerial UEs.

# References

RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

# Appendix

Some further details on no-transmit zones from the ECC Decision 22(07)

- A1.2 Operational conditions The operational conditions to be defined and implemented at national level provide additional measures to the technical conditions in order to protect other services. 703-733 MHz: Protection of DTT receivers and RAS sites Aerial UE operating in 703-733 MHz should not transmit when less than 30 m above ground level to avoid interference to DTT receivers; Nationally determined no-transmit zones are required around RAS sites operating in 1400-1427 MHz for aerial UE operating in the 703-718 MHz frequency band, as appropriate. 832-837 MHz: Protection of RAS sites Nationally determined no-transmit zones are required around RAS sites operating in 1660-1670 MHz for aerial UE operating in the 832-837 MHz frequency band, as appropriate. 2500-2570 MHz/2570-2620 MHz: Protection of RAS sites and radars Nationally determined no-transmit zones are required around RAS sites operating in 2690-2700 MHz for aerial UE operating in the 2500-2570 MHz or 2570-2620 MHz frequency band, as appropriate; Nationally determined no-transmit zones might be required around radars operating in 2700-2900 MHz for aerial UE operating in the 2500-2570 MHz or 2570-2620 MHz frequency band.

Another frequency band than 703-733 MHz shall be used for landing and take-off.