3GPP TSG RAN WG1 #112bis-e																	R1-2304285

e-Meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0483 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core, NR_cov_enh-Core, NR_IAB_enh-Core, NR_MBS-Core,. NR_NTN_Solutions-Core, NR_UE_pow_sav_enh-Core TEI17, NR_L1enh_URLLC-Core, NR_unlic-Core, NR_2step_RACH-Core |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Incorrect RRC parameter name in clause 6. - Incorrect parameter reference for PUCCH power control in clause 7. - Unclear definition of PUSCH resource configuration in clause 8.1A. - Ambiguous use of ‘or’ in clause 14 - PUCCH configuration in pucch-sSCell is missing in clause 12. - Missed reference to multiplexing of UCI with different priority indexes when conditions are satisfied in clause 9.2.5. - Misaligned RRC parameter name in clause 9.2.6. - Misaligned RRC parameter name in clause 10.4. - Incorrect reference/definition for tepoch in clause 4.2. - Unclear value of NPDSCHrepeat for multicast PDSCH if pdsch-AggregationFactor-r16 is not included in SPS-Config and misaligned RRC parameter names in clause 9.1.2. - Unclear when a UE multiplexes HARQ-ACK in a PUSCH when VT-DAIUL=0 or when VT-DAIUL,U=0 or VT-DAIUL,M=0 in clause 9.1.2.2. - Incorrect assumption that there are always two NFI fields in a DCI format in clause 9.1.3.3 - Unclear PUCCH resource to use for multiplexing HARQ-ACK for unicast and multicast when scheduled by a DCI format or when UE is not provided pucch-ConfigurationListMulticast1 in clause 18. - Incorrect parameter name for a PUCCH resource in clause 18. - DCI format 4_0 is missing from the DCI formats associated with the assumption on the PDCCH DMRS EPRE to SSS EPRE ratio when UE has not been provided dedicated higher layer parameters in clause 4.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct TCI-UL-State to ul-TCI-StateList in clause 6. - Correct PO_PUCCH,b,f,cqu to PO_UE_PUCCHqu in clause 7. - Clarify that PUSCH resource configuration is provided by MsgA-PUSCH-Resource in clause 8.1A. - Clarify the “A or B” means A, B, A and B (it is not “either A or B”). - Add reference to PUCCH-sSCell in clause 12. - Capture multiplexing of UCI with different priority indexes when conditions are satisfied in clause 9.2.5. - Correct PUCCH-DMRS-Bundling to pucch-DMRS-Bundling, PUCCH-Frequencyhopping-Interval to pucch-FrequencyHoppingInterval, and PUCCH-TimeDomainWindowLength to pucch-TimeDomainWindowLength in clause 9.2.6. - Correct PDCCHSkippingDurationList to pdcch-SkippingDurationList in clause 10.4. - Correct reference/definition for tepoch in clause 4.2. - Clarify that for multicast PDSCH, NPDSCHrepeat=1 if pdsch-AggregationFactor-r16 is not included in SPS-Config and align RRC parameter names in clause 9.1.2. - Clarify when a UE multiplexes HARQ-ACK in a PUSCH when VT-DAIUL=0 or when VT-DAIUL,U=0 or VT-DAIUL,M=0 in clause 9.1.2.2. - Update description in pseudo-code to capture that a DCI format can include either one or two NFI fields in clause 9.1.3.3. - Clarify PUCCH resource to use for multiplexing HARQ-ACK for unicast and multicast when scheduled by a DCI format or when UE is not provided pucch-ConfigurationListMulticast1 in clause 18. - Correct SPS-PUCCH-AN to n1PUCCH-AN in clause 18. - Add DCI format 4_0 to DCI formats associated with the assumption on the PDCCH DMRS EPRE to SSS EPRE ratio when UE has not been provided dedicated higher layer parameters in clause 4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/Inconsistent specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2, 6, 7, 8.1A, 9.1.2, 9.1.2.2, 9.1.3.3, 9.2.5, 9.2.6, 10.4, 12, 14, 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

## 4.1	Cell search

Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell.

A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211].

A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, or for a DCI format 2_7, or for a DCI format 4_0.

## *** Unchanged text is omitted ***

## 4.2	Transmission timing adjustments

## *** Unchanged text is omitted ***

If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot. The UE does not change NTA during an actual transmission time window for a PUSCH or a PUCCH transmission [6, TS 38.214].

Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2

where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant TACommonTACommonDriftTACommonDriftVariant[12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

## *** Unchanged text is omitted ***

# 6	Link recovery procedures

## *** Unchanged text is omitted ***

For the PCell or the PSCell and for sets q0 and q1, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where a UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE assumes same antenna port quasi-collocation parameters as the ones associated with index qnew for PDCCH monitoring in a CORESET with index 0.

If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state for the PCell or the PSCell [6, TS 38.214], after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE

-	if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH, using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and the PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

For the remaining of this clause, if a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinkingId, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2.

For the PCell or the PSCell, if BFR MAC CE [11, TS 38.321] is provided in Msg3 or MsgA of contention based random access procedure, and if a PUCCH resource is provided with PUCCH-SpatialRelationInfo, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in clause 5.1.4a or in clause 5.1.5 of [11, TS 38.321], the UE transmits the PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, where qnew is the SS/PBCH block index selected for the last PRACH transmission.

If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state for the PCell or the PSCell and the UE provides BFR MAC CE in Msg3 or MsgA of contention based random access procedure, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in [11, TS 38.321], the UE

-	if SSB-MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

A UE can be provided, by schedulingRequestID-BFR-SCell, a configuration for PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 for the UE to transmit PUCCH [11, TS 38.321]. If the PCell or the PSCell is associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can be provided by schedulingRequestID-BFR a first configuration for PUCCH transmission with a LRR and, if the UE provides twoLRRcapability, the UE can be provided by schedulingRequestID-BFR2 a second configuration for PUCCH transmission with a LRR. If the UE is provided only the first configuration, the UE transmits a PUCCH with LRR for either set q0,0 or q0,1. If the UE is provided both the first and second configurations, the UE uses the first configuration to transmt a PUCCH with LRR associated with set q0,0 and the second configuration to transmit a PUCCH with LRR associated with set q0,1 [11, TS 38.321].

The UE can provide in a first PUSCH MAC CE index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s). After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any

-	transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew, if any, for periodic CSI-RS or SS/PBCH block reception, as described in clause 9.2.2, and using a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, if

-	the UE is provided PUCCH-SpatialRelationInfo for the PUCCH,

-	a PUCCH with the LRR was either not transmitted or was transmitted on the PCell or the PSCell, and

-	the PUCCH-SCell is included in the SCell(s) indicated by the MAC-CE

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

If a UE is provided dl-OrJointTCI-StateList or ul-TCI-StateList indicating a unified TCI state, after 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	if SSB -MTC-AdditionalPCI is not provided, monitors PDCCH in all CORESETs, on the SCell (s) indicated by the MAC CE, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as the one corresponding to qnew, if any, and using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with the smallest value of ul-powercontrolId for the corresponding SCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with the smallest value of ul-powercontrolId for the corresponding SCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with the smallest value of ul-powercontrolId for the corresponding SCell

## *** Unchanged text is omitted ***

# 7	Uplink Power control

## *** Unchanged text is omitted ***

In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State and for an indicated TCI-State or TCI-UL-State as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS

-	in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State

-	in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_UE_PUCCHquPO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State

-	in clause 7.3.1, if p0AlphaSetforSRS is provided,

-	if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State

-	else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set

*** Unchanged text omitted ***

## 8.1A	PUSCH for Type-2 random access procedure

For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the SCS configuration for the active UL BWP.

A UE does not transmit a PUSCH in a PUSCH occasion if the PUSCH occasion associated with a DMRS resource is not mapped to a preamble of valid PRACH occasions or if the associated PRACH preamble is not transmitted as described in clause 7.5 or clause 11.1 or clause 17.2. A UE can transmit a PRACH preamble in a valid PRACH occasion if the PRACH preamble is not mapped to a valid PUSCH occasion.

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by MsgA-PUSCH-Resource.

A UE determines time resources and frequency resources for PUSCH occasions in an active UL BWP from msgA-PUSCH-Config or separateMsgA-PUSCH-Config for the active UL BWP. If the active UL BWP is not the initial UL BWP and msgA-PUSCH-Config or separateMsgA-PUSCH-Config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-Config or separateMsgA-PUSCH-Config provided for the initial UL BWP.

*** Unchanged text omitted ***

### 9.1.2	Type-1 HARQ-ACK codebook determination

*** Unchanged text omitted ***

while c<NcellsDL

Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations

while s<NcSPS

Set nD=0 – slot index

while nD<NcDL

if {

a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, NPDSCHrepeat=1, and

HARQ-ACK information for the SPS PDSCH is associated with the PUCCH

}

ojACK = HARQ-ACK information bit for this SPS PDSCH reception

j=j+1;

end if

nD=nD+1;

end while

s=s+1;

end while

c=c+1;

end while

*** Unchanged text omitted ***

#### 9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

*** Unchanged text omitted ***

A UE does not expect to detect a DCI format switching a DL BWP within N2 symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where N2 is defined in [6, TS 38.214].

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, and

-	is not provided fdmed-ReceptionMulticast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, or

-	is provided pdsch-HARQ-ACK-Codebook = 'semi-static' only for one of unicast and multicast HARQ-ACK information

the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives

-	only a SPS PDSCH release, or

-	only unicast SPS PDSCH(s) associated with transport blocks having enabled HARQ-ACK information report, or only a TCI state update, or only a PDSCH that is scheduled by a DCI format 1_0 having enabled associated HARQ-ACK information report if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or

-	multicast SPS PDSCH(s) with transport blocks having enabled associated HARQ-ACK information report or scheduled by a DCI format 4_1 having enabled associated HARQ-ACK information report if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information,

with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the TCI state update or only for the PDSCH reception as described in clause 9.1.2.

VT-DAIUL=0 if the PUSCH is scheduled by a DCI format that includes a DAI field and the DAI field is set to '0'; otherwise, VT-DAIUL=1.

If a UE is provided fdmed-ReceptionMulticast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH

-	for the first set SU of NcellsDL,U serving cells if a value VT-DAIUL,U of the DAI field associated with unicast HARQ-ACK information is VT-DAIUL,U=1 [5, TS 38.212]

-	for the second set SM of NcellsDL,M serving cells if a value VT-DAIUL,M of the DAI field associated with multicast HARQ-ACK information is VT-DAIUL,M=1 [5, TS 38.212]

The UE does not generate unicast or multicast HARQ-ACK information for multiplexing in the PUSCH transmission when VT-DAIUL,U=0 or VT-DAIUL,M=0, respectively, unless the UE receives respectively

-	only a unicast or a multicast SPS PDSCH release, or

-	only unicast SPS PDSCH(s) or multicast SPS PDSCH(s) having enabled associated HARQ-ACK information reports, or

-	only a TCI state update, or a PDSCH with enabled associated HARQ-ACK information report that is scheduled by a DCI format 1_0 or a DCI format 4_1 with counter DAI field value of 1 on the PCell

in the Mc occasions for candidate PDSCH receptions in which case the UE generates only the corresponding unicast or multicast HARQ-ACK information.

VT-DAIUL,U=0 if the corresponding value of the DAI field is set to '0'; otherwise, VT-DAIUL,U=1. VT-DAIUL,M=0 if the corresponding value of the DAI field is set to '0'; otherwise, VT-DAIUL,M=1 [5, TS 38.212].

*** Unchanged text omitted ***

#### 9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

If a UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure.

Set g to the value of a PDSCH group index field in a last DCI format that provides a value of g and indicates a PUCCH transmission occasion.

Set i(g) to denote a PUCCH transmission occasion for multiplexing HARQ-ACK information

Set k to the value of a PDSCH-to-HARQ_feedback timing field, if any, in a DCI format providing a value of g

-	If the DCI format does not include a PDSCH-to-HARQ_feedback timing field, set k to the value provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16

Set h(g) to the value of the New feedback indicator field in the last DCI format providing the value of g if the New feedback indicator field includes 1 bit, or to the MSB of the New feedback indicator field in the last DCI format providing the value of g if the New feedback indicator field includes 2 bits

Set hg+1mod2(g) to a value of the LSB of the New feedback indicator field in the last DCI format providing the value of g if the New feedback indicator field includes 2 bits, or to ∅ otherwise

Set VDAIg+1mod2 to the value of a total DAI field for group g+1mod2 in the last DCI format providing the value of g

-	If g=1 and the UE detects a DCI format that does not include a PDSCH group index field in a PDCCH reception that is after the PDCCH reception for the last DCI format detection providing the value of g and indicating a same slot for a PUCCH transmission occasion, set VDAIg+1mod2=∅

### 9.2.5	UE procedure for reporting multiple UCI types

*** Unchanged text omitted ***

If a UE would multiplex CSI reports that include Part 2 CSI reports in a PUCCH resource, the UE determines the PUCCH resource and a number of PRBs for the PUCCH resource or a number of Part 2 CSI reports assuming that each of the CSI reports indicates rank 1, or rank combination of {1, 1} if applicable. If the higher layer parameter csi-ReportMode of CSI reports is set to 'Mode2', the UE determines the PUCCH resource and a number of PRBs for the PUCCH resource or a number of Part 2 CSI reports assuming that each CRI in the CSI report is associated with a resource pair.

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot and, when applicable as described in clauses 9.2.5.1, 9.2.5.2 and 9.2.5.3, the UE is configured to multiplex different UCI types or UCI of different priority indexes in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types or UCI of different priority indexes if the following conditions are met. If one of the PUCCH transmissions or PUSCH transmissions is in response to a DCI format detection by the UE, the UE expects that the first symbol S0 of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions

*** Unchanged text omitted ***

### 9.2.6	PUCCH repetition procedure

A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where

-	if the PUCCH resource is indicated by a DCI format and includes pucch-RepetitionNrofSlots, NPUCCHrepeat is provided by pucch-RepetitionNrofSlots

-	otherwise, NPUCCHrepeat is provided by nrofSlots

If the UE is provided subslotLengthForPUCCH, a slot for a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots includes a number of symbols indicated by subslotLengthForPUCCH.

For NPUCCHrepeat>1,

-	the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots

-	a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols

-	a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH)

-	the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for repetitions of the PUCCH transmission in different slots

-	if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is not provided pucch-DMRS-Bundling = 'enabled'

-	the UE performs frequency hopping per slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in slots with even number and starting from a second PRB, provided by secondHopPRB, in slots with odd number. The slot indicated to the UE for the first repetition of the PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in NPUCCHrepeat slots is counted regardless of whether or not the UE transmits the PUCCH in the slot

-	the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot

-	if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is provided pucch-DMRS-Bundling = 'enabled'

-	the UE performs frequency hopping per interval of NPUCCHinterval consecutive slots, that start from a slot indicated to the UE and where the UE would transmit a first repetition of the PUCCH, where NPUCCHinterval is the value of pucch-FrequencyHoppingInterval, if provided; otherwise, NPUCCHinterval is the value of pucch-TimeDomainWindowLength

-	the UE transmits the PUCCH over intervals until the UE transmits the PUCCH in NPUCCHrepeat slots, where the first interval has number 0 and each subsequent interval is counted regardless of whether or not the UE transmits the PUCCH in a slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in intervals with even number and starting from a second PRB, provided by secondHopPRB, in intervals of frequency hopping intervals with odd number

-	the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot

-	if the UE is not configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot

*** Unchanged text omitted ***

## 10.4	Search space set group switching and skipping of PDCCH monitoring

A UE can be provided

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell,

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell.

If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17 for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set.

A UE can be provided a set of durations by pdcch-SkippingDurationList for Type3-PDCCH CSS set or USS set for PDCCH monitoring on an active DL BWP of a serving cell. If the UE is not provided pdcch-SkippingDurationList, the following procedures related to skipping of PDCCH monitoring are not applicable.

If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList.

When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList.

A UE can be provided by searchSpaceSwitchDelay a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2.

Table 10.4-1: Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | - |
| 5 | 160 | - |
| 6 | 320 | - |

A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration μ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure.

*** Unchanged text omitted ***

A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the largest Xs if the SCS configuration μ among all configured DL BWPs in the set of serving cells equals to 6, otherwise, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets.

A UE can be provided a set of durations by pdcch-SkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits.

If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '0' value for the bit indicates no skipping in PDCCH monitoring

-	a '1' value for the bit indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations

If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '00' value for the bits indicates no skipping in PDCCH monitoring

-	a '01' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations

-	a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations

-	a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the third value in the set of durations, if any; otherwise, if the set of durations includes two values, a use of the '11' value is reserved

A UE can be provided group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided pdcch-SkippingDurationList for the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits for the serving cell.

If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '0' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any

-	a '1' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '00' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any

-	a '01' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 2 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '11' value is reserved

A UE can be provided a set of durations by pdcch-SkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits.

*** Unchanged text omitted ***

# 12	Bandwidth part operation

*** Unchanged text omitted ***

If a UE is provided controlResourceSetZero and searchSpaceZero in PDCCH-ConfigSIB1 or PDCCH-ConfigCommon, the UE determines a CORESET for a search space set from controlResourcesetZero as described in clause 13 and for Tables 13-1 through 13-10, and determines corresponding PDCCH monitoring occasions as described in clause 13 and for Tables 13-11 through 13-15. If the active DL BWP is not the initial DL BWP, the UE determines PDCCH monitoring occasions for the search space set only if the CORESET bandwidth is within the active DL BWP and the active DL BWP has same SCS configuration and same cyclic prefix as the initial DL BWP.

For each UL BWP in a set of UL BWPs of the PCell or of the PUCCH-SCell or of the PUCCH-sSCell, the UE is configured resource sets for PUCCH transmissions as described in clause 9.2.1.

A UE receives PDCCH and PDSCH in a DL BWP according to a configured SCS and CP length for the DL BWP. A UE transmits PUCCH and PUSCH in an UL BWP according to a configured SCS and CP length for the UL BWP.

*** Unchanged text omitted ***

# 14	Integrated access-backhaul operation

*** Unchanged text omitted ***

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH.

For each cell of an IAB-DU in a set of cells of the IAB-DU, the IAB-DU can be provided:

-	an identity of the IAB-DU cell by iab-DU-CellIdentity

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI and/or by positionInDCI-AI-r17

-	a set of availability combinations by availabilityCombinations-r16 or by availabilityCombinationsRBGroups-r17, where each availability combination in the set of availability combinations includes

-	resourceAvailability-r16 indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one resourceAvailability-r17 indicating availability of soft resources in all RB sets in one or more slots for the IAB-DU cell, or one or multiple RB set groups by rb-SetGroups-r17 with each RB set groups by RB-SetGroup-r17 indicating resourceAvailability-r17 for soft resources in one or more slots for the associated rb-Sets-r17, and

-	a mapping for the soft symbol, and/or for soft resources, availability combinations provided by resourceAvailability-r16 or resourceAvailability-r17 to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId-r16 or availabilityCombinationId-r17, respectively

With reference to a slot of an IAB-DU cell, if the IAB-DU is not provided an indication of hard, soft or unavailable type per RB set by Frequency-Domain HSNA Configuration List [16, TS 38.473], the indication of availability for the slot is based solely on availabilityCombinations-r16.

The IAB-DU can assume a same SCS configuration for availabilityCombinations-r16 or availabilityCombinationsRBGroups-r17 for a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell.

An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol and/or a soft RB set in an RB set group availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes maxlog2maxAIindex+1,1 bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol or a soft RB set in an RB set group in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3.

*** Unchanged text omitted ***

For a serving cell of an IAB-MT, the IAB-MT can be provided a set of TCI states or a set of RS resource indexes corresponding to a SS/PBCH block or to a CSI-RS resource index for a slot where a PDSCH EPRE adjustment is indicated by DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The PDSCH EPRE can be derived from a downlink CSI-RS EPRE as described in [6, TS 38.214] and a power offset provided by the DL Tx Power adjustment field in DL TX Power Adjustment MAC CE as described in [11, TS 38.321]. The downlink CSI-RS EPRE refers to the CSI-RS indicated by Reference CSI-RS ID in DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The DL TX Power Adjustment provides the offset between PDSCH EPRE and CSI-RS EPRE. For a downlink DM-RS and/or PT-RS associated with a PDSCH, the IAB-MT may assume that the ratio of PDSCH EPRE to DM-RS EPRE, and/or PT-RS EPRE to PDSCH EPRE, is obtained as for a "UE" in [6, TS 38.214]. If no TCI state or RS resource index is provided to the IAB-MT, the IAB-MT may assume that a same PDSCH EPRE adjustment applies to all TCI states or RS resource indexes configured for the IAB-MT. A PDSCH EPRE adjustment provided by DL Tx Power Adjustment MAC CE may be associated with

-	multiplexing mode of the IAB-node, if provided, and/or

-	when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in non-overlapping frequency resources, if provided, or when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in overlapping frequency resources, if provided, and/or

-	slots indicated by slot indexes, if provided.

*** Unchanged text omitted ***

# 18	Multicast Broadcast Services

*** Unchanged text omitted ***

A UE can be configured, per DL BWP by cfr-ConfigMulticast, an MBS frequency resource within the DL BWP for PDCCH and PDSCH receptions [4, TS 38.211]. If cfr-ConfigMulticast does not include locationAndBandwidthMulticast, the MBS frequency resource is the DL BWP. In clauses referring to a higher layer parameter value provided by PDCCH-Config or PDSCH-Config or SPS-Config for a DL BWP, when applicable a corresponding higher layer parameter value for multicast PDCCH, PDSCH, or SPS PDSCH receptions is provided as described in [12, TS 38.331].

In clauses referring to a higher layer parameter value provided by a first or second PUCCH-Config, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast PDCCH or PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by n1-PUCCH-AN or SPS-PUCCH-AN-List, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast SPS PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by pdsch-HARQ-ACK-Codebook or pdsch-HARQ-ACK-CodebookList, when applicable a corresponding higher layer parameter value for HARQ-ACK codebooks associated with multicast HARQ-ACK information is provided as described in [12, TS 38.331].

A UE monitors PDCCH for scheduling PDSCH receptions or for activation/release of SPS PDSCH receptions for a corresponding SPS PDSCH configuration as described in clause 10.1.

*** Unchanged text omitted ***

A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5.

If a UE multiplexes in a PUCCH HARQ-ACK information of same priority associated with unicast DCI formats and with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource from pucch-Config/pucch-ConfigurationList, as described in clause 9.2.3, is a last unicast DCI format.

If the UE multiplexes in a PUCCH only multicast HARQ-ACK information of same priority that is according to both the first and second HARQ-ACK reporting modes, the last DCI format that the UE uses to determine the PUCCH resource from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided; otherwise, from pucch-Config/pucch-ConfigurationList, as described in clause 9.2.3, is a last DCI format associated with multicast HARQ-ACK information that is according to the first HARQ-ACK reporting mode.

If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and both the first and second HARQ-ACK information are according to the first HARQ-ACK reporting mode, the UE determines the PUCCH resource based on the last multicast DCI format, as described in clause 9.2.3.

*** Unchanged text omitted ***