3GPP TSG RAN#100                                          	RP-231259

Taipei, June 12 – 14, 2023

Source:              CATT, GOHIGH

Title:                   Discussion on the scope of Rel-18 NR sidelink evolution

Agenda Item:     9.3.1.5

Document for:   Discussion and Decision

Introduction

In RAN#94-e, the work item for NR sidelink evolution was approved [1]. The work item objective for NR Sidelink evolution was further revised and agreed in RAN#98-e [2]. In RAN#99, the pending issue regarding the decision about CA objective 1 in the WID was discussed and tentatively agreed [3]. However, it was agreed to check the status of WI again in RAN#100 for possible down-scoping.

This paper discusses the overall progress of WI, as well as how to proceed regarding the potential decision of the above objective.

Discussions

During the discussion of original WID [1], most companies expressed concern that the scope of the WID is beyond the available TU. From the experience of Rel-17 Sidelink, where the whole WI has to be delayed by one quarter as a result of uncompleted remaining issues, it is expected that the likelihood of completing all four objectives is almost zero.

But anyway, all four objectives were pushed into WID. Fortunately, due to strong request from many concerned companies, the approved WID [2] postponed the discussion of the following two objectives.

To check in RAN#97 for objectives 1 and 3, taking into account the progress on objectives 2 and 4, aiming to have specification work for both objective 1 and 3.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#97)

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to FR1 licensed spectrum and ITS band in FR1.

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (This part of the work is put on hold until further checking in RAN#97)

- Update evaluation methodology for commercial deployment scenario

- Work is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible.

  - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In RAN#97-e and RAN#98-e, the WID was further discussed, and the following conclusions were reached:

Co-exsistence (RAN#97-e)

- For the Co-Ex objective, RAN1 continues the work on dynamic resource pool sharing based on existing agreements dnd WID with high priority for Type A devices and operating combination A.

- Start the FR2 objective from RAN#97-e and focus only on updating evaluation methodology in 4Q 2022. Determine in RAN#98-e, whether to continue the study or study+specification work for FR2 until the end of R18.

- For the CA objective, continue to put on-hold and determine whether it can be started in RAN#98-e.

Note it was decided that the CA objective would be revisited in March 2023 (RAN#99), including the possibility of including it in Rel-18 with minimal WG efforts, or dropping it from Rel-18 but re-consider it in Rel-19.

In RAN#99, the overall situation was evaluated. The majority of companies, as well as the WG chair, again raised concern about the progress of the WI, which is almost behind schedule. But instead of implementing necessary solutions to ensure on-time completion of the WI, the scope was expanded by deciding to re-introduce CA into the WID [3]:

Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4]

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to intra-band CA for the ITS band in FR1 (Band n47).

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Only Mode 2 operation

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Time resources for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

- No consideration for limited transmission and reception capability

- No primary/secondary carrier differentiation

- Reuse the LTE sidelink CA design for the following aspects:

  - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication

- The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18.

- Note: The SL CA work in Rel-18 mainly targets some V2X use cases

Although this watered-down version is a last attempt to demonstrate that these objectives can be completed within the Rel-18 time frame, but considering there is only one RAN1 meeting and there are so many remaining issues on other SL objectives in the last RAN1 meeting, even without the addition of CA objectives, the timely completion of WI is high questionable. In fact, from the draft SR distributed by the rapporteur, there are many important remaining issues left:

- Specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only.

  - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

    - Remaining details of cyclic prefix extension for COT sharing case

    - Remaining details of EDT, COT-SI and stop-resume behavior in a shared channel occupancy

    - Remaining details of multi-channel access procedures for S-SSB

    - Remaining resource allocation details of multi consecutive slots transmission (MCSt)

    - Whether any update necessary on SL mode 1 (LBT failure reporting to gNB) and mode 2 RA (consistent LBT failure)

    - Remaining SCI-1 and SCI-2 design for SL-U (joint design with PHY structure agenda)

  - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

    - Remaining details of PSCCH/PSSCH transmission (e.g., 1 or 2 AGC symbols, resource selection impact, etc.)

    - Remaining details of PSFCH transmission (e.g., PSSCH-PSFCH mapping, etc.)

    - Remaining details of N PSFCH occasion(s) (e.g., location, UE behaviour, etc.)

    - Remaining details of S-SSB transmission (e.g., gap/repetition value, more than one RB set, etc.)

    - Remaining details of additional S-SSB occasions (e.g., gap/K value, etc.)

    - Whether any other update necessary on the existing SL procedure (e.g., power control, etc.)

    - Remaining SCI-1 and SCI-2 design for SL-U (joint design with channel access and RA agenda)

- Study support of beam management for NR sidelink in FR2 licensed spectrum

  - Further details on initial beam paring, including at least the following

    - Reference signal(s) associated with candidate procedures where initial beam pairing is performed before and during unicast establishment

    - Beam reporting and beam indication

  - Further details on beam maintenance procedure including at least sidelink CSI-RS periodicity and multiple sidelink CSI-RS in a slot

  - Further details on beam failure recovery procedure including at least when BFI is triggered by PSFCH carrying SL-HARQ feedback

Even for the objectives for dynamic resource pool sharing for LTE/NR V2X co-channel coexistence, it would be incorrect to claim that there are no remaining issues. For example, the following two issues, still need to be concluded:

Remaining details about NR SL resource exclusion’s impact on NR SL re-evaluation procedure.

Remaining details about NR SL resource exclusion’s impact on NR SL pre-emption procedure.

This is because re-evaluation/pre-emption are two important features of the NR SL, if not most. With the introduction of co-channel co-existence, the impact of these procedures needs to be carefully evaluated and necessary specifications need to be introduced. From the previous discussion, some companies want to re-interpret legacy Rel-17 specification for this purpose. However, other companies disagree. Nevertheless, even for re-interpretation, consensus is needed. Currently, the discussion is left without any conclusion. Therefore, the Rel-18 SL specification is incompletion.

Given the current, starting with new objectives is not desirable. To proceed, it is evident that further down-scope have to be made. Rather than starting new objective and further delay the completion of the whole WI into Rel-19 time frame, we should postpone the start of the new objective into Rel-19 and guarantee the completion of the current scope in Rel-18 time frame. Therefore, we have the following observation and proposals:

Observation 1: The current work progress is behind schedule and the list of remaining open issues are so substantial that it is unlikely for the WI to be completed by the last WG meeting.

Observation 2: Even for the objectives for dynamic resource pool sharing for LTE/NR V2X co-channel coexistence, whose completion is the pre-requisite of starting CA, there are still remaining issues.

Proposal 1:  For Rel-18 SL Evo, to avoid the delay of the WI completion, further scope reduction is needed.

Proposal 2:  CA objectives in the WID shall be postponed to Rel-19.

Another issue needs some discussion is objective 3 sidelink operation on FR2 licensed spectrum :

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2]

- Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

- Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2]

  - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

It can be seen that in order to accommodate the objective into limited available TU the study is only limited to the support of sidelink beam management , including initial beam-pairing, beam maintenance, and beam failure recovery, etc. Also, by ‘reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible’, the wording in the WID indicates that unless necessary the enhancement should not be conducted. This also applies to various resource allocation enhancement scheme associated with the beam based transmission. Same as other NR features, there are certainly enhancement that can be done beyond basic beam management functionality. However, the limited TU cannot accommodate such discussions. To ensure a timely completion of the Rel-18 SL WI, these aspects can be address in a continued study phase of the Rel-19 SL WI.

Proposal 3: The study of FR2 enhancement should strictly follow the agreed wording of the objectives and confined to the support of basic beam management mechanisms. Further enhancement, such as multiple beams support, resource allocation improvements with beamformed transmission, etc, can be considered in a continued study phase in the Rel-19 SL Evo WI.

Conclusion

In this contribution we discuss the progress of the WI and potential down-scope required. We have the following observation and proposal.

Observation 1: The current work progress is behind schedule and the list of remaining open issues are so substantial that it is unlikely for the WI to be completed by the last WG meeting.

Observation 2: Even for the objectives for dynamic resource pool sharing for LTE/NR V2X co-channel coexistence, whose completion is the pre-requisite of starting CA, there are still remaining issues.

Proposal 1:  For Rel-18 SL Evo, to avoid the delay of the WI completion, further scope reduction is needed.

Proposal 2:  CA objectives in the WID shall be postponed to future release.

Proposal 3: The study of FR2 enhancement should strictly follow the agreed wording of the objectives and confined to support of basic beam management mechanisms. Further enhancement, such as multiple beams support, resource allocation improvements with beamformed transmission, etc, can be considered in a continued study phase in the Rel-19 SL Evo WI.

Reference

- RP-213678, “New WID on NR sidelink evolution”, OPPO, LGE

- RP-222806, “ WID revision: NR sidelink evolution  ”,  OPPO

- RP-230077, “ WID revision: NR sidelink evolution  ”,  OPPO

- RP-222691, “Draft report of electronic meeting RAN #97e”, ETSI MCC

- RP-221937, “Status report for WI NR sidelink evolution”, OPPO

- R1-2203981, “work plan for NR sidelink evolution”, OPPO, LGE