3GPP TSG RAN Meeting #100	RP-231397

Taipei, June 12 - 14, 2023

Agenda Item:		9.3.1.8

Source:		    ZTE, Sanechips

Title:				Discussion on Rel-18 network energy saving

Document for:    Discussion and Decision

- Introduction

In RAN#98 meeting, the Rel-18 WID[1] for network energy savings for NR was approved to specify network energy saving techniques for NR. The corresponding SI phase has been completed in RAN#98e meeting[2]. During the SI phase, it has been concluded that the power consumption of radio access network can be split into two parts: the dynamic part which is only consumed when data transmission/reception is ongoing, and the static part which is consumed all the time to maintain the necessary operation of the radio access devices, even when the data transmission/reception is not on-going. For SCell with low or no DL traffic load, SSB transmission occupies most of the DL occasions, in which NW cannot enter into sleep mode or shut down the transmitter for the sake of energy saving. So SSB-less SCell operation is an important solution to reduce or optimize NW power consumption.

The objectives of this Rel-18 WID are listed as below and the SSB-less related content is highlighted.

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3] - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

In this contribution, the RAN4 progress during recent two RAN4 meetings are summarized. At the same time, some clarification on the scenarios of SSB-less SCell operation are provided to move forward the intention of NW energy saving in this Rel-18 WI.

- Discussion

  - RAN4 progress

In RAN4, three scenarios of SSB-less scenarios (i.e. scenario 1/2/2a) have been identified.  Among the three scenarios, based on the previous discussions in RAN4#107 meeting, Scenario 2 is agreed to be deprioritized, while both Scenario 1 and Scenario 2a are agreed to be prioritized scenarios for SSB-less SCell operation.

Topic #1: SSB-less SCell operation Sub-topic 1-1: Scenarios Issue 1-1-1/2/3: Scenario 1 / 2 / 2a Agreements - Continue RAN4 work on the following SSB-less SCell scenarios - Scenario 1: SCell without SSB transmission and with TRS transmission - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective. - Deprioritize RAN4 work on the following SSB-less SCell scenario - Scenario 2: SCell without SSB transmission and without TRS transmission - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

Observation 1: Based on current RAN4 progress in this Rel-18 WID, Scenario 2 is deprioritized, both Scenario 1 and 2a are prioritized for SSB-less SCell operation.

However, during the discussion in RAN4#107 meeting, there was no consensus reached to send LS and session chair recommended to further clarify the scenarios in the scope of the WI in the RANP.

In the following section, we provide some further clarification on Scenario 2a from the perspective of the motivation, the network energy saving gain, the RRM impact and the potential RAN2 impacts.

  - Scenario discussion

### 2.2.1. Motivation of Scenario 2a

There is clear demand for Scenario 2a in practice. A couple of use cases with heavy uplink traffic load were identified in NR, including remote driving, machine vision and factory video surveillance etc. Under these use cases, UL traffic would be much heavier with limited DL data traffic. In PCell, TDD configuration is often designed such that it supports more DL slots which is not UL friendly.  So, the SSB-less SCell can be mainly used for the support of additional UL traffic under such UL heavy use cases. The DL traffic and NW signalling would be off-loaded to the PCell or other SCell. Since only UL traffic could be scheduled in the SSB-less SCell, network can shut down the transmitter at the SSB-less SCell accordingly from the energy power saving perspective. Therefore, significant network power saving gain could be foreseeable.

- Observation 2: Especially for the use cases of heavy UL traffic with limited DL transmission, SSB-less SCell can provide an energy efficient solution of supporting the additional UL traffic without DL traffic on the SCell by shutting down the NW transmitter of SSB-less SCell.

Besides, another motivation of proposing Scenario 2a is due to the concerns from other two interested scenarios (e.g. Scenario 1 and Scenario 2). Regarding Scenario 2, UE vendors has already shown concerns on the potential performance degradation of PDCCH/PDSCH due to lack of fine T/F synchronization which is used to be performed based on TRS, however in the Scenario 2, there are still some downlink traffic scheduled without any downlink TRS signal transmission. Regarding Scenario 1, the performance gain of NW power saving in practice is still doubtful as A-TRS should be QCL-ed to P-TRS signal even without SSB transmission in the SSB-less SCell. In short, P-TRS is still mandatory transmission based on the current specification. With a kind of always-on signal for P-TRS, it is not clear whether obvious NW power saving can be achieved even without SSB while P-TRS has to be transmitted by the NW in Scenario 1.

- Observation 3: Both Scenario 1 and Scenario 2 have some drawbacks. Scenario 2 may have challenge to make sure there is no performance degradation of PDCCH/PDSCH. The network power saving gain under Scenario 1 is expected to be limited due to the existence of always-on P-TRS transmission.

As a result, Scenario 2a is proposed as an important solution to realize NW power saving under the scope of SSB-less SCell operation. In addition, Scenario 2a is fully aligned with the objective of this WID and consistent with the SSB-less SCell operation.

- Observation 4: Scenario 2a is an important scenario to achieve NW power saving which is also fully aligned with the objective of this WID and consistent with the SSB-less SCell operation.

- Proposal 1: RAN tasks RAN4/RAN2 to support Scenario 2a and specify the corresponding requirements.

### 2.2.2. Network energy saving gain of Scenario 2a

During the study phase, multiple energy consumption model and technical analysis are provided in [2] from the perspective of NW power saving. For TDD, the NW power consumption for DL and UL are separately modeled, according to the relative power values in Table 5.1-3[2], it can be found that the power consumption of active UL is obviously much less than that of active DL. For FDD, it is summarized that “For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study” in Chapter 5.1[2].

Table 5.1-3: Relative power values P for reference configuration Set 1, Set 2 and Set 3

| Power state | BS Category 1 |  |  | BS Category 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| Deep sleep | 1 |  |  | 1 |  |  |
| Light sleep | 25 |  |  | 2.1 |  |  |
| Micro sleep | 55 | 50 | 38 | 5.5 | 5 | 3 |
| Active DL | 280 | 200 | 152 | 32 | 26 | 17.6 |
| Active UL | 110 | 90 | 80 | 6.5 | 5.8 | 4.2 |

Therefore, shutting down the NW DL transmitter at the SSB-less SCell could basically achieve significant power saving. For BS Category 1, 65.5% to 71.7% of network power saving gain could be achieved and for BS Category 2, 80.7% to 83.3% of power saving gain could be achieved. So Scenario 2a is beneficial to achieve the significant NW power saving gain based on the SSB-less SCell operation, which is one of the efficient scenarios and requested by the NR use cases in practice.

- Observation 5: By shutting down the NW DL transmitter at the SSB-less SCell, comparing with enabling both DL transmitter and UL receiver, significant NW power saving can be achieved.

### 2.2.3. RRM impact of Scenario 2a

Under Scenario 2a, there is no DL transmission from NW, NW only needs to receive UL. So several steps within the procedure of the SSB-less SCell activation can be skipped, and the SSB-less SCell activation delay can be reduced down to 3 ms as same as the the case of SCellwithoutSSB in legacy intra-band CA. Since there is no need to receive DL signal, to perform the AGC, to conduct the fine T/F tracking and TCI state indication. UE only needs to obtain the time synchronization of the SSB-less SCell from Pcell or other Scell downlink as the basis of UL synchronization.

- Observation 6: For Scenario 2a, multiple steps such as AGC, fine T/F tracking and TCI state indication can be skipped during the SCell activation procedure. UE only needs to obtain the time synchronization from Pcell or other Scell downlink as the basis of UL synchronization of SSB-less Scell.

- Observation 7: Scell activation delay for Scenario 2a could be reduced down to 3ms which is the same as MAC-CE decoding time for Scell activation.

### 2.2.4. Potential RAN2 impact of Scenario 1 and Scenario 2a

Per the above objective in the WID [1], RAN2 has been identified as the responsible working group to specify the SSB-less SCell operation for inter-band CA.

With the latest RAN4 agreement from RAN4#107 as mentioned in section 2.2.1 to continue work on scenario 1 and 2a, the potential RAN2 impact has been analyzed and discussed with agreements made in RAN2#122:

---------------------------------------------------------------------------------------------------------------------------------------

RAN2#122-Agreements

1.	If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

2.	If RAN4 concludes it is feasible, RAN2 can further work on at least the following specification impacts:

-	RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

-	UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

---------------------------------------------------------------------------------------------------------------------------------------

RAN2 understands at least two issues require specification update, which are common for both scenario 1 and 2a:

- Issue 1: Timing reference of SSB-less SCell

In the field description for absoluteFrequencySSB indicating that no SSB frequency is absent for this cell if the field is defined for the SCell-less SCell only for the intra-band case, as highlighted below:

FrequencyInfoDL field descriptions

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is only supported in case the SCell for which the UE obtains the timing reference is in the same frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting RedCap, on handover, corresponds to the cell-defining SSB.

To support scenario 1 and 2a, the field description needs to be updated, e.g. by saying that “If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is only supported in case the SCell for which the UE obtains the timing reference is in the same frequency band or a different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.”

- Issue 2: UE capability reporting

A UE capability scellWithoutSSB is defined to indicate whether the UE supports configuration of SCell that does not transmit SSB in the intra-band CA case as shown below:

| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA but not supported for inter-band CA. | FS | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

To support scenario 1 and 2a, the capability needs to be updated, e.g. by saying that “Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signaling for intra-band CA but not supported for and/or inter-band CA.”

Observation 8: At least the following specification impacts require RAN2 work to support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, which are common for both scenario 1 and 2a:

- RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

- UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

Some other enhancement can also be considered for UE to turn off its receiver RF chain to further reduce power consumption in scenario 2a with the following examples given:

- Example 1: Explicit NW indication to inform UE that there would be no other DL transmission and UE would turn off RF upon reception of such indication.

- Example 2: Allow UE to turn off its receiver RF chain if PDCCH/PDSCH is not configured in SCell.

- Conclusion

In this contribution, we summarize the RAN4 progress on SSB-less SCell operation and give some further clarification on Scenario 2a. The following observations and proposals are suggested.

Observation 1: Based on current RAN4 progress in this Rel-18 WID, Scenario 2 is deprioritized, both Scenario 1 and 2a are candidates for SSB-less SCell operation.

- Observation 2: For the use cases of UL traffic heavy with quite limited DL transmission, the SSB-less SCell could be mainly used for the UL traffic supplementation by shutting down the NW transmitter of SSB-less SCell.

- Observation 3: Both Scenario 1 and Scenario 2 have some drawbacks. Scenario 2 leads to potential performance degradation of PDCCH/PDSCH. The network power saving gain under Scenario 1 should be limited due to the existence of always on P-TRS transmission.

- Observation 4: Scenario 2a is an important candidate scenario to achieve NW power saving which is also fully aligned with the objective of this WID and consistent with the SSB-less SCell operation.

- Proposal 1: RAN tasks RAN4/RAN2 to support Scenario 2a and specify the corresponding requirements.

- Observation 5: By shutting down the NW DL transmitter at the SSB-less SCell, comparing with enabling both DL transmitter and UL receiver, significant NW power saving is achieved.

- Observation 6: For Scenario 2a, multiple steps such as AGC, fine T/F tracking and TCI state indication can be skipped during the SCell activation procedure. UE only needs to obtain the time synchronization from Pcell or other Scell downlink as the basis of UL synchronization of SSB-less Scell.

Observation 7: Scell activation delay for Scenario 2a could be reduced down to 3ms which is the same as MAC-CE decoding time for Scell activation.

Observation 8: At least the following specification impacts require RAN2 work to support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, which are common for both scenario 1 and 2a:

- RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

- UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

- References

RP-223540, “New WID: Network energy savings for NR”, Huawei

RP-222821, 3GPP TR 38.864 V1.0.0, Study on network energy savings for NR.

R4-2306369, “WF on RRM requirements for NR network energy saving”, Huawei

R4-2310086, “WF on Network Energy Saving RRM requirements”, Huawei