3GPP TSG RAN Meeting #100	RP-231470

Taipei, June 12-14, 2023

Source:	Moderator (AT&T)

Title:	pCR for TR 38.848 on Deployment scenarios and their characteristics

Agenda item:	9.2.2

Document for:	Agreement

- Introduction

This pCR proposes text to add Deployment scenarios and their characteristics on Ambient IoT into TR 38.848 v0.1.0.

- Proposed changes

# <Unchanged text is omitted>

## 4.2	Deployment scenarios and connectivity topologies

Deployment scenarios for Ambient IoT have been studied on the basis of a list of characteristics, and the representative use case(s) applicable to a scenario. The possible descriptions of the characteristics are as follows:

Table 4.2-1: Characteristics of deployment scenarios

| Characteristic | Possible description entries |
| --- | --- |
| Environment (of the device) | Indoor Outdoor Indoor or outdoor |
| Basestation characteristic (if any) | Macro-cell-based deployment Micro-cell-based deployment Pico-cell-based deployment None |
| Connectivity topology | See section 4.2.1 |
| Spectrum | Licensed FDD Licensed TDD Unlicensed |
| Coexistence with existing 3GPP technologies | Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type. Deployed on new sites without an assumption of an existing 3GPP deployment. |
| Traffic assumption | Device-terminated (DT) Device-originated (DO) DO traffic includes DO autonomous (DO-A), and DO device-terminated triggered (DO-DTT) |
| Device characteristic | See Section 4.1: Device A Device B Device C |

# The study has considered Ambient IoT deployments in-band to NR, in guard-band of NR, and in standalone band from NR.

# <Unchanged text is omitted>

### 4.2.2	Deployment scenarios

#### 4.2.2.1	Deployment scenario 1: Device indoors, basestation indoors

With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1.

Table 4.2.2.1-1: Characteristics of deployment scenario 1

| Applicable representative use cases | Characteristics | Description (NOTE 1) |  |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |  |
|  | Basestation characteristic (if any) | Micro- or pico-cell |  |
|  | Connectivity topology | Topology (1), (2), (3) |  |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |  |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |  |
|  | Traffic assumption | DT and DO |  |
|  | Device characteristic | Device A or Device B or Device C |  |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.2	Deployment scenario 2: Device indoors, basestation outdoors

With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.1-2.

Table 4.2.2.1-2: Characteristics of deployment scenario 2

| Applicable representative use cases | Characteristics | Description (NOTE 1) |  |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |  |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |  |
|  | Connectivity topology | Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor |  |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |  |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |  |
|  | Traffic assumption | DT and DO |  |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2), Device B may support Topology (2), (3) |  |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.3	Deployment scenario 3: Device indoors, UE-based reader

With Ambient IoT device indoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.1-3.

Table 4.2.2.1-3: Characteristics of deployment scenario 3

| Applicable representative use cases | Characteristics | Description (NOTE 1) |  |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |  |
|  | Basestation characteristic (if any) | None |  |
|  | Connectivity topology | Topology (4) |  |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | - |
|  | Coexistence with existing 3GPP technologies | NA |  |
|  | Traffic assumption | DT and DO |  |
|  | Device characteristic | Device A or Device B or Device C |  |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.4	Deployment scenario 4: Device outdoors, basestation outdoors

With Ambient IoT device outdoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.1-4.

Table 4.2.2.1-4: Characteristics of deployment scenario 4

| Applicable representative use cases | Characteristics | Description (NOTE 1) |  |
| --- | --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor |  |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |  |
|  | Connectivity topology | Topology (1), (2), (3) |  |
|  | Spectrum | Licensed FDD, licensed TDD, or unlicensed. |  |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |  |
|  | Traffic assumption | DT and DO |  |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2),Device B may support Topology (2), (3) |  |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.5	Deployment scenario 5: Device outdoors, UE-based reader

With Ambient IoT device outdoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.1-5.

Table 4.2.2.1-5: Characteristics of deployment scenario 5

| Applicable representative use cases | Characteristics | Description (NOTE 1) |  |
| --- | --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor |  |
|  | Basestation characteristic (if any) | None |  |
|  | Connectivity topology | Topology (4) |  |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | - |
|  | Coexistence with existing 3GPP technologies | NA |  |
|  | Traffic assumption | DT and DO |  |
|  | Device characteristic | Device A or Device B or Device C |  |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

# <Unchanged text is omitted>