3GPP TSG RAN Meeting #101						   RP-231957

Bangalore, India, September 11-15, 2023

Agenda Item:	9.3.2.7

Source:	THALES, Huawei, Ericsson

Title:	On Network Verified UE Location for Rel-18 NR NTN

Document for:	Discussion/Decision

# Introduction

The work item descriptions of Rel-18 NR NTN were updated in RAN #100 meeting [1]. According to the updated WID, the objective of network verified UE location starts after RAN#98-e meeting, to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view.

The mirror positions ambiguity for multi-RTT positioning method remained unresolved until the RAN1#114 meeting. The RAN1#114 meeting [2] stated that three methods could be used to solve the ambiguity without impact on RAN1 specifications from RAN1 perspective:

- By gNB or LMF implementation

- Existing ECID method

- UL-AoA

In this contribution, we propose to add a note to the NR NTN enhancements work item description [1] of Network Verified UE location objective about the mirror ambiguity resolution method.

# Discussion

RAN1 concluded that existing E-CID method could be used to solve mirror positions ambiguity that could appear according with the geometry when the multi-RTT positioning method with a single satellite in view is used.

RAN1 sees no impact of the method on the specification. However, E-CID might be updated to support NTN scenario (e.g. mapped cell ID).

Observation 1: “Mapped Cell ID” used in NTN for cell mapping is not listed in Measurement Quantity value of E-CID MEASUREMENT INITIATION REQUEST

Observation 2: “Mapped Cell ID” used in NTN for cell mapping is not listed as IE of E-CID MEASUREMENT INITIATION RESPONSE

Moreover, others impacts on RAN3 specifications should be studied to support the E-CID based-method for network verified UE location in Rel-18. For that, an update in the WID [1] is necessary to include E-CID in the field of study and specification. We propose to add a NOTE for this.

Proposal 1: add the following NOTE in the network verified UE location objective in the NR NTN enhancement WID, “Note: E-CID method could be used in complement to multi-RTT method to solve the mirror-image ambiguity. E-CID method should be analyzed and enhanced, if needed for NTN scenario.”

# Conclusion

Based on the discussions above, we have the following observation and proposals.

Observation 1: “Mapped Cell ID” used in NTN for cell mapping is not listed in Measurement Quantity value of E-CID MEASUREMENT INITIATION REQUEST

Observation 2: “Mapped Cell ID” used in NTN for cell mapping is not listed as IE of E-CID MEASUREMENT INITIATION RESPONSE

Proposal 1: add the following NOTE in the network verified UE location objective in the NR NTN enhancement WID, “Note: E-CID method could be used in complement to multi-RTT method to solve the mirror-image ambiguity. E-CID method should be analyzed and enhanced, if needed for NTN scenario.”

# Reference

- RP-231484, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements,” June 2023.

- Chairman’s Notes, 3GPP TSG RAN1 #114 meeting, August 2023.