3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230701

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0935 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_ECall_MIME_GetMSD_ContentId |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As can be seen in RFC8147 9.1.1.1. Attributes of the <ack> Element ref element should reference the Content-ID of the body part being acknowledged. Ttcn3 implementation derives this value above mentioned from MIME header Content-ID which value is an url encased in brackets, ref value should be the same content but removing the brackets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added f_IMS_ECall_MIME_GetMSD_ContentId function to return the required url without being encased in bracktes “<>”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Legacy and NR eCall test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	f_IMS_ECall_MIME_GetMSD_ContentId	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## f_IMS_ECall_MIME_GetMSD_ContentId

| Function name | f_IMS_ECall_MIME_GetMSD_ContentId |
| --- | --- |
| Reason for change | As can be seen in RFC8147 9.1.1.1. Attributes of the <ack> Element ref element should reference the Content-ID of the body part being acknowledged. Ttcn3 implementation derives this value above mentioned from MIME header Content-ID which value is an url encased in brackets, ref value should be the same content but removing the brackets. |
| Summary of change | Added f_IMS_ECall_MIME_GetMSD_ContentId function to return the required url without being encased in bracktes “<>”. |
| TTCN module | Common\IMS\IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_ECall_MIME_GetMSD_ContentId(MessageBody p_MessageBody) runs on IMS_PTC return charstring { var MIME_Part v_MIME_Part; v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(p_MessageBody.mimeMessageBody, tsc_MimeType_EmergencyCallDataMSD); return v_MIME_Part.header.contentId.msgId; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_ECall_MIME_GetMSD_ContentId(MessageBody p_MessageBody) runs on IMS_PTC return charstring { var MIME_Part v_MIME_Part; v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(p_MessageBody.mimeMessageBody, tsc_MimeType_EmergencyCallDataMSD); return f_MIME_ContentId_GetAddrSpec(v_MIME_Part.header.contentId); } <<SKIPPED CODE>>