3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230893

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0938 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_MOCallHoldResume_BuildSDP_OfferRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When running 34.229-1 test cases where a Call “On Hold” intervenes the UE may include, according to IR.92 IMS profile an EVS codec in first SDP offer (Clause 2.4.3.3) although during the call establishment AMR-WB will be agreed. Later on, when Holding the call, initial INVITE is going to be taken to determine expected codecs in offer for next INVITE to put the call “On Hold”. This will cause test case to fail as initial INVITE offer codec included EVS but AMR-WB was then agreed thus SS expecting an EVS (for Hold INVITE) while UE includes AMR-WB, codec agreed during call establishment. All the cases where this function is used, will require specific codecs to be included in SDP, will only depend on RAT being tested: AMR-WB for Legacy IMS and EVS for NR IMS test cases. Function is simplified depending on RAT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a condition to expect always a AMR-WB codec for Hold procedures for Legacy IMS test cases, independant on initial offer codecs included. Simplified function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.15.11, G.15.13, G.15.20 & G.15.23. 15.11, 15.11a, 15.21a, 15.21c,15.25 & 19.1.5 8.26.NR5GC, 8.27.NR5GC, 8.34.NR5GC, 8.35.NR5GC & 8.36.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_IMS_MOCallHoldResume_BuildSDP_OfferRX	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_IMS_MOCallHoldResume_BuildSDP_OfferRX

| Function name | f_IMS_MOCallHoldResume_BuildSDP_OfferRX |
| --- | --- |
| Reason for change | When Holding the call, initial INVITE is going to be taken to determine expected codecs in offer for next INVITE to put the call “On Hold”. This will cause test case to fail as initial INVITE offer codec included EVS but AMR-WB was then agreed thus SS expecting an EVS (for Hold INVITE) while UE includes AMR-WB, codec agreed during call establishment. |
| Summary of change | Added a condition to expect always a AMR-WB codec for Hold procedures for Legacy IMS test cases, independant on initial offer codecs included. Simplified function. |
| TTCN module | Common\IMS\IMS_Procedures_CallHold.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_MOCallHoldResume_BuildSDP_OfferRX(SDP_Message p_SDP_Message_PrevOffer) runs on IMS_PTC return template (present) SDP_Message { /* @sic R5-145726: use of f_IMS_BuildSDP_UpdatedOfferAudio_RX, f_IMS_BuildSDP_UpdatedOfferAudioVideo_RX sic@ */ var template (present) SDP_Message v_SDP_Message; var boolean v_ResourcesEstablished := true; var IMS_SDP_AudioCodec_Type v_AudioCodec := EVS_16000; var IMS_SDP_VideoCodec_Type v_VideoCodec := H265_90000; var SDP_media_desc v_SDP_Media_Desc_Audio := p_SDP_Message_PrevOffer.media_list[0]; var SDP_media_desc v_SDP_Media_Desc_Video; if (f_SDP_MediaDescr_GetAudioCodec(v_SDP_Media_Desc_Audio, v_AudioCodec) == "") { // @sic R5-211440: EVS_16000 is used for NR5GC sic@ v_AudioCodec := AMR_16000; } if (lengthof(p_SDP_Message_PrevOffer.media_list) == 1) { // => audio only v_SDP_Message := f_IMS_BuildSDP_UpdatedOfferAudio_RX(p_SDP_Message_PrevOffer, v_ResourcesEstablished, v_AudioCodec); } else { v_SDP_Media_Desc_Video := p_SDP_Message_PrevOffer.media_list[1]; if (f_SDP_MediaDescr_GetVideoCodec(v_SDP_Media_Desc_Video, v_VideoCodec) == "") { // @sic R5s211582: H265_90000 is used for NR5GC sic@ v_VideoCodec := H264_90000; } v_SDP_Message := f_IMS_BuildSDP_UpdatedOfferAudioVideo_RX(CallHold, p_SDP_Message_PrevOffer, v_AudioCodec, v_VideoCodec); } return v_SDP_Message; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_MOCallHoldResume_BuildSDP_OfferRX(SDP_Message p_SDP_Message_PrevOffer) runs on IMS_PTC return template (present) SDP_Message { /* @sic R5-145726: use of f_IMS_BuildSDP_UpdatedOfferAudio_RX, f_IMS_BuildSDP_UpdatedOfferAudioVideo_RX sic@ */ var template (present) SDP_Message v_SDP_Message; var boolean v_ResourcesEstablished := true; var boolean v_IsNR5GC := f_IMS_PTC_RanTypeIsNR5GC(); var IMS_SDP_AudioCodec_Type v_AudioCodec := AMR_16000; var IMS_SDP_VideoCodec_Type v_VideoCodec := H264_90000; if (v_IsNR5GC) { // @sic R5-211440: EVS_16000 is used for NR5GC sic@ v_AudioCodec := EVS_16000; v_VideoCodec := H265_90000; } if (lengthof(p_SDP_Message_PrevOffer.media_list) == 1) { // => audio only v_SDP_Message := f_IMS_BuildSDP_UpdatedOfferAudio_RX(p_SDP_Message_PrevOffer, v_ResourcesEstablished, v_AudioCodec); } else { v_SDP_Message := f_IMS_BuildSDP_UpdatedOfferAudioVideo_RX(CallHold, p_SDP_Message_PrevOffer, v_AudioCodec, v_VideoCodec); } return v_SDP_Message; } <<SKIPPED CODE>>