3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230711

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0192 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for positioning test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following implementation of TTCN CR R5s230502, new templates car_POS_SYSTEM_LoadScenario and car_POS_SYSTEM_Start are defined for Confirmaiton messages and included in f_PosSystem_LoadScenario and f_PosSystem_Start respectively to receive feedback of when the scenario is actually configured and started. But the CnfFlag is set to “false” in the associated request templates, so TTCN will currently not wait for confirmation and the CNF ASPs will cause test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected CnfFlag values to “true” in POS_SYSTEM_CTRL_REQ templates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail some test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1, 7.2.2.2, 7.3.3.1, 7.3.4.1, 7.3.4.2, 7.3.4.3, 7.3.4.4, 7.3.5.1 9.3.1.2, 9.3.3.1, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | Template cas_POS_SYSTEM_Start |
| --- | --- |
| Reason for change | Following implementation of TTCN CR R5s230502, new templates car_POS_SYSTEM_LoadScenario and car_POS_SYSTEM_Start are defined for Confirmaiton messages and included in f_PosSystem_LoadScenario and f_PosSystem_Start respectively to receive feedback of when the scenario is actually configured and started. But the CnfFlag is set to “false” in the associated request templates, so TTCN will currently not wait for confirmation and the CNF ASPs will cause test case failure. |
| Summary of change | Corrected CnfFlag values to “true” in POS_SYSTEM_CTRL_REQ templates. |
| TTCN module | PosSystem_Control.ttcn |
| MCC160 Comment |  |

Before change

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_Start( PositioningSystemList_Type p_PosSystemListId ) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := false,

Request := {

Start := true

}

};

After change

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_Start( PositioningSystemList_Type p_PosSystemListId ) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := true,

Request := {

Start := true

}

};

## Change 2

| Function name | Template cas_POS_SYSTEM_LoadScenario |
| --- | --- |
| Reason for change | Following implementation of TTCN CR R5s230502, new templates car_POS_SYSTEM_LoadScenario and car_POS_SYSTEM_Start are defined for Confirmaiton messages and included in f_PosSystem_LoadScenario and f_PosSystem_Start respectively to receive feedback of when the scenario is actually configured and started. But the CnfFlag is set to “false” in the associated request templates, so TTCN will currently not wait for confirmation and the CNF ASPs will cause test case failure. |
| Summary of change | Corrected CnfFlag values to “true” in POS_SYSTEM_CTRL_REQ templates. |
| TTCN module | Common\ PosSystem_Control.ttcn |
| MCC160 Comment |  |

Before change

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_LoadScenario(PositioningSystemList_Type p_PosSystemListId,

template (value) PositioningScenario_Type p_PositioningScenario) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := false,

Request := {

LoadScenario := p_PositioningScenario

}

};

After change

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_LoadScenario(PositioningSystemList_Type p_PosSystemListId,

template (value) PositioningScenario_Type p_PositioningScenario) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := true,

Request := {

LoadScenario := p_PositioningScenario

}

};