3GPP TSG RAN #102                                         RP-233111

Edinburgh, GB, December 11 – 15, 2023

Agenda Item:	9.1.2.4

Source:	RAN1 Vice-chair (CMCC)

Title:	Moderator's summary on new SI Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

Document for:	Discussion and Decision

- Introduction

This document provides the summary of all the contributions submitted to 9.1.2.4 agenda item (Study on AI (Artificial Intelligence)/ML (Machine Learning) for Mobility) of RAN#102 meeting. Contributions [1]~[30] present their views about the justification and working scope for AI mobility for NR system.

As per RAN Chair’s guidance, the following potential objectives should be discussed:

- Discussion

### 2.1 Type of mobility (including RRC state)

The related discussion/proposals in companies’ contributions are copied as below:

Table 2.1-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232780 | Ericsson | both L3-based mobility and L1/L2-triggered mobility (LTM) should be considered in the SI Note, we assume that CHO is not part of the scope, and we don’t think it should be added since the scope would be too big |
| RP-232876 | NEC | Proposal 1: New SI for AI/ML-based mobility includes both AI/ML for L1/L2 triggered mobility (LTM) and AI/ML for L3-based mobility. |
| RP-232911 | OPPO | Proposal 4: AI/ML-based mobility focuses on connected mode UEs only. Proposal 5: L3 mobility and LTM will both be studied for AI/ML-based mobility enhancement. |
| RP-232918 | Samsung | Proposal 1: The entire SI should focus on L3-based mobility. |
| RP-232984 | Spreadtrum | Proposal 1: L3-based mobility and L1/L2-triggered mobility (LTM) are both considered. Proposal 4: The support of AI/ML-based model/function in target cell should be considered during HO and Cell reselection. |
| RP-233059 | vivo | Proposal 1: For the operation mode, focus on standalone mode during the SI phase, i.e., DC is not in the SI scope. Proposal 2: For the RRC state of mobility, in addition to HO in RRC_CONNECTED, Cell Reselection in RRC_INACTIVE/IDLE can also be considered. Proposal 3: For the mechanisms of mobility, further clarify that Conditional HO is also included in L3-based mobility. |
| RP-233216 | QC | Both L3 based mobility and LTM |
| RP-233277 | MTK | L3-based mobility (including. basic HO in Rel-15 and CHO in Rel-16) and L1/L2-triggered mobility (LTM) are both considered |
| RP-233316 | HW | Study AI/ML for L3 based handover in Rel-19. (HO+CHO) |
| RP-233427 | CMCC | Proposal 1: R19 study on AI/ML-based mobility mainly focus on L3-based mobility. (HO+CHO) |
| RP-233468 | LG | If TU is constrained, L3 mobility should be prioritized over LTM. For L3 mobility, both network-triggered HO and conditional HO are considered. |
| RP-233480 | Fujitsu | Both L3-based mobility and LTM are suggested to be studied in this SI. |
| RP-233510 | CEWiT | Study both L3 and LTM based mobility |
| RP-233528 | Kyocera | Both L3-based mobility and LTM should be in-scope of study |
| RP-233543 | NTT Docomo | • Proposal: Consider both L3 Mobility & LTM at the NW-side with all 4 sub-use cases • RRM measurement prediction • HO target/candidate prediction • Prediction-based event operation • HO strategy/parameter optimization |
| RP-233551 | Nokia | •L3-based mobility and L1/L2-triggered mobility (LTM) are both considered |
| RP-233576 | CT | For the RRC state of mobility, in addition to HO in RRC_CONNECTED, Cell Reselection in RRC_INACTIVE/IDLE can also be considered. Conditional HO is also included in L3-based mobility. |
| RP-233600 | CU | - L3 based HO supported with AI/ML can be studied in Rel-19, especially in basic handover for FR2 bands. - Intra-CU LTM supported with AI/ML can be deprioritized in Rel-19. |
| RP-233620 | ZTE | Proposal 1: Both L3-based Mobility and LTM are considered in the study of Rel-19 AI/ML for Mobility. Proposal 2: Focus on the scenario in which handover target cell/beam is determined by the network. AI based target cell/beam selection for CHO execution is down-prioritized. |
| RP-233646 | Lenovo | L3-based mobility and L1/L2-triggered mobility (LTM) are both considered |
| RP-233654 | Intel | L3-based mobility and L1/L2-triggered mobility (LTM) |
| RP-233673 | Apple | Proposal 1: L3, L2 (LTM) and L1 mobility should be studied; for L3 the study should focus on HO and CHO. |
| RP-233870 | Sharp | L3 and LTM triggered Mobility enhancements |

Table 2.1-2

| ## Both L3 Mobility and LTM are in scope | ## L3 Mobility is prioritized |
| --- | --- |
| ## [2, 6, 8, 10, 14, 18, 21, 25, 26, 29, 30] ## [3, 13, 15, 17]: support CHO additionally, or L3 Mobility covers both HO and CHO ## [5, 9]: not support CHO | ## [1]: L3 Mobility is basic and essential, L3 Mobility covers both HO and CHO ## [7]: L3 Mobility covers both HO and CHO, LTM will be complicated ## [11]: due to workload ## [4]: intra-CU LTM can be down prioritized ## [16]: L3 Mobility covers both HO and CHO |

#### Moderator’s summary:

Based on the above proposals, 17 companies support to study both L3-based mobility and L1/L2-triggered mobility (LTM), while 5 companies prefer to focus on L3-based mobility mainly from workload point of view.

In addition, some companies further clarify whether CHO is included in L3-based mobility, wherein 7 companies support to include CHO in this study and 2 companies think CHO is not in the scope/down-prioritized. Moderator’s view is that we don’t need to exclude CHO at this stage considering the typical application scenarios or CHO. Whether any specific optimization is needed or not can be left for RAN2 to decide.

Besides HO in RRC_CONNECTED, 3 companies [3,13,21] propose that cell re-selection in RRC_IDLE/RRC_INACTIVE can also be considered. Due to limited support, the moderator suggests not to consider cell re-selection in this release. Moreover, [13,17] explicitly propose not to include or down prioritize DC, i.e., SCG addition/change, in this release, and only [14] supports such case. Moderator thinks we can focus on handover procedure of PCell only.

According to the majority view, the following draft proposal for type of mobility are given:

Moderator’s Proposal 1:

- L3-based mobility and L1/L2-triggered mobility (LTM) are both considered in R19, with further clarification as below,

- L3-based mobility includes HO and CHO;

- Focuses on RRC_CONNECTED UE only;

- PCell change only;

### 2.2 HO optimization in Network side (Including role of UE side)

The related discussion/proposals in companies’ contributions are copied as below:

Table 2.2-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232758 | Lekha | Proposal 2: Study and evaluate performance of predictive mobility in intelligent transportation networks. |
| RP-232780 | E/// | the HO optimization should be done in Network side only (i.e., NW receives the predictions from UE and based on predictions it selects the target cell). RAN WGs should consider existing mobility frameworks in this study and the study is not about specifying new mobility frameworks. |
| RP-232876 | NEC | Proposal 2: Regarding AI/ML for LTM, the following objectives can be considered: - Enhancement on target cell / beam prediction based on AI/ML model ; - Enhancement on L1 measurements prediction and reporting based on AI/ML model; - Enhancement on TA prediction based on AI model Proposal 3: Regarding AI/ML for L3-based mobility, the following objectives can be considered: - Enhancement on handover prediction/request based on AI/ML model; - Enhancement on temporal/spatial RRM measurements and temporal RLM measurements prediction based on AI/ML model; - Enhancement on dynamic adjustment on handover parameters/events based on AI/ML model; - Study the LCM scenarios including: • Model training at NW-side and transferred to UE-side; • Further model training at peer side; • Model training at UE-side. - Study the benefit of AI/ML for L3-based mobility enhancement over NG-RAN AI/ML mobility enhancement |
| RP-232911 | OPPO | Proposal 1: For AI/ML-based mobility studies, two types of AI/ML models can be considered: - Network-side model - UE-side model Proposal 2: The UE-side AI/ML model for mobility is under network control. Proposal 3: For AI/ML-based mobility, study the evaluation and specification impacts at least for the following aspects: - Candidate target cell and/or beam prediction - Unintended event prediction, e.g., HO failure, RLF, Ping-Pong HO, short of stay HO, etc. - RRM measurement prediction |
| RP-232918 | Samsung | Proposal 2: From the baseline objectives, [/UE side] part should be removed as indicated in RP-232745 . |
| RP-232984 | Spreadtrum | Proposal 2: Candidate/target Cell-level trajectory and Beam-level trajectory prediction should be studied in both NW-side and UE-side for HO and Cell reselection optimization. |
| RP-232998 | CATT | Proposal 1: Study AI/ML based candidate cell prediction in L3-based mobility with the below assumptions: - Both intra/inter-frequency cell prediction can be considered and intra-frequency cell prediction can be as a starting point; - Cell prediction can be performed only at network side; - Avoid the overlap work with RAN3 AI for NG-RAN. Proposal 2: Study AI/ML based candidate/target beam(s) and cell(s) prediction in LTM with the below assumptions: - Both intra/inter-frequency beam/cell prediction can be considered and intra-frequency beam/cell prediction can be as a starting point; - Beam/cell prediction can be performed only at network side; - Avoid the overlap work with RAN3 AI for NG-RAN, if per-SSB-beam-level UE trajectory prediction is supported in RAN3 Rel-19 AI for NG-RAN. |
| RP-233059 | vivo | Proposal 5: Potential sub-use cases to be considered: - Candidate/target cell prediction in L3-based mobility, or candidate/target beam(s) and cell(s) prediction in LTM, and considering the associated HO timing prediction; - Cell-level/beam-level RRM measurement (i.e., RSRP/RSRQ/SINR) prediction of serving/neighbor cells in the temporal/spatial/frequency domain; - Unintended events prediction, e.g., HOF, RLF, too-early HO, too-late HO, HO to wrong cell; - Measurement events prediction, e.g., event A3. |
| RP-233080 | NVIDIA | • Establish a study item on AI/ML for mobility in 3GPP Release 19, covering: • AI/ML-based target/candidate cell/beam prediction • AI/ML-based RRM measurement (e.g., RSRP, SINR) • AI/ML-based handover parameter tuning |
| RP-233216 | QC | • UE-side models • Beam-level measurement prediction, HO failure/RLF prediction, and measurement events prediction • Temporal and spatial prediction assisted mobility leveraging Rel-18 AI/ML study • Coordinate with RAN3 on network-side AI/ML models • Candidate/target cell(s) and beam(s) prediction |
| RP-233277 | MTK | • HO optimization in Network side [/UE side], including – Candidate/target cell prediction in L3-based mobility to improve candidate cell configuration and target cell determination; , or, candidate/target beam(s) and cell(s) prediction in LTM to improve candidate cell configuration and target cell determination with pre-sync and cell switch[RAN2/RAN3]. |
| 3427 | CMCC | Proposal 3: R19 study on AI/ML-based mobility should focus more on air interface, and the progress on R18 RAN3 AI-based mobility can be reused as much as possible with necessary enhancements. Proposal 4: Follow the legacy principle on HO decision for AI/ML-based mobility, i.e. the HO decision is always under NW control. |
| RP-233468 | LG | - HO optimization via candidate cell prediction should focus on network-sided prediction. - NW-sided candidate cell prediction is RAN-centric work, thus only RAN3 involvement is needed for this work. |
| RP-233480 | Fujitsu | Model for HO optimization is preferred to be at NW side. |
| RP-233528 | Kyocera | HO optimization in UE side should be kept in SID, and to discuss if one in NW side is really needed [maybe conflict with RAN3] |
| RP-233531 | IIT | Proposal 1: We agree to consider AI/ML-based Mobility as a Study Item led by RAN 2 for Rel 19 • AI/ML-based prediction of measurements • AI/ML-based periodicity of measurements • AI/ML-based HO Decision |
| RP-233551 | Nokia | - HO optimization in Network side, including - Prediction of candidate/target beam(s) and/or cell(s) - ML-based optimization of handover parameters |
| RP-233654 | Intel | - HO optimization in Network side: - Candidate/target cell prediction in L3-based mobility - Candidate/target beam(s) and cell(s) prediction in LTM |
| RP-233801 | Fraunhofer | Proposal: For AI/ML-based mobility, our focus is to investigate the impacts on evaluation and specification regarding the following aspects: - Optimizing Handovers: - Failure Handling Improvement: - Beam and Cell Prediction: |
| RP-233870 | Sharp | Based on the conclusions and findings of Rel-18 study on AI/ML for Air Interface, initiate an additional AI/ML air interface study item on mobility led by RAN2 in Rel-19 considering: - HO optimization e.g., target cell/target beam predictions - L3 and LTM triggered Mobility enhancements - RRM and event/measurement prediction e.g., predicting and avoiding HoF etc. - Different scenarios e.g., high mobility, dense deployment, FR1/2 etc. |

Table 2.2-2

| Scope | Sidedness: | Other comments |
| --- | --- | --- |
| Candidate/target cell prediction in L3-based mobility, or, candidate/target beam(s) and cell(s) prediction in LTM | Network Side only: Explicitly indicated by [2, 5, 7, 8, 9, 15, 22, 29] Network Side & UE Side: Explicitly indicated by [10, 26] | [10] further states that for UE-side model, UE can use its model to help the network make the decision, but whether to activate/use UE side model should be under network control. [26]: “UE side” should be kept, and TSG-RAN should discuss whether or not the “HO optimization in Network side” is conflict with “AI/ML for NG-RAN” led by RAN3 |

#### Moderator’s summary:

On the ground that we should reuse the existing framework for the mobility under connected mode, majority companies show support to the RAN Chair’s guidance, i.e., remove the UE Side from the role of making decision. However, current formulation seems to cause a different understanding that UE side model should be completely excluded from HO optimization, and hence [26] proposes “TSG-RAN should discuss whether or not the “HO optimization in Network side” is conflict with “AI/ML for NG-RAN” led by RAN3”. Maybe one clarification we can add to mitigate the confusion is that such kind HO optimization could take application of UE side model as prerequisite, another one is that we can clearly exclude the duplicate study that has already been investigated in “AI/ML for NG-RAN” led by RAN3 (Already a general constraint for this item).

Moderator’s Proposal 2:

- Study HO optimization in Network side, where HO decision is always under NW control

- Candidate/target cell(s) prediction in L3-based mobility, or, candidate/target beam(s) and cell(s) prediction in LTM

- Avoid duplicate study with “AI/ML for NG-RAN” led by RAN3

### 2.3 RRM measurement and event prediction

The related discussion/proposals in companies’ contributions are copied as below:

Table 2.3-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232780 | E/// | both beam level and cell level measurement prediction should be considered in the SI: beam level predictions are more relevant for LTM, while cell level predictions are more relevant for L3-based mobility. Regarding the types of predictions, we think that all three types of prediction (i.e., temporal/spatial/frequency) should be considered during the SI. - RRM measurement and event prediction, including - Beam-level measurement prediction - Cell-level measurement prediction - HO failure/RLF prediction - Measurement events prediction |
| RP-232911 | OPPO | Proposal 3: For AI/ML-based mobility, study the evaluation and specification impacts at least for the following aspects: - Candidate target cell and/or beam prediction - Unintended event prediction, e.g., HO failure, RLF, Ping-Pong HO, short of stay HO, etc. - RRM measurement prediction |
| RP-232918 | Samsung | Proposal 3: RAN2 focuses on Cell-level measurement prediction, and beam-level measurement prediction can be postponed until beam management use case is progressed in the RAN1-led WI. Proposal 4: Agree with the baseline objectives for LCM and RRM aspects as indicated in RP-232745. |
| RP-232930 | Xiaomi | Study following use cases [RAN2] －HOF prediction to reduce HOF rate －Cell measurement result prediction to improve throughput and reduce latency －ToS prediction to reduce ping pong handover |
| RP-232984 | Spreadtrum | Proposal 3: RRM measurement and event prediction should be studied. |
| RP-232998 | CATT | Proposal 3: The scope of Beam-level measurement prediction study could focus on beam level time-domain RSRP prediction across cells using network-sided or UE-sided model as a starting point. Proposal 4: Cell-level measurement prediction using network-sided or UE-sided model could be studied from the aspects of: - Time-domain measurement prediction, avoiding the overlap work with Candidate/target cell prediction in L3-based mobility; - Frequency-domain measurement prediction. Proposal 5: Clarify the necessity and the study area scope of the HO failure/RLF prediction use case; otherwise this use case should be deprioritized. Proposal 6: Measurement events prediction could be studied from the aspect of: - To predict whether the measurement RSRP can stably exceed the handover threshold during TTT by UE for e.g. an advanced measurement report/handover preparation. |
| RP-233059 | vivo | Proposal 5: Potential sub-use cases to be considered: - Candidate/target cell prediction in L3-based mobility, or candidate/target beam(s) and cell(s) prediction in LTM, and considering the associated HO timing prediction; - Cell-level/beam-level RRM measurement (i.e., RSRP/RSRQ/SINR) prediction of serving/neighbor cells in the temporal/spatial/frequency domain; - Unintended events prediction, e.g., HOF, RLF, too-early HO, too-late HO, HO to wrong cell; - Measurement events prediction, e.g., event A3. |
| RP-233216 | QC | • UE-side models • Beam-level measurement prediction, HO failure/RLF prediction, and measurement events prediction • Temporal and spatial prediction assisted mobility leveraging Rel-18 AI/ML study • Coordinate with RAN3 on network-side AI/ML models • Candidate/target cell(s) and beam(s) prediction |
| RP-233277 | MTK |  |
| RP-233316 | HW | - Study optimization of handover performance metrics for both UE side and network side model: - - Cell-level measurement prediction, e.g., using intra-frequency measurement results to forecast the RRM measurement of inter-frequency/inter-RAT cells - - Beam-level measurement prediction for L3 mobility - - HO failure/RLF prediction - - Measurement events prediction |
| RP-233406 | InterDigital | Proposal 9: RAN2 R19 AIML study can include the following objective: - AI/ML based RRM measurement prediction in temporal and frequency domain - Beam-level measurement prediction - Cell-level measurement prediction (e.g., using intra-frequency measurement results to forecast the RRM measurement of inter-frequency/inter-RAT cells) |
| RP-233468 | LGE |  |
| RP-233480 | Fujitsu | With cell/beam-level measurement prediction, both UE-sided model and NW-sided model can predict the potential abnormal cases including beam failure/HOF/RLF, short-time stay and mobility ping-pong. Both NW-sided model and UE-sided are suggested to be studied. |
| RP-233528 | Kyocera | The beam-level/cell-level measurement prediction may be studied, but TSG-RAN should discuss to increase RAN1’s TU allocation |
| RP-233576 | CT | - Unintended events prediction, e.g., HOF, RLF, too-early HO, too-late HO, HO to wrong cell; |
| RP-233351 | Nokia | - RRM measurement and event prediction, including - Beam-level measurement prediction - Cell-level measurement prediction, e.g., using intra-frequency measurement results to forecast the RRM measurement of inter-frequency/inter-RAT cells - HO failure/RLF predictionMobility-related event prediction |
| RP-233620 | ZTE | - RRM measurement prediction: - Study L3 cell level measurement results prediction (e.g. intra/inter-frequency and inter-RAT) for L3 based Mobility, prioritize network-sided model (RAN2) - Study L1 beam level measurement results prediction (e.g. intra/inter-frequency) for LTM (RAN1, RAN2) |
| RP-233646 | Lenovo | – RRM measurement and event prediction, including - Beam-level measurement prediction - Cell-level measurement prediction, e.g., using intra-frequency measurement results to forecast the RRM measurement of interfrequency/inter-RAT cells - HO failure/RLF prediction - Measurement events prediction – Beam failure prediction in UE side |
| RP-233654 | Intel | RRM optimization at UE-side, including - Event prediction (e.g. Measurement events prediction, HO failure/RLF prediction, etc) |
| RP-233673 | Apple | Proposal 3: all RRM measurement and event prediction listed in [1] can be kept for the study. |
| RP-233870 | Sharp | RRM and event/measurement prediction e.g., predicting and avoiding HoF etc. |

Table 2.3-2

| Scope | Supporting company | Sidedness: |
| --- | --- | --- |
| Beam level measurement prediction | [1, 2, 3, 5, 6, 7, 8, 9, 10, 13, 14, 15, 17, 19, 20, 21, 22, 25, 23, 30] | Network Side and UE Side: Explicitly indicated by [2, 7, 22, 25], though [2] further indicated that UE Side can be deprioritized. |
| Cell level measurement prediction | [1, 2, 5, 7, 8, 9, 10, 11, 14, 15, 17, 22, 3, 12, 13, 16, 19, 25, 20, 21, 23, 30] | Network Side and UE Side: Explicitly indicated by [2, 7, 22, 25], though [2] further indicated that UE Side can be deprioritized. |
| HOF, RLF | Support explicitly: [1, 3, 5, 7, 8,10, 11, 12, 13, 14, 15, 17, 21, 25, 26, 28, 29, 30] Negative: [16, 22] | Network Side and UE Side: Explicitly indicated by [7, 25] |
| Measurement event prediction | [2, 3, 5, 7, 8, 10, 13, 15, 17, 22, 25, 29] | Network Side and UE Side: Explicitly indicated by [2, 7,25], though [2] further indicated that UE Side can be deprioritized. UE Side only: Explicitly indicated by [29] |
| HO strategy/parameter optimization | [2, 6, 8, 20, 23] | Network Side only Explicitly indicated by [2, 8] |
| Ping-Pong HO | [1, 10, 20, 25] |  |
| BF prediction | [1, 16, 25, 28] |  |
| ToS prediction | [10, 12, 20, 25] |  |
| RLM measurement | [6] |  |
| TA prediction | [6] |  |
| Event prediction error | [16] |  |

Additional points on the preference of sidedness of model:

3 companies [3, 13, 17]: support both network and UE side model but prefer UE side model

1 company [9]: support both, but prefer network side model

1 company [13]: two-sided model can be also considered

#### Moderator’s summary:

For other potential scope, majority companies focus on following aspects:

- Beam-level measurement prediction

- Cell-level measurement prediction, e.g., using intra-frequency measurement results to forecast the RRM measurement of inter-frequency/inter-RAT cells

- HO failure/RLF prediction

- Measurement events prediction

From the contributions, few companies explicitly indicate whether these aspects should be done in either network side or UE side or both. For L1 beam measurement few companies also mentioned which domain should be studied e.g., time or frequency or spatial domain etc. Moderator think these prediction cases are mainly based on RRM measurement which can be done by UE. And some case e.g. “using intra-frequency measurement results to forecast the RRM measurement of inter-frequency/inter-RAT cells” can also be done in network. So, some discussion in RAN2 is needed.

There are also other prediction cases as indicated in the table 2-2. Considering the minority support and also RAN2 workload, moderator suggests not to consider them in this release. In addition, only 1 company mentions to consider two-sided model. Considering what happened in RAN1 on the two-sided model, it seems single sided model is more reasonable approach. Hence moderator propose:

Moderator’s Proposal 3:

- To study following prediction cases based on single side model:

- Beam level measurement prediction

- Cell level measurement prediction

- Measurement event prediction

- HOF, RLF prediction

### 2.4 LCM framework and others

Table 2.4-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232780 | E/// | The conclusions from Rel-18 AI/ML study should be used as baseline (e.g. data collection framework, performance monitoring, functionality and model activation/deactivation/switching/fallback from Rel-18 can be reused, if applicable) |
| RP-232911 | OPPO | Proposal 7: For AI/ML-based mobility, the general LCM framework discussed in Rel-18 AI/ML can be reused as much as possible, especially for model transfer/delivery and data collection for training. Any mobility-specific AI enhancement is still not precluded if needed, e.g., new mechanisms for model inference and monitoring. |
| RP-232918 | Samsung | Proposal 4: Agree with the baseline objectives for LCM and RRM aspects as indicated in RP-232745. |
| RP-232930 | xiaomi | Assess the potential specification impact [RAN2] －Reuse LCM procedure in Rel-18 AI/ML study as much as possible －Data collection for training/inference/performance monitoring －Assistance info exchange between UE and NW |
| RP-233277 | MTK | • Study the impacts on LCM framework if identified: LCM framework and others • The conclusions in Rel-18 AI/ML study should be used as baseline. • Other impacts are further studied |
| RP-233316 | HW | - The LCM framework for AI/ML for Air Interface (PHY) should be reused for AI/ML for Air Interface (mobility). |
| RP-233406 | InterDigital | Proposal 6: The LCM framework from R18 RAN1 SI can be the baseline for RAN2-led AIML study. |
| RP-233427 | CMCC | Proposal 5：The functional framework and LCM procedure introduced in R18 AI/ML for air interface SI can be reused for R19 study on AI/ML-based mobility with necessary enhancements. |
| RP-233468 | LGE | - LCM including model delivery/monitoring/switching/deactivation related to predictive mobility is important for controllable and robust predictive mobility performance. - Identification of UE capabilities related to predictive mobility is important for extending existing UE capability signaling. |
| RP-233480 | Fujitsu | Both functionality-based LCM and model-ID-based LCM need to be studied in AI/ML for mobility. |
| RP-233510 | CEWiT | Reuse the outcomes of Beam management Rel18 SI wherever possible |
| RP-233528 | Kyocera | The conclusions in Rel-18 AI/ML study should be used as baseline Impacts on existing LCM framework can be discussed, if any issue is identified |
| RP-233351 | Nokia | - Study the required extensions to the AI/ML for air interface functional framework developed in the Rel-18 TR 38.843 to support the mobility use cases [RAN2, RAN1] - RAN2 to start the study. RAN1 study aspects may be identified once RAN2 has progressed its studies. |
| RP-233600 | CU | - Proposal 3. The LCM framework for AI/ML for air Interface in Rel-18 should be taken as baseline for Rel-19 AI/ML for Air Interface (mobility). |
| RP-233620 | ZTE | Proposal 5: The study of AI/ML based Mobility should focus on mobility specific LCM procedure and parameters, the conclusions in Rel-18 AI/ML study should be used as a baseline. |
| RP-233654 | Intel | The conclusions in Rel-18 AI/ML study for LCM, data collection, model transfer/delivery (if needed) are used as baseline for AI/ML mobility |
| RP-233673 | Apple | Proposal 4: no need to include LCM in this study, this is a good candidate for down-scoping. |

#### Moderator summary:

Majority companies support to reuse the concluded LCM frame in Rel18 for AI mobility. [1, 9, 10] mentioned that some enhancement could be needed. [10] gives such example like model inference and monitoring will touch performance metrics, which is use case specific also in RAN1. [17] doesn’t want to include anything related to LCM because they believe this SID should focus on pure mobility issues. And [29] mentioned any study on LCM in this SID should avoid overlapping with Rel18 RAN1 WID.

Moderator’s Proposal 4:

- Study AI/ML Mobility with potential AI mobility specific enhancement based on the Rel. 18 AI/ML LCM framework and the progress of Rel. 19 AI/ML-air interface normative work if any.

### 2.5 RAN4 work and RRM

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232930 | xiaomi | ●Interoperability and testability aspects [RAN4] |
| RP-232998 | CATT | Proposal 8: For Interoperability and testability aspects, considering the common part (section 7.4.2) in TR38.843 as baseline, and define new requirements/test metrics for the mobility specific (sub-) use cases by RAN4. Proposal 9: Do not include RAN4 RRM enhancement in this AI/ML based mobility study item. |
| RP-233277 | MTK |  |
| RP-233528 | Kyocera | RAN4-led topic such as UE RRM enhancement should be out of scope |
| RP-233351 | Nokia | - Study the RAN4 requirements and other RAN4 implications for the above-mentioned mobility use cases and how to ensure predictable UE behavior and performance [RAN4] |

#### Moderator summary:

RAN4 related proposals from companies mainly focus on the RAN4 requirement, as well as inter-operability, and some of them explicitly propose not to discuss RAN4 RRM in this item. It looks the guidance from Chair is quite acceptable to companies.

Moderator’s Proposal 5:

- RAN4 related UE RRM enhancement is not included in this item.

### 2.5 Other

#### 2.5.1 Evaluation

Table 2.5.1-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232758 | Lekha | Proposal 3: Study and investigate the AI ML models in establishing Group HO. Proposal 4: Evaluate the KPIs performance of the AI ML based mobility enhancements and compare with the existing methods. |
| RP-232780 | E/// | The evaluation of the AI/ML aided mobility benefits should consider at least the following KPIs: Too early/too late/ping-pong HO UE Throughput RLF/HOF |
| RP-232911 | OPPO | Proposal 6: The methodology and assumptions in TR 38.843 and TR 36.839 can be start point for study on AI/ML-based mobility. Proposal 6a: The EVM assumption in RAN2 could be simplified compared to RAN1 study |
| RP-232998 | CATT | Proposal 7: Companies are encouraged to give the simulation evaluations which are based on the defined target performance metrics and simulation assumptions. It is no need to calibrate the simulation results among companies. Evaluation based on theoretical/numerical analyses from various dimensions (e.g. Performance gain, power saving) w/o simulation could also be considered. |
| RP-233059 | vivo | Proposal 6: RAN2 performance evaluation of sub-use case should be included in the SID. RAN1 and RAN4 involvement in the performance evaluation can be triggered by RAN2, if needed. Proposal 7: Down-select promising sub-use case(s) based on initial performance evaluations can be assessed by RAN#104 (Jun. 2024). Subsequent study of other WGs (e.g., RAN1, RAN4)can be started based on the down-selected sub-use cases. |
| RP-233277 | MTK | Specify the methodology for evaluating the performance benefits of AI/ML based algorithms: – Methodology based on statistical models (from TR 38.901) for system level simulation as a starting point. - Need for common assumptions for the simulation setting, e.g., network deployment and configuration and UE trajectory. – System-level and intermediate KPIs: - Determine the KPIs and benchmarks for each use cases. The conventional system-level KPIs for mobility performance evaluation incl. HOF/RLF rate, ToS/PingPong rate, HO Interruption time and RSRP distribution are considered as baseline. Other system KPIs, e.g., overhead, power consumption should be discussed as part of the study; - Intermediate KPIs to evaluate the AI/ML algorithms performance e.g., prediction accuracy, RSRP difference as well as inference latency, computation complexity should be discussed as part of the study. – Generalization performance evaluation and methodology should be discussed as part of the study. |
| RP-233316 | HW | - Handover performance metrics as defined in TR 36.839 can be used as reference for handover performance evaluation in NR for AI/ML based mobility evaluation. A set of handover performance metrics can be selected as handover performance enhancement targets. New performance metrics are not precluded considering new handover features defined in NR. - Study optimization of handover performance metrics for network side model: - - Candidate/target cell prediction in L3-based mobility - Study optimization of handover performance metrics for both UE side and network side model: - - Cell-level measurement prediction, e.g., using intra-frequency measurement results to forecast the RRM measurement of inter-frequency/inter-RAT cells - - Beam-level measurement prediction for L3 mobility - - HO failure/RLF prediction - - Measurement events prediction |
| RP-233406 | InterDigital | Proposal 7: Objective for RAN2 R19 AIML study should include the evaluation of performance gains vs complexity of selected use cases. Proposal 8: Objective for RAN2 R19 AIML study should include the evaluation of generalization, robustness, interoperability and testability of selected use cases. |
| RP-233543 | NTT Docomo | Proposal: Study the complexity and potential performance of AI/ML model for HO optimization by using valid, simple, and aligned evaluation cases, assumptions, and methodology. Proposal: To study the complexity and potential performance gains, consider the following aspects of the evaluation methodology, » Scenario – Both FR1 & FR2 – Dense deployment scenario (e.g., based on NR-UMi) with UE mobility model » Baseline HO schemes – For simplified intermediate KPI: Legacy non-AI/ML methods. – For system-level eventual KPI: L3 HO (w/ or w/o CHO) and Rel. 18 LTM. » Performance metrics – Model complexity (model size, parameter number, FLOPs) w/ performance evaluations. – Intermediate KPIs on the performance of direct output of AI/ML models. ✓ Prediction accuracy, Prediction successful ratio, etc. – Eventual KPIs for mobility. ✓ L3 Mobility: HoF rate, Ping-Pong rate, interruption occasions, etc. ✓ LTM: Ping-Pong rate, interruption occasions, short ToS rate, etc. |
| RP-233620 | ZTE | Performance evaluation: - Identify performance metrics for AI based L3 mobility, and evaluate the benefits of AI/ML based L3 mobility (RAN2) - Identify performance metrics for AI based LTM, and evaluate the benefits of AI/ML based LTM (RAN1) - Performance metrics can include, e.g. UE measurement power consumption, prediction accuracy, handover/LTM latency, handover/LTM failure rate, Ping-Pong rate, UE throughput. |
| RP-233654 | Intel | - Performance evaluation between AI/ML-based HO and existing HO mechanism - Optimization target and performance evaluation metrics may include: - HO failure - Ping-pong - Measurement and report reduction - Duration of stay in the candidate/target cell |

Table 2.5.2-1

| KPI | Supporting company | Other comments |
| --- | --- | --- |
| Too early HO | [5] |  |
| Too late HO | [5] |  |
| Ping-pong HO | [5, 2, 9, 10, 15, 29] |  |
| UE throughput | [5, 9] |  |
| RLF/HOF | [5, 2, 9, 10, 15, 29] | [2] L3 HO only |
| Interruption occasion | [2] |  |
| Short of stay | [2, 10, 29] | [2] LTM only |
| Power consumption | [9] |  |
| Prediction accuracy | [9, 10, 15] | [15] Intermediate KPI |
| Handover latency | [9, 10] |  |
| Handover interruption time | [10, 15, 20] |  |
| Measurement overhead | [10] |  |
| RSRP distribution | [15] | [15] Intermediate KPI |
| RSRP difference | [15] | [15] Intermediate KPI |
| Inference latency | [15] | [15] Intermediate KPI |
| Computing complexity | [15] | [15] Intermediate KPI |
| Measurement and report reduction | [20, 29] |  |

[7, 10]: take KPI in 36.839 as baseline

[15]: EVM from 38.901 as starting point

[22]: based on simulation w.o. calibration among companies or theoretical /numerical calculation can be accounted too

[24]: Objective for RAN2 R19 AIML study should include the evaluation of generalization, robustness, interoperability and testability of selected use cases

#### Moderator summary:

Based on last meeting’s agreement, target performance metrics and impacts should be clarified, with this spirit, companies propose their understanding on related metrics and how to evaluate.

The performance metrics for simulation from contributions are bit diverse among companies. In table 2.5.2-1, few of them are supported by at least 3 companies:

- Ping-pong HO (6)

- HOF/RLF (rate) (6)

- Short of stay (rate) (3)

- Handover interruption (3)

- Predication accuracy (3)

Moderator think performance metrics need be anyway further discussion in RAN2. The listed metrics could be a start point.

Two companies [10, 15] mention how to identify evaluation methodology by considering either 38.901 or 36.839. Considering the relationship between mobility study and RAN1 study on beam management and also most likely LTM is in the scope, what has been captured in 38.843 should be leveraged by this study. In addition, TR 36.839 captured the evaluation on Hetnet mobility, which is valuable for reference for RAN2 too.

Moderator’s Proposal 6:

- Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Capture following performance metrics as example and leave the further discussion to RAN2:

- Ping-pong HO (6)

- HOF/RLF (rate) (6)

- Short of stay (rate) (3)

- Handover interruption (3)

- Predication accuracy (3)

#### 2.5.2 potential specification impact

Table 2.5.3-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-232911 | OPPO | Proposal 8: Assess potential specification impacts of AI/ML-based mobility from the following aspects: - Evaluate different optimization actions to be taken based on the predictions and their impact on protocol stacks - Study mobility-specific LCM procedures and parameters - Study mobility-specific testability aspects, core requirements, and performance requirements |
| RP-233216 | QC | • Identify specification impacts, such as • UE and network signaling support and assistance information • AI/ML functionality/model life cycle management • Leverage Rel-18 AI/ML study |
| RP-233277 | MTK | • Assess potential specification impact: – Protocol aspects: Consider aspects related to e.g., RRM measurement procedures, L3-based and LTM mobility procedures, AI trigger events, as well as configuration corresponding to the AI/ML approaches for each use case; – PHY aspects: Consider aspects related to e.g., the additional physical layer signaling/measurement and the corresponding processing. RAN1 only starts the work after sufficient progress is made on the use case study in RAN2. |
| RP-233543 | NTT Docomo | • Proposal: Study and identify the use case-specific specification impacts of AI/ML Mobility use cases based on the Rel. 18 AI/ML LCM framework and the progress of Rel. 19 AI/ML-air interface normative work if any. • Proposal: Study and identify other potential specification impacts, e.g., enhancements utilizing inference results and mobility procedures. |

#### Moderator summary:

To assess potential specification impacts of AI/ML-based mobility is kind of business as usual, we can simply take it as one aspect of this item.

#### 2.5.3 Miscellaneous

Table 2.5.3-1

| TDoc | Source | Proposals |
| --- | --- | --- |
| RP-233406 | IDC[24] | Proposal 1: AI/ML algorithms and models details are out of scope for RAN2-led R19 AIML study. Proposal 2: For RAN2-led R19 AIML study offline AIML model training is assumed and Details of AI/ML model training are out of scope. Proposal 3: Scope of RAN2-led R19 AIML study is limited to existing NR architecture and interfaces. Proposal 4: The scope of RAN2-led AIML study should focus on the impacts to the Uu interface. |

#### Moderator summary:

Proposals from [24] indicate the general consideration on how to limit the discussion scope, which could be more or less left to WGs.

- Summary

With above moderator summaries and proposals, moderator would like to propose the scope of SI on AI/ML for mobility as below:

Study and evaluate potential benefits and gains of AI/ML aided mobility under connected mode, considering the following aspects:

- Type of mobility:

  - Both Network triggered L3-based mobility and L1/L2-triggered mobility (LTM) are considered

    - L3-based mobility includes HO and CHO;

    - Focuses on RRC_CONNECTED UE only;

    - PCell change only;

- HO optimization in Network side (HO decision is always under NW control) including:

  - Candidate/target cell(s) prediction in L3-based mobility, or, candidate/target beam(s) and cell(s) prediction in LTM

- RRM measurement and event prediction based on single side model, including

  - Beam-level measurement prediction

  - Cell-level measurement prediction

  - HO failure/RLF prediction

  - Measurement events prediction

- Assess potential specification impacts of AI/ML aided mobility.

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Short of stay, Handover interruption, Predication accuracy, etc.) and corresponding complexity.

  - Note: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- RAN4 requirements and other RAN4 implications for the above-mentioned mobility improvement.

- Study AI/ML Mobility with potential AI mobility specific enhancement based on the Rel. 18 AI/ML LCM framework and the progress of Rel. 19 AI/ML-air interface normative work if any.

  - Note: The conclusions from Rel-18 AI/ML study should be used as baseline (e.g. data collection framework, performance monitoring, functionality and model activation/deactivation/switching/fallback from Rel-18 can be reused, if applicable)

- Note 1: No intention to change the existing framework for the mobility under connected mode

- Note 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3

- Reference

RP‑233427 	Views on R19 AI/ML for Mobility 	CMCC

RP‑233543 	Views on Rel-19 study on AI/ML for mobility 	NTT DOCOMO, INC.

RP‑233576 	Views on AIML-based mobility enhancement 	China Telecom,vivo

RP‑233600 	Consideration on AI /ML for mobility 	China Unicom

RP‑232780 	Study on AI/ML for mobility 	Ericsson

RP‑232876 	Study on AIML for mobility 	NEC

RP‑233316 	AI/ML for mobility in Rel-19 	Huawei, HiSilicon

RP‑233551 	Rel-19 AI/ML for Air Interface (Mobility) Study scope 	Nokia, Nokia Shanghai Bell

RP‑233620 	Views on Rel-19 AI/ML for Mobility 	ZTE, Sanechips

RP‑232911 	Discussion on scope of AI/ML mobility 	OPPO

RP‑232918 	Scope of Study on AI/ML for Mobility 	Samsung

RP‑232930 	Discussion on AIML mobility 	Xiaomi EV Technology

RP‑233059 	Views on Rel-19 AI/ML for Mobility Study 	vivo

RP‑233216 	Views on scope of AI/ML for mobility study in Rel-19 	Qualcomm Incorporated

RP‑233277 	Rel-19 AI/ML Mobility: Scope and Initial evaluation 	MediaTek Inc.

RP‑233468 	Work scope of AIML mobility 	LG Electronics Inc.

RP‑233673 	On AI/ML for air interface (mobility) 	Apple

RP‑233510 	Discussion on AI/ML for Mobility 	CEWiT

RP‑233801 	RAN2 AI/ML for Mobility in Rel-19 	Fraunhofer HHI, Fraunhofer IIS

RP‑232758 	Study on AI/ML based Mobility Enhancement in Release 19 	Lekha Wireless Solutions

RP‑232984 	AIML-based mobility in Rel-19 	Spreadtrum Communications

RP‑232998 	Views on AI/ML for mobility 	CATT

RP‑233080 	Study on AI and ML for 5G mobility 	NVIDIA

RP‑233406 	Scope of Study on AI/ML for Mobility in Rel-19 	InterDigital, Inc.

RP‑233480 	Views on scope for Study on AI /ML for mobility in Rel-19 	Fujitsu

RP‑233528 	Study on AI/ML for mobility 	Kyocera

RP‑233531 	Views on AI/ML for Mobility 	Indian Institute of Tech (M), IIT Kanpur

RP‑233646 	Consideration of beam failure prediction in AI based mobility 	Lenovo

RP‑233654 	Discussion on Rel-19 AIML for Air Interface (Mobility) SI 	Intel Corporation

RP‑233870 	AI/ML for Air Interface (Mobility) SI considerations for Rel-19 Sharp