3GPP TSG RAN meeting #103									               RP-240171

Maastricht, Netherlands, March 18-21, 2024

## Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Evolution of NR duplex operation: Sub-band full duplex (SBFD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_duplex_evo |  |  |  |  |
| Unique ID | 1020090 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234035 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 7% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xinghua Song, He (Jackson) Wang |
|  | Company | Huawei, Samsung |
|  | Email | songxinghua@huawei.com, h0809.wang@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#116, the following agreements were made:

SBFD TX/RX/measurement procedures

Agreement

For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier.

- FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Agreement:

For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- FFS: Further details

FFS: Details when two TDD-UL-DL patterns are configured

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

For semi-static indication of SBFD subband time location,

- When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period.

  - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon

  - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

  - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

  - FFS details

Agreement

The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - Subject to RAN4 guidance on the size of subband/guardband, if any

  - FFS reference starting RB and reference SCS

Agreement

For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs.

For determining UL/DL usable PRBs, consider the following options.

- Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

- Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

Agreement

For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL usable PRBs are allowed

  - FFS SSB symbols

- DL receptions within DL usable PRBs are allowed

- UL transmissions outside UL usable PRBs are not allowed

- DL receptions outside DL usable PRBs are not allowed

  - This restriction is not applicable for CLI measurement

CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3.

RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0

- DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

Agreement

For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Other options are not precluded.

Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary:

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

SBFD random access operation

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Conclusion

No new PRACH format is introduced in Rel-19 duplex WI.

Agreement

For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options:

- Option 1: Use one single RACH configuration with possible enhancement

  - The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE

  - FFS: Further details

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

  - The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE

  - FFS: Further details

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

Conclusion

If PRACH is allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following:

- The benefits include at least one or more of the following:

  - reduced random access latency

  - reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots

  - improved coverage of PRACH with sparse UL resources

  - increased cell range of PRACH with sparse UL resources

- PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario.

For future meetings

Companies to consider whether the existing random access configuration tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2) need to be enhanced.

Agreement

For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols.

- FFS PRACH repetition in SBFD symbols.

- FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, further study the following two options:

- Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols

  - a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid

- Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots

RAN1 to leverage the study in Rel-18 as baseline.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects:

- Msg2[/Msg4 PDSCH] reception in DL subband(s)

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping

- Msg3 repetition

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control

- FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE

Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

For future meetings:

In RAN1#116bis meeting, at least the following issues will be discussed:

- Whether to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode

- Details of the two options for configuring ROs for SBFD aware UEs in RRC CONNECTED mode, including RO validation rules, SSB-RO mapping rules, whether/how to allow SBFD aware UE and non-SBFD aware UE to use different PRACH preamble formats.

- Whether/how to support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols.

- Whether/how to enhance the existing random access configuration tables for unpaired spectrum

- Whether/how to support PRACH repetition

CLI handling

Conclusion

For the down-selection of gNB-to-gNB co-channel CLI handling schemes and UE-to-UE co-channel CLI handling schemes, at least the following aspects should be considered:

- Applicable scenario, performance benefits based on analysis and/or demonstrated by evaluations for SBFD

  - Note: Companies are encouraged to provide more simulation results for SBFD to RAN1#116bis based on the simulation assumptions agreed during the SI.

- Specification impact in RAN1 and RAN3.

- gNB/UE implementation complexity.

- Operational details of the scheme including feasibility and practicability.

Agreement

Consider the following candidate gNB-to-gNB co-channel CLI handling schemes for further down-selection

- gNB-to-gNB co-channel CLI and/or channel measurements

- Spatial domain based schemes

  - Beam nulling

  - Beam pairing

- Coordinated scheduling in time and/or frequency

- Power control based schemes

  - gNB Tx power control

  - UE Tx power control

Note: gNB-to-gNB co-channel CLI and/or channel measurements can be the enablers for some of the above CLI handling schemes.

Agreement

Consider the following candidate UE-to-UE co-channel CLI handling schemes for further down-selection

- UE-to-UE co-channel CLI measurement and reporting

- Coordinated scheduling in time and/or frequency

- Spatial domain based schemes

- Power control based schemes

- Note: UE-to-UE co-channel CLI measurement and reporting can be the enablers for some of the above CLI handling schemes.

Agreement

gNB Tx power control based schemes are not considered in the down-selection of gNB-to-gNB co-channel CLI handling schemes.

Agreement

For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Method#4: UE measures RSSI within guard band, if guard band exists

Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply.

Note: Method#4 does not imply that guard band is explicitly configured.

For future meetings:

Companies are to refer to Proposal 2-2a (gNB-gNB CLI handling) and Proposal 3-2a (UE to UE CLI handling) in R1-2401635 for future meetings. Companies are encouraged to provide additional details on potential spec impact and operational details of their preferred CLI handling scheme for further down-selection in RAN1#116bis.

#### 2.1.2	Remaining Open issues

SBFD TX/RX/measurement procedures

  - Remaining details on semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode

  - Remaining details on semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode

  - Remaining details on UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE

  - Remaining details on transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

  - Enhancement on resource allocation in frequency domain in SBFD symbols, including

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

  - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

  - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

  - Remaining details on collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

SBFD random access operation

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode

  - Remaining details on justification and specification, if justified, of SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access

CLI handling

  - Remaining details on gNB-to-gNB co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

  - Remaining details on UE-to-UE co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117)

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

Not started yet.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

Not started yet

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

Not started yet

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#116

R1-2400111	Workplan for Evolution of NR Duplex Operation	Huawei, HiSilicon

R1-2400432	Discussion on SBFD TX/RX/measurement procedures	CATT

R1-2400557	Discussion on reception and transmission procedure for SBFD operation	xiaomi

R1-2400838	On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

R1-2400053	Discussion on SBFD TX/RX/measurement procedures	Spreadtrum Communications

R1-2400108	On subband full duplex design	Huawei, HiSilicon

R1-2400154	Discussion for SBFD TX/RX/measurement procedures	New H3C Technologies Co., Ltd.

R1-2400177	Discussion on SBFD TX/RX/measurement procedures	Tejas Network Limited

R1-2400207	Discussion on SBFD operation	Transsion Holdings

R1-2400241	Discussion on Rel-19 SBFD operation	vivo

R1-2400269	Discussion on SBFD TX/RX/measurement procedures	TCL

R1-2400300	Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE

R1-2400325	Discussion on SBFD TX/RX/measurement procedures	CMCC

R1-2400606	Discussion on SBFD Tx/Rx/measurement procedures	OPPO

R1-2400643	SBFD procedures in RRC_CONNECTED mode	Sharp

R1-2400660	Discussion on SBFD TX/RX/measurement procedures	China Telecom

R1-2400687	Discussion on SBFD TX RX and measurement procedures	InterDigital, Inc.

R1-2400731	SBFD operation and procedures	Samsung

R1-2400775	Discussion on Tx/Rx/Measurement procedures for SBFD operation	Fujitsu

R1-2400802	Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

R1-2400826	Discussion on SBFD TX/RX/measurement procedures	Panasonic

R1-2400852	SBFD operations	Sony

R1-2401011	Views on SBFD TX/RX/measurement procedures	Apple

R1-2401081	On semi-static indication of SBFD resources	Google Inc.

R1-2401116	Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

R1-2401159	Considerations on SBFD Tx/Rx/measurement procedure	KT Corp.

R1-2401188	Discussion on SBFD TX/RX/measurement procedures	ITRI

R1-2401191	Tx Rx procedure for SBFD	ASUSTeK

R1-2401214	Discussion on SBFD TX/RX/measurement procedures	WILUS

R1-2401217	SBFD TX/RX/measurement procedures	Ericsson

R1-2401227	Discussion on SBFD TX/RX/measurement procedures	ETRI

R1-2401249	Discussion on SBFD TX/RX/measurement procedures	LG Electronics

R1-2401263	Views on sub-band full duplex operations	Lenovo

R1-2401273	Discussion on SBFD TX/RX/ measurement procedures	CEWiT

R1-2401294	Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

R1-2401440	SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

R1-2401652	Summary #1 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2401653	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2401654	Summary #3 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2400326	Discussion on SBFD random access operation	CMCC

R1-2401117	Discussion on SBFD random access operation	NTT DOCOMO, INC.

R1-2401218	SBFD Random access operation	Ericsson

R1-2400054	Discussion on SBFD random access operation	Spreadtrum Communications, BUPT

R1-2400109	On subband full duplex random access operation	Huawei, HiSilicon

R1-2400153	Discussion for SBFD random access operation	New H3C Technologies Co., Ltd.

R1-2400178	Discussion on SBFD Random Access Operation	Tejas Network Limited

R1-2400242	Discussion on random access for Rel-19 SBFD	vivo

R1-2400270	Discussion on SBFD random access operation	TCL

R1-2400301	Discussion on SBFD random access operation	ZTE

R1-2400433	Discussion on SBFD random access operation	CATT

R1-2400558	Discussion on SBFD random access operation	xiaomi

R1-2400607	Discussion on SBFD random access operation	OPPO

R1-2400651	Random access in SBFD symbols	Sharp

R1-2400661	Discussion on SBFD random access operation	China Telecom

R1-2400688	Discussion on enhancements for SBFD random access operations	InterDigital, Inc.

R1-2400732	Random access on SBFD resources	Samsung

R1-2400803	Discussion on random access for subband non-overlapping full duplex	NEC

R1-2400827	Discussion on SBFD random access operation	Panasonic

R1-2400839	SBFD random access operation	Nokia, Nokia Shanghai Bell

R1-2400853	SBFD RACH operations	Sony

R1-2400877	Discussion on Random Access for subband non-overlapping full duplex	SK Telecom

R1-2401012	Views on SBFD random access operation	Apple

R1-2401084	SBFD random access operation	Lenono

R1-2401189	Discussion on SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access	ITRI

R1-2401192	Random access procedure for SBFD	ASUSTeK

R1-2401210	Discussion on random access operation on SBFD symbols	Fujitsu

R1-2401215	Discussion on SBFD random access operation	WILUS

R1-2401228	SBFD random access operation	ETRI

R1-2401250	Discussion on SBFD random access operation	LG Electronics

R1-2401295	Discussion on SBFD random access operation	MediaTek Inc.

R1-2401441	SBFD Random Access Operation	Qualcomm Incorporated

R1-2401676	Summary#1 on SBFD random access operation	Moderator (CMCC)

R1-2401677	Summary#2 on SBFD random access operation	Moderator (CMCC)

R1-2401678	Summary#3 on SBFD random access operation	Moderator (CMCC)

R1-2400110	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2400302	Discussion on CLI handling for Rel-19 duplex operation	ZTE

R1-2401442	Enhancements for CLI handling	Qualcomm Incorporated

R1-2400055	Discussion on CLI handling	Spreadtrum Communications

R1-2400156	Discussion on CLI handling	New H3C Technologies Co., Ltd.

R1-2400179	Discussion on SBFD CLI Handling	Tejas Network Limited

R1-2400243	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2400327	Discussion on CLI handling	CMCC

R1-2400434	Discussion on CLI handling for NR duplex evolution	CATT

R1-2400559	Discussion on CLI handling for SBFD operation	xiaomi

R1-2400608	CLI handling in NR duplex operation	OPPO

R1-2400689	Discussion on CLI handling for SBFD operations	InterDigital, Inc.

R1-2400733	Cross-link interference handling for SBFD	Samsung

R1-2400776	Discussion on CLI handling for SBFD operation	Fujitsu

R1-2400804	CLI handling for NR duplex operation	NEC

R1-2400828	Discussion on CLI handling	Panasonic

R1-2400835	Cross-link interference management for duplexing evolution	Lenovo

R1-2400840	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2400854	CLI handling for SBFD	Sony

R1-2401013	Views on CLI handling	Apple

R1-2401118	Discussion on CLI handling for sub-band full duplex (SBFD)	NTT DOCOMO, INC.

R1-2401216	Discussion on CLI handling for evolution of NR duplex operation	WILUS

R1-2401219	CLI handling	Ericsson

R1-2401229	Discussion on CLI handling for SBFD operation	ETRI

R1-2401246	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Inc.

R1-2401251	Discussion on CLI handling	LG Electronics

R1-2401274	Discussion on CLI handling	CEWiT

R1-2401296	Discussion on CLI handling in SBFD	MediaTek Inc.

R1-2401633	Summary #1 of CLI handling	Moderator (Huawei)

R1-2401634	Summary #2 of CLI handling	Moderator (Huawei)

R1-2401635	Summary #3 of CLI handling	Moderator (Huawei)

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template