3GPP TSG-RAN4 Meeting #110	R4-2401774

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2119 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) CR for TS 38.101-1 to correct the Finterferer (offset) for intra-band CA ACS and IBB requirements (R18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) In clause 7.5A.1, the meaning of FInterferer (offset) is different between <=2700MHz case and >=3300MHz case. Based on the definition in clause 3.2 symbols, it’s said that FInterferer (offset) Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured) Thus, the meaning of <=2700MHz case is correct which is aligned with LTE spec, i.e. frequency offset between the center frequency of the interferer and the center frequency of the carrier closest to the interferer. 2) The similar misunderstanding about FInterferer (offset) in intra-band CA IBB requirements need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To update FInterferer (offset) in both intra-band CA ACS and IBB in order to align the meaning between LTE and NR spec. - To add some clarification for FInterferer (offset) definition for intra-band CA in clause 3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In clause 7.5A.1, the meaning of FInterferer (offset) is different between <=2700MHz case and >=3300MHz case. There are some misalignment between LTE and NR spec for intra-band CA FInterferer (offset). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.5A.1, 7.6A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIBC	Allowed reference sensitivity relaxation due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed reference sensitivity relaxation due to support for intra-band non-contiguous CA operation

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured). For intra-band contiguous CA, the FInterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF, shift	RF reference frequency for Supplementary Uplink (SUL) bands, the uplink of all FDD bands, and TDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PMaxOutputPower 	The rated maximum ATG UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability [RatedMOPATG]

PPowerClass	The nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PREFSENS_SL 	The REFSENS power for Sidelink

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	The lowest RB index of transmitted resource blocks

RBstart_CA	The lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in clause 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

<<Next of Change>>

7.5A.1	Adjacent channel selectivity for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.5A.1-1 and 7.5A.1-1a for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.1-2, 7.5A.1-2a, 7.5A.1-3 and 7.5A.1-3a.

Table 7.5A.1-1: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D |  |
| ACS | dB | 26.0 | 33.0 | 25.2 |  |

Table 7.5A.1-1a: ACS for intra-band contiguous CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz

|  |  | NR CA bandwidth class |  |
| --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 20.0 | 17.0 |

Table 7.5A.1-2: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
| PInterferer | dBm | Aggregated power + 24.5 dB | Aggregated power + 31.5 dB | Aggregated power + 23.7 dB |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 - Foffset | BWchannel CA/2 + Foffset / -BWchannel CA/2 + Foffset | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

Table 7.5A.1-2a: Test parameters for intra-band contiguous CA with FDL_high<2700 MHz and FUL_high<2700 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 18.5 dB | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1-3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49.5 + 10log(NRB,c/NRB_agg) | -56.5 | -48.7 + 10log(NRB,c/NRB_agg) |  |
| PInterferer | dBm | -25 | -25 | -25 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 -Foffset | BWchannel CA/2 + Foffset / -BWchannel CA/2 + Foffset | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

Table 7.5A.1-3a: Test parameters for intra-band contiguous CA with FDL_high <2700 MHz and FUL_high<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -43.5 + 10log(NRB,c/NRB_agg) | -40.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

<<Next of Change>>

7.6A.2.1	In-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6A.2.1-1 and 7.6A.2.1-1a for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6A.2.1-1: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 10.0 | 6 | 13.8 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FIoffset, case 1 | MHz | 30 | BWchannel CA+ BWchannel CA/2 | 75 |  |
| FIoffset, case 2 | MHz | 50 | BWInterferer + FIoffset, case 1 | 125 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattrn OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |

Table 7.6A.2.1-1a: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below |  |
|  | dB | 16.0 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table 7.6A.2.1-2: In-band blocking for intra-band contiguous CA  with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | - Foffset /2 –FIoffset, case 1 and Foffset /2 +FIoffset, case 1 | ≤ - Foffset /2 –FIoffset, case 2 and ≥Foffset /2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: - Foffset /2 – FIoffset, case 1; b: Foffset /2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n2, n3, n25, n38, n41, n66, n484, n40 | Finterferer (offset) | MHz | - Foffset /2 –FIoffset, case 1 and Foffset /2 +FIoffset, case 1 | ≤ - Foffset /2 –FIoffset, case 2 and ≥Foffset /2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: - Foffset /2 – FIoffset, case 1; b: Foffset /2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

<<End of Change>>