3GPP TSG-RAN Meeting #104 																				RP-241675

Shanghai, China, June 17-20, 2024

Title:	Moderator summary for Rel-19 RAN4 Baskets

Agenda item:			9.1.5

Source:	RAN4 Chair (Huawei)

Document for:	Report

Introduction

This contribution summarizes the offline draft session discussions for Rel-19 RAN4 basket WIs.

Disucssions on how to consolidate the basket WIs

The proposals from RAN4 Chair were summarized below. And the RP-240894 was endorsed in prinicple on Monday session in RAN#104 meeting. The corresonding table capturing the idea for consolication is copied below.

| Category | Rel-18 basket WIs |  | Rel-19 basket WIs proposal |  | TR | Scope |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC basket | 6 items | DC_R18_1BLTE_1BNR_2DL2UL (RP-241218) DC_R18_2BLTE_1BNR_3DL2UL (RP-241246) DC_R18_xBLTE_1BNR_yDL2UL (RP-241167) DC_R18_xBLTE_2BNR_yDL2UL (RP-241331) DC_R18_xBLTE_yBNR_zDL2UL (RP-241228) DC_R18_xBLTE_yBNR_zDL3UL (N/A) | DC_R19_xBLTE_yBNR - Obj#1: DC_R19_1BLTE_1BNR_2DL2UL - Obj#2: DC_R19_xBLTE_yBNR_3DL2UL (x+y=3) - Obj#3: DC_R19_xBLTE_yBNR_zDLqUL (x+y>3, 3<z≤6, 2≤q≤3) - Obj#4: DC_R19_LTE_NR_SUL_combos | 1 item | Yes Yes No No | Incl. 2Tx/3Tx Incl. 2Tx/3Tx Incl. 2Tx/3Tx |
| NR CA/DC/SUL basket | 6 items | NR_CA_R18_Intra (RP-241141) NR_CADC_R18_2BDL_xBUL (RP-241227) NR_CADC_R18_3BDL_xBUL (RP-241092) NR_CADC_R18_yBDL_xBUL (RP-241329) NR_SUL_combos_R18 (RP-241249) NR_2SUL_cell_combos_R18 (RP-241204) | NR_CADC_SUL_R19 - Obj#1: NR_CA_R19_Intra - Obj#2: NR_CADC_R19_2BDL_xBUL - Obj#3: NR_CADC_R19_3BDL_xBUL - Obj#4: NR_CADC_R19_yBDL_xBUL (y>3) - Obj#5: NR_SUL_combos_R19 | 1 item | Yes Yes Yes Yes | Incl. 2Tx/3Tx, UL-MIMO/TxD+CA Incl. 2Tx/3Tx, 3Tx UL-MIMO/TxD+CA Incl. 2Tx/3Tx, 3Tx UL-MIMO/TxD+CA Incl. 2Tx/3Tx, 3Tx UL-MIMO/TxD+CA |
| LTE basket | 1 item | LTE_CA_R18_xBDL_yBUL (RP-241243) | LTE_CA_R19_xBDL_yBUL | 1 item | Yes |  |
| V2X/SL | 1 item | NR_LTE_V2X_PC5_combos_R18 (N/A) | N/A |  |  |  |
| HPUE basket | 7 items | HPUE_NR_FR1_TDD_R18 (RP-241203) HPUE_NR_FR1_FDD_R18 (RP-241321) LTE_NR_HPUE_FWVM_R18 (RP-240944) | HPUE_NR_FR1_bands_R19 | 4 items | Yes Yes Yes Yes | PC1/1.5/2, single F/T band, PC1 for FWVM PC1/1.5/2, single LTE, PC1 for FWVM PC1.5/2, 2Tx/3Tx PC1.5/2, 2Tx/3Tx |
|  |  | LTE_NR_HPUE_FWVM_R18 (RP-240944) | HPUE_LTE_bands_R19 |  |  |  |
|  |  | HPUE_FR1_DC_LTE_NR_R18 (RP-241145) | HPUE_DC_LTE_NR_R19 |  |  |  |
|  |  | HPUE_NR_FR1_TDD_intra_CA_R18 (RP-241255) HPUE_FR1_TDD_NR_CADC_SUL_R18 (RP-241310) HPUE_FR1_FDD_NR_CADC_R18 (RP-241322) | HPUE_NR_CADC_SUL_R19 |  |  |  |
| Other basket | 8 items | NR_bands_UL_MIMO_R18 (RP-241261, RP-241231/2, RP-241332) 4Rx_NR_bands_R18 (RP-241229, RP-241230, RP-241048) NR_FDD_bands_R18_redcap (N/A) | NR_bands_xFeature_R19 | 4 items | No Yes Yes No | Incl. UL-MIMO, 4Rx, 8Rx, RedCap |
|  |  | DL_intrpt_combos_TxSW_R18 (RP-241311) | DL_intrpt_combos_TxSW_R19 |  |  |  |
|  |  | LTE_NR_Simult_RxTx_R18 (RP-241252) | LTE_NR_Simult_RxTx_R19 |  |  |  |
|  |  | NR_bands_R18_BWs (RP-241176) | NR_bands_R19_BWs |  |  |  |
|  |  | R18_3Tx_NR_CA_ENDC (RP-241085/6) LTE_bands_R18_M1_M2_NB1_NB2 (Ericsson) |  |  |  |  |

According to offline discussions, the following issues need be discussed further:

Issue #1: how to handle HPUE band combination WIs - Whether to merge HPUE EN-DC/NR-CA basket WIs into normal basket WIs (Nokia, Ericsson, AT&T)

Discussions:

Qualcomm: support to merge them.

LGE: Keep PC3 in the normal basket.

Ericsson: What is the reason?

ZTE: OK to separate them. For the spreadsheet, it is difficult to merge them.

CATT: The worksplit should be clarified. The merger does not change the workload. We should follow the procedure first to work on normal PC and the high power.

OPPO: The movitation of consolidation is for the workload reduction form management. It is very difficult to merge further. Prefer to keep them separate.

Apple: prefer to keep HPUE separate from normal. The PC3 is prerequisite for HPUE.

Samsung: Prefer to have separate WIs. The pre-condition is PC3.

AT&T: Understand the argument. At the same time in the HPUE, there are three power classes. We still have the similar issue. Separate WIs does not help addressing the CR redundancy and spec quality issue.

Vivo: Support to separate WIs. PC3 should be introduced first.

Huawei: prefer to keep them separate. From procedure perspective. The requirements for normal PC and high power class are different.

CHTTL: Share the similar view as Apple and Vivo. In HPUE WIDs, we can say that PC3 and high power can be finished in the same meeting.

Nokia: Similar spreadsheet needs for seaprate WIDs. Support to merge them.

Verizon: Merging them does not impact the workload.

Apple: In SR, whether PC3 is finished needs be checked and clarified.

Agreements:

- Keep PC3 and HPUE class band combiantions in the normal basket WIs and HPUE basket WIs separately.

  - In the WIDs of HPUE, the rule how to complete the work as described in TR will be captured.

Issue #2: How to handle 3Tx - How to capture 3Tx, in normal basket + HPUE basket or in HPUE basket only with fallback mode (OPPO)

Discussions:

OPPO: Based on Rel-18 experience, people are only interested in the 3Tx HPUE.

CHTTL: What if companies only request PC3 with 3Tx.

CMCC: we do not nee worry about it.

CATT: Only HPUE is important. If there is such request, we can update the WID.

Agreements:

- Capture 3Tx with high power classes and PC3 as fallback in HPUE_DC_LTE_NR_R19 and HPUE_NR_CADC_SUL_R19.

Issue #3: Whether and how to handle non-block approval aspects - A separate objective could be added to handle band combos under none-block approval process if necessary in the same basket, e.g., NR_CADC_SUL_R19 (CATT)

In the moderator’s view, which topic within the WI will be treated in non-block approval approach could be decided in RAN4 level. And in Rel-18 all the HPUE basket WIs were treated in non-block approval manner. There seems no need to have a dedicated objective for non-block approval process in WID.

Discussions:

AT&T: Consider the block approval for HPUE basket WIs in the future RAN4 meeting.

Issue #4: Detailed objectives for each Rel-19 potential WIs - Detailed objectives are provided in the table below for discussions.

Agreement: (marked by Green)

| Category | Rel-18 basket WIs |  | Rel-19 basket WIs proposal |  | TR |
| --- | --- | --- | --- | --- | --- |
| EN-DC basket | 6 items | DC_R18_1BLTE_1BNR_2DL2UL (RP-241218) DC_R18_2BLTE_1BNR_3DL2UL (RP-241246) DC_R18_xBLTE_1BNR_yDL2UL (RP-241167) DC_R18_xBLTE_2BNR_yDL2UL (RP-241331) DC_R18_xBLTE_yBNR_zDL2UL (RP-241228) DC_R18_xBLTE_yBNR_zDL3UL (N/A) | DC_R19_xBLTE_yBNR - Obj#1: DC_R19_1BLTE_1BNR_2DL2UL - Obj#2: DC_R19_xBLTE_yBNR_3DL2UL (x+y=3) - Obj#3: DC_R19_xBLTE_yBNR_zDLqUL (x+y>3, 3<z≤6, 2≤q≤3) - Obj#4: DC_R19_LTE_NR_SUL_combos Note: low band- low band combinations should not be under the block approval approaches by default. It is not precluded for RAN4 to discuss the band combination feasibility. | 1 item | Yes Yes No Yes |
| NR CA/DC/SUL basket | 6 items | NR_CA_R18_Intra (RP-241141) NR_CADC_R18_2BDL_xBUL (RP-241227) NR_CADC_R18_3BDL_xBUL (RP-241092) NR_CADC_R18_yBDL_xBUL (RP-241329) NR_SUL_combos_R18 (RP-241249) NR_2SUL_cell_combos_R18 (RP-241204) | NR_CADC_SUL_R19 - Obj#1: NR_CA_R19_Intra - Obj#2: NR_CADC_R19_2BDL_xBUL - Obj#3: NR_CADC_R19_3BDL_xBUL - Obj#4: NR_CADC_R19_yBDL_xBUL (y>3) - Obj#5: NR_SUL_combos_R19 Note: low band- low band combinations should not be under the block approval approaches by default. It is not precluded for RAN4 to discuss the band combination feasibility. Note: Include UL-MIMO/TxD + CA | 1 item | Yes Yes Yes No Yes |
| LTE basket | 1 item | LTE_CA_R18_xBDL_yBUL (RP-241243) | LTE_CA_R19_xBDL_yBUL - Obj: follow RP-241243 | 1 item | Yes |
| V2X/SL | 1 item | NR_LTE_V2X_PC5_combos_R18 (N/A) | N/A |  |  |
| HPUE basket | 7 items | HPUE_NR_FR1_TDD_R18 (RP-241203) HPUE_NR_FR1_FDD_R18 (RP-241321) LTE_NR_HPUE_FWVM_R18 (RP-240944) | HPUE_NR_FR1_bands_R19 - Obj#1: TDD band with PC1.5/2 for the different device types and PC1 only for FWVM - Obj#2: FDD band with PC2 for the different device types and PC1 only for FWVM Note: The bands with the default power class shall be introduced before the high power UE is requested for this band. | 4 items | Yes Yes Yes Yes |
|  |  | LTE_NR_HPUE_FWVM_R18 (RP-240944) | HPUE_LTE_bands_R19 - Obj: PC2 with single Tx and PC1 only for FWVM Note: The feasibility for PC1/PC2 needs be confirmed first |  |  |
|  |  | HPUE_FR1_DC_LTE_NR_R18 (RP-241145) | HPUE_DC_LTE_NR_R19 - Obj: follow RP-241145 |  |  |
|  |  | HPUE_NR_FR1_TDD_intra_CA_R18 (RP-241255) HPUE_FR1_TDD_NR_CADC_SUL_R18 (RP-241310) HPUE_FR1_FDD_NR_CADC_R18 (RP-241322) | HPUE_NR_CADC_SUL_R19, include 2Tx and 3Tx - Obj#1: Intra-band UL TDD HPUE and intra-band DL FDD HPUE with single UL carrier - Obj#2: Inter-band combinations with high power on TDD band(s) with or without SUL - Include PC3 TDD +PC3 TDD case - Obj#3: Inter-band combinations with high power on FDD band(s) - Include PC3 FDD +PC3 FDD case - Obj#4: inter-band combination with high power on either TDD, FDD band or both - Include PC3 FDD + PC3 TDD cases Note: for 3Tx, both high power classes and PC3 for band combinations will be included in this WID. Note: The common requirements should be done first before working on the band specific requirements Note: Include UL-MIMO/TxD + CA |  |  |
| Other basket | 8 items | NR_bands_UL_MIMO_R18 (RP-241261, RP-241231/2, RP-241332) 4Rx_NR_bands_R18 (RP-241229, RP-241230, RP-241048) NR_FDD_bands_R18_redcap (N/A) | NR_bands_xFeature_R19 - Obj#1: UL-MIMO for single band - Obj#2: 4Rx - Obj#3: 8Rx - Other objectives can be added in the future if agreeable. | 4 items | No Yes Yes Yes |
|  |  | DL_intrpt_combos_TxSW_R18 (RP-241311) | DL_intrpt_combos_TxSW_R19 - Obj: follow RP-241311 |  |  |
|  |  | LTE_NR_Simult_RxTx_R18 (RP-241252) | LTE_NR_Simult_RxTx_R19 - Obj: follow RP-241252 |  |  |
|  |  | NR_bands_R18_BWs (RP-241176) | NR_bands_R19_BWs - Obj: follow RP-241176 or revised RP-241176 with adding CBW for n48 |  |  |
|  |  | R18_3Tx_NR_CA_ENDC (RP-241085/6) LTE_bands_R18_M1_M2_NB1_NB2 (Ericsson) |  |  |  |

Discussions:

Agreements:

Issue #5 - For potential new NR single band, now HPUE(PC2+) is considered, perhaps PC3 can also be considered into baskets (CATT)

The clarificaiton on the proposal would be needed. The issue#5 was addressed in the discussions for issue#4.

Issue #6: Spreadsheet inside the basket WIs - Whether to use the combined spreadsheet or not

Discussions:

Ericsson/Nokia: perfer to merge into a single spreadsheet. Otherwise, there is no point to consolidate the WID.

LGE: it is easier for rapportuer to maintain the separate spreadsheets.

CATT: Only single excel file with multiple spread sheet.

CHTTL: RAN4 needs separate spreadsheet to be managed by multiple people. When companies requests the band combiantions, they request in one spreadsheet.

ZTE: In general we share the same view as CHTTL.

OPPO: Separate.

Nokia: We should avoid the mistake.

AT&T: for single spreadsheet, we prefer to have less spreadsheets. We do not like multiple spreadsheet for each objective.

Agreements:

- Not to merge the spreadsheets (multiple files) in this meeting, and merge them into one spreadsheet with addtional collumn(s) to indicate the corresponding objective in the next meeting.

Recommended WF

The recommended WF is as follows.

Agreements:

- Keep PC3 and HPUE class band combiantions in the normal basket WIs and HPUE basket WIs separately.

  - In the WIDs of HPUE, the rule how to complete the work as described in TR will be captured.

- Capture 3Tx with high power classes and PC3 as fallback in HPUE_DC_LTE_NR_R19 and HPUE_NR_CADC_SUL_R19.

- Not to merge the spreadsheets (multiple files) in this meeting, and merge them into one spreadsheet with addtional collumn(s) to indicate the corresponding objective in the next meeting.

The agreements on the objectives for each WI are as follows marked by Green.

| Category | Rel-18 basket WIs |  | Rel-19 basket WIs proposal |  | TR |
| --- | --- | --- | --- | --- | --- |
| EN-DC basket | 6 items | DC_R18_1BLTE_1BNR_2DL2UL (RP-241218) DC_R18_2BLTE_1BNR_3DL2UL (RP-241246) DC_R18_xBLTE_1BNR_yDL2UL (RP-241167) DC_R18_xBLTE_2BNR_yDL2UL (RP-241331) DC_R18_xBLTE_yBNR_zDL2UL (RP-241228) DC_R18_xBLTE_yBNR_zDL3UL (N/A) | DC_R19_xBLTE_yBNR - Obj#1: DC_R19_1BLTE_1BNR_2DL2UL - Obj#2: DC_R19_xBLTE_yBNR_3DL2UL (x+y=3) - Obj#3: DC_R19_xBLTE_yBNR_zDLqUL (x+y>3, 3<z≤6, 2≤q≤3) - Obj#4: DC_R19_LTE_NR_SUL_combos Note: low band- low band combinations should not be under the block approval approaches by default. It is not precluded for RAN4 to discuss the band combination feasibility. | 1 item | Yes Yes No Yes |
| NR CA/DC/SUL basket | 6 items | NR_CA_R18_Intra (RP-241141) NR_CADC_R18_2BDL_xBUL (RP-241227) NR_CADC_R18_3BDL_xBUL (RP-241092) NR_CADC_R18_yBDL_xBUL (RP-241329) NR_SUL_combos_R18 (RP-241249) NR_2SUL_cell_combos_R18 (RP-241204) | NR_CADC_SUL_R19 - Obj#1: NR_CA_R19_Intra - Obj#2: NR_CADC_R19_2BDL_xBUL - Obj#3: NR_CADC_R19_3BDL_xBUL - Obj#4: NR_CADC_R19_yBDL_xBUL (y>3) - Obj#5: NR_SUL_combos_R19 Note: low band- low band combinations should not be under the block approval approaches by default. It is not precluded for RAN4 to discuss the band combination feasibility. Note: Include UL-MIMO/TxD + CA | 1 item | Yes Yes Yes No Yes |
| LTE basket | 1 item | LTE_CA_R18_xBDL_yBUL (RP-241243) | LTE_CA_R19_xBDL_yBUL - Obj: follow RP-241243 | 1 item | Yes |
| V2X/SL | 1 item | NR_LTE_V2X_PC5_combos_R18 (N/A) | N/A |  |  |
| HPUE basket | 7 items | HPUE_NR_FR1_TDD_R18 (RP-241203) HPUE_NR_FR1_FDD_R18 (RP-241321) LTE_NR_HPUE_FWVM_R18 (RP-240944) | HPUE_NR_FR1_bands_R19 - Obj#1: TDD band with PC1.5/2 for the different device types and PC1 only for FWVM - Obj#2: FDD band with PC2 for the different device types and PC1 only for FWVM Note: The bands with the default power class shall be introduced before the high power UE is requested for this band. | 4 items | Yes Yes Yes Yes |
|  |  | LTE_NR_HPUE_FWVM_R18 (RP-240944) | HPUE_LTE_bands_R19 - Obj: PC2 with single Tx and PC1 only for FWVM Note: The feasibility for PC1/PC2 needs be confirmed first |  |  |
|  |  | HPUE_FR1_DC_LTE_NR_R18 (RP-241145) | HPUE_DC_LTE_NR_R19 - Obj: follow RP-241145 |  |  |
|  |  | HPUE_NR_FR1_TDD_intra_CA_R18 (RP-241255) HPUE_FR1_TDD_NR_CADC_SUL_R18 (RP-241310) HPUE_FR1_FDD_NR_CADC_R18 (RP-241322) | HPUE_NR_CADC_SUL_R19, include 2Tx and 3Tx - Obj#1: Intra-band UL TDD HPUE and intra-band DL FDD HPUE with single UL carrier - Obj#2: Inter-band combinations with high power on TDD band(s) with or without SUL - Include PC3 TDD +PC3 TDD case - Obj#3: Inter-band combinations with high power on FDD band(s) - Include PC3 FDD +PC3 FDD case - Obj#4: inter-band combination with high power on either TDD, FDD band or both - Include PC3 FDD + PC3 TDD cases Note: for 3Tx, both high power classes and PC3 for band combinations will be included in this WID. Note: The common requirements should be done first before working on the band specific requirements Note: Include UL-MIMO/TxD + CA |  |  |
| Other basket | 8 items | NR_bands_UL_MIMO_R18 (RP-241261, RP-241231/2, RP-241332) 4Rx_NR_bands_R18 (RP-241229, RP-241230, RP-241048) NR_FDD_bands_R18_redcap (N/A) | NR_bands_xFeature_R19 - Obj#1: UL-MIMO for single band - Obj#2: 4Rx - Obj#3: 8Rx - Other objectives can be added in the future if agreeable. | 4 items | No Yes Yes Yes |
|  |  | DL_intrpt_combos_TxSW_R18 (RP-241311) | DL_intrpt_combos_TxSW_R19 - Obj: follow RP-241311 |  |  |
|  |  | LTE_NR_Simult_RxTx_R18 (RP-241252) | LTE_NR_Simult_RxTx_R19 - Obj: follow RP-241252 |  |  |
|  |  | NR_bands_R18_BWs (RP-241176) | NR_bands_R19_BWs - Obj: follow RP-241176 or revised RP-241176 with adding CBW for n48 |  |  |
|  |  | R18_3Tx_NR_CA_ENDC (RP-241085/6) LTE_bands_R18_M1_M2_NB1_NB2 (Ericsson) |  |  |  |

Annex: Recommended status for each tdoc under this topic

The recommended status for each tdoc under this topic

| Tdoc number | Title | Sources | Recommended status |
| --- | --- | --- | --- |
| RP-241218 | New WID: Rel-19 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL) | CHTTL | Noted |
| RP-241246 | New WID: Rel-19 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | Huawei, HiSilicon | Noted |
| RP-241167 | New WID: Rel-19 DC of x bands LTE inter-band CA (x345) and 1 NR band | Nokia | Noted |
| RP-241331 | New WID on Rel-19 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | LG Electronics Deutschland | Noted |
| RP-241228 | New WID: Rel-19 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and y bands (3<=y<=5 and x+y <= 6) NR inter-band CA (yDL/1UL) | ZTE Corporation | Noted |
| RP-241141 | New WID: Basket Rel-19 WID NR_CA_R18_Intra | Ericsson | Noted |
| RP-241227 | New WID: Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | ZTE Corporation | Noted |
| RP-241092 | New WID: Rel-19 NR Inter-band CA DC for 3 bands DL with x bands UL (x=1,2) | ZTE Corporation | Noted |
| RP-241329 | New WID: Rel-19 NR inter-band Carrier Aggregation/Dual Connectivity for y bands DL (y=4, 5, 6) with x bands UL (x=1, 2) | Ericsson GmbH, Eurolab | Noted |
| RP-241249 | New WID: Rel-19 Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | Huawei, HiSilicon | Noted |
| RP-241204 | New WID: NR CA band combinations with two SUL cells in Rel-19 | CMCC, China Telecom | Noted |
| RP-241243 | New WI: Rel-19 LTE-Advanced Carrier Aggregation for x bands (2<=x<= 6) DL with y bands (y=1, 2) UL | Huawei, HiSilicon | Revised |
| RP-241203 | New WID: Rel-19 High power UE (power class 1.5) for NR FR1 TDD single band | CMCC | Noted |
| RP-241321 | New WID on Rel-19 High power UE (power class 2) for NR FR1 FDD single band | China Unicom | Noted |
| RP-240944 | New WID of High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands Rel-19 | Nokia | Noted |
| RP-241145 | New WID on Rel-19 High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL) | Ericsson | Revised |
| RP-241255 | New WID: High power UE (power class 1.5 or 2) for intra-band Carrier Aggregation combinations of a single NR FR1 TDD band | Huawei, HiSilicon | Noted |
| RP-241310 | New WID: Rel-19 High power UE (power class 1,5 and 2) for a single FR1 NR TDD band in UL of NR inter-band CA/DC combinations with/without NR SUL (supplementary uplink) with y bands downlink (y=2,3,4,5,6) and x bands uplink (x=1,2) | China Telecom | Noted |
| RP-241322 | New WID Rel-19 High power UE (power class 2) for FR1 NR FDD band in UL of NR CADC combinations | China Unicom | Noted |
| RP-241261 | New WI: Additional NR bands and intra-band contiguous UL CA band combinations for UL-MIMO in Rel-19 | Huawei, HiSilicon | Noted |
| RP-241231 | Motivation on R19 basket WID on NR intra-band contiguous CA with UL MIMO | ZTE Corporation, Sanechips, Samsung | Noted |
| RP-241232 | New WID R19 NR intra-band contiguous CA with UL MIMO | ZTE Corporation, Samsung | Noted |
| RP-241332 | Motivation for NR 3Tx HPUE inter-band ULCA with FDD UL MIMO | SoftBank Corp., OPPO, Nokia, Huawei, HiSilicon, ZTE, MediaTek | Noted |
| RP-241229 | Motivation on R19 WID nRx(n=4, 8) for NR bands (<2.6GHz) | ZTE Corporation, Sanechips, China Telecom | Noted |
| RP-241230 | New WID: Rel-19 nRx(n=4, 8) for NR bands (<2.6GHz) | ZTE,China Telecom | Noted |
| RP-241048 | Discussion on 8Rx receiver | vivo | Noted |
| RP-241311 | New WID on Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink | China Telecom | Revised |
| RP-241252 | New WI: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19 | Huawei, HiSilicon | Revised |
| RP-241176 | New Basket WID on adding channel bandwidth support to existing NR bands | Ericsson Limited | Revised |
| RP-241086 | Motivation for R19 3Tx basket WI | OPPO,Samsung,MediaTek | Noted |
| RP-241085 | Rel-19 basket WI for NR inter-band UL CA and EN-DC with 3Tx | OPPO,Samsung,MediaTek | Noted |
| RP-241336 | Motivation for R19 Low-Low inter-band CA basket WI | CATT, China Telecom, Spark | Noted |
| RP-241337 | New WID on Rel-19 NR inter-band Carrier Aggregation for Low-Low bands | CATT, China Telecom, Spark | Noted |
| RP-240894 | Proposed RAN4 REL-19 basket WIs | RAN4 chair (Huawei) | Noted |
| RP-241168 | On RAN4 basket WI planning | Nokia, Ericsson | Noted |
| RP-241309 | Discussion on RAN4 basket WI plan | China Telecom | Noted |
| RP-241335 | Considerations for potential improvements in Rel-19 RAN4 basket work | CATT | Noted |