3GPP TSG-RAN Meeting #107	RP-250171

Incheon, Korea, March 12th – 14th, 2025

Agenda item:	15.2.3

Title:	Discussion on potential scoping for Ambient IoT in Rel-20

Source:	OPPO

Document for:	Discussion and Decision

# 1. Introduction

During RAN#106 meeting, a workshop on 5G-Advanced in Rel-20 was held for RAN1/2/3 led topics and a RAN Chair’s summary on categorization of topics based on WG contributions was endorsed in [1] (copied below), where the technical topics in the first column (which includes Ambient IoT) are considered to be high priority for 5G-Advanced in Rel-20.

Subsequently, during RAN#106 meeting a way forward on Ambient IoT work scope and workplan for Rel-19 and Rel-20 was also endorsed [2], where the endorsed workplan for A-IoT in 5G-A Rel-20 includes the following.

Workplan for Rel-20: - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase. For both Rel-19 and Rel-20, co-ordination with SA2/3 is expected

To this end, it is expected that further details and refinement of scope for A-IoT in R20 are to be discussed during this RAN#107 meeting, in preparation for an approval in RAN#108 (June 2025). In this contribution, we provide our views and suggestions on the detailed scope of A-IoT work in Rel-20 that should be considered for approval in the next RAN plenary meeting.

# Discussions

According to the approved WID of Rel-19 A-IoT [3], the general scope for the normative work includes deployment scenario 1 with topology 1 (specifically D1T1-B in TR 38.769 [4]), Device 1 only (~1 µW peak power consumption), FR1 licensed spectrum in FDD with CW in the UL spectrum, A-IoT BS located indoor, and traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). Based on the study report in [4], this general scope represents only a subset of what had been studied during the study phase of A-IoT in Rel-19. For the leftover aspects that had been studied in WGs but not to be specified and supported in Rel-19, it was endorsed in [2] the workplan for Rel-20 will include both D2T2 and Device 2 with limited number of additional use cases, deployments, connectivity topologies, devices, and traffic types from TR38.848 [5] (which is a RAN level study TR).

Firstly, according to TR38.769 [4] from the WG-level study, D1T1 and D2T2 encompasses several sub-scenarios based on whether the CW is inside / outside topology or no CW is used in the deployment. For the indoor use cases, D1T1-B (BS indoor, device indoor, CW outside topology) was selected to be supported and specified due to simplicity, quick to market, no intermediate UE involvement (less specification effort) and only 3 remaining WGs left in Rel-19. For D1T1-A1 (CW node inside topology 1, ‘CW’ in CW2D and ‘R2’ in D2R are different, ‘CW’ in CW2D and ‘R1’ in R2D are same, ‘R1’ in R2D and ‘R2’ in D2R are different), D1T1-A2 (same reader for R2D and D2R and same node for the reader and CW) and D1T1-C (no CW) could provide further deployment options in Rel-20. Moreover, Device 2a/b provides a larger coverage (D2R communication distance) than Device 1. However, compared to D1T1-B, the necessity and additional benefit(s) from supporting A1/2 should be further studied / identified.

Observation 1: The necessity and additional benefit(s) from supporting D1T1-A1/2 should be further discussed and justified for Rel-20 in RAN.

For the deployment scenarios in D2T2, where the BS is located outdoor and device is indoor (which would require a help from an intermediate UE located in indoor), these will enable the support to new/additional A-IoT indoor use cases in Rel-20 such as 5.2 Medical instruments inventory management and positioning, 5.6 Smart homes, 5.15 Smart laundry, 5.11 Online modification of medical instruments status, and 5.26 Elderly Health Care in TR 38.848 [5]. Typically, the number of A-IoT devices in these use cases are relatively/much smaller than a warehouse and factory, it is not affordable and sometimes infeasible (e.g., due to regulations) to deploy a BS indoor especially for them in the premise. The situation would be worse when considering that these devices may be distributed with physical blockages in-between or into different rooms separated by walls. Therefore, they will rely on a UE reader which could be a smartphone connects to a serving base station in order to make an inventory or sending commands to the A-IoT devices that are located indoor. Furthermore, with the use of UE readers, it is possible for the already deployed BS can be reused with only software updates to support of A-IoT services. As such, it is functional necessary and endorsed for the inclusion in workplan for Rel-20 [2].

However, not all deployment scenarios in D2T2 will need to be supported in Rel-20. For scenarios where a CW is needed for backscattering devices, the complication would be in the CW interference handling for coexistence (to both Uu operation and nearby equipment) and the cost of installing a CWN. In some cases, e.g., medical equipment and premises, the operation is sensitive and critical to any radio interferences. For smart home use, it is impractical and expensive for smartphone users to purchase and install a CWN and keep in ON at all time. Furthermore, the additional complexity and cost in a smartphone UE to implement interference suppression and/or to avoid the full duplex problem seem not justifiable. It would be much simpler and overall cheaper to use devices that support active transmission (e.g., Device 2b).

Since the workplan for Rel-20 includes already Device 2, based on Rel-19 study in the WGs, it is clear and a common understanding that device (un)availability direction 2 would be needed to extend the sustainable operating time of a device.

Proposal 1: Deployment scenario D2T2-C, Device 2b and device (un)availability direction 2 should be the main focus for A-IoT normative work in Rel-20.

For the traffic type of DO-A, which was briefly studied in Rel-19 in RAN WGs, it is necessary to be supported in A-IoT for use cases where data is generated by the device of an emergency nature (e.g., triggered by sudden and critical events in medical use or failure) or sensor data reporting. As studied and identified in TR 38.848 [5], this is one traffic type that should be supported in A-IoT operations but not yet so far. Hence, this should be included as part of normative work in Rel-20.

Proposal 2: DO-A traffic type should be supported for A-IoT normative work in Rel-20.

Another popular use case group and deployment studied and identified in during the RAN-level study for A-IoT is the outdoor scenario. In TR 38.848 [5], it includes rUC5 (outdoor inventory), rUC6 (outdoor sensor), rUC7 (outdoor positioning) and rUC8 (outdoor command). For the outdoor use, the coverage should support at least micro-cell or pico-cell based deployment, where the target distance should be between 50m to 500m, which is above the design target for the existing device 1 and 2a. Note that, outdoor use cases have not been evaluated so far in Rel-19. If it needs to be supported by Rel-20 specification, the normative work should be based on what is/are supported in Rel-19 for indoor. That is, no UE-based reader (deployment scenario 4), topology 1 only and Device 2b (the only viable device at this stage for a short coverage). It is not advisable to consider Device 1 and 2a due to insufficient coverage and the requirement of a carrier wave node, which creates a challenge in the outdoor coexistence with NR Uu operation.

On the other hand, Topology 4 poses an interesting deployment usage where a BS does not need to be involved. This Topology 4 could be used in both indoor and outdoor deployment scenario without the need of large coverage. But additional work is required relating to resource allocation for the UE reader without a BS. For Device C, which supports up to 10mW peak power consumption but device architecture is still unknown, would require a study in both RAN1 and RAN4. Furthermore, for outdoor deployment where a CWN is not used, it is unclear whether device energy harvesting could be based on RF energy source. Considering Device C peak power consumption is much higher than Device 2, other energy sources maybe needed and this aspect should be also studied, including the sustainable operating time.

Proposal 3: For outdoor use cases,

- Deployment scenario 4, Topology 1 and Device 2b could be considered for normative work in Rel-20.

- A study phase is needed at least for Topology 4, Device C and other energy sources in Rel-20.

# Conclusion

Overall, our proposal for the scope of A-IoT item in Rel-20 is as followed.

Overall proposal for A-IoT in Rel-20:

- Deployment scenario D2T2-C, Device 2b and device (un)availability direction 2 should be the main focus for A-IoT normative work in Rel-20.

  - The necessity and additional benefit(s) from supporting D1T1-A1/2 should be further discussed and justified for Rel-20 in RAN.

- DO-A traffic type should be supported.

- For outdoor use cases,

  - Deployment scenario 4, Topology 1 and Device 2b could be considered for normative work in Rel-20.

  - A study phase is needed at least for Topology 4, Device C and other energy sources in Rel-20.

# References

[1] RP-243292 “RAN Chair’s Summary of 5G-Advanced in Rel-20”, RAN#106

[2] RP-243280 “Way forward for Ambient IoT for REL-19 and REL-20”, RAN#106

[3] RP-243326 “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106

[4] TR 38.769 “Study on solutions for ambient IoT (Internet of Things)”, V2.0.1 (2024-12)

[5] TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN”, V1.0.0 (2023-09)

[6] RP-242801 “Discussion on Rel-19 Ambient IoT potential work item scope”, OPPO, RAN#106