3GPP TSG RAN Meeting #107						RP-250512

Incheon, Korea, 12th -14th March, 2025

Agenda item:	16

Source:	CATT

Title:	Consideration and proposals on IMT-2030 TPRs

Document for:	Discussion and Decision

# Background

High-level considerations for 6G timeline were provided and endorsed in TSG#102 referring to [1]. Additional consideration for 6G timeline was discussed and endorsed in TSG#103 referring to [2]. In the last meeting, SI on Study on 6G Scenarios and Requirements was approved with justifications [8]:

In December ’24, a LS was received by RAN in RP-243202, in which ITU WP 5D kindly invites External Organizations to provide their inputs on the minimum technical performance requirements (TPR) for IMT-2030 radio interface technologies, including but not limited to,

- Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (4-13 February 2025).

- Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)

and objectives [8]:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- [To fill in additional objectives, on general 6G RAN study, in March’2025]

Certain guidance for this meeting was provided by RAN Chair[9]:

- RAN#107

  - Aim to finalize the candidate set of TPRs

  - Initial discussion on the associated values of the identified TPRs

    - A corresponding summary of the values as an input for future progress

  - Possible LS to ITU-R as necessary, depending on the progress of the study

Between the last and this plenary, some discussions from ITU were also made. From our observation, some outcome on preliminary list of Minimum Technical Performance Requirements(MTPR) includes quantitative MTPR -  Average Spectral efficiency, Peak spectral efficiency, 5th percentile user Spectral efficiency, Bandwidth, Connection density, Control plane latency, User plane latency, Energy Efficiency, Mobility, Mobility interruption time, Peak data rate, User Experienced Data Rate, Sensing-related capabilities, Positioning and Reliability. AI-related capabilities will also be a MTPR while “qualitative” or “quantitative” has not been determined. Some other candidate MTPRs, [Joint requirement on data rate, latency, reliability and capacity],[Coverage],[Security],[Resilience],[Sustainability],[Interoperability] are still under discussion.

Following above background, candidate set of TPRs aim to be finalized and discussion on values would be initialized in this meeting. CATT provides proposals correspondingly.

# Discussion

## Remaining issues of candidate TPRs

The TPRs without square brackets seem reasonable to CATT. However, couple of them are to be determined:

- AI-related capabilities

ITU discussed whether quantitative IMT-2030 KPIs are needed. From our perspective, considering AI as a technology to improve internal communication performance, it can be well reflected in the “inherited” KPIs such as spectral efficiency in 3 evolved usage scenarios. When AI is applied in the system to provide other services, the main advantages shown by IMT-2030 would be its new functions, which could also be applied to improve internal communication performance. With that, qualitative KPI(s) to be inspected would be more proper to present AI, rather than a non-generalization/case-by-case quantitative KPI, i.e. method is “qualitative”.

Its definition is proposed as below:

Table 1. TPR of AI

| Candidate TPR | Definition | Evaluation method |
| --- | --- | --- |
| TPR of AI | The IMT-2030 system is required to provide mechanisms and/or signalling related to functionalities that support AI infrastructure/architecture for AI applications and also utilize AI to improve internal-IMT system performance. These functionalities should at least include AI advanced life cycle management (data collection, advanced training, model inference, etc), data processing and computing. | Inspection |

Proposal 1. Proposed TPR of AI is qualitative with definition in Table 1.

- Sensing-related capabilities

Although there was common ground that there will be quantitative sensing-related TPR, concrete TPRs were not yet decided. In case that sensing UAV is selected for 6G as an example, that apparently comes to KPIs on detection probability, localization accuracy and velocity accuracy.

Table 2.  TPRs of Sensing

| Candidate TPR | Definition | Evaluation method |
| --- | --- | --- |
| Detection probability | The probability of correctly detecting the presence of the object. | Simulation |
| Localization accuracy | The difference between the measured position and the actual position of the sensing target. | Simulation |
| Velocity Accuracy | The difference between the estimated velocity and the actual velocity of the sensing target. | Simulation |

Proposal 2. Proposed TPR of Sensing is quantitative with definitions in Table 2.

•	Energy efficiency

We observed the case of quantitative evaluation of relative energy saving using power model for energy efficiency in ITU. The relative energy savings could be obtained based on the relative ratio between the energy consumption/efficiency to a baseline. Furthermore, the baseline could be energy consumption/efficiency of IMT-2030 in certain condition. There are following possibilities in our mind to be further discussed. One is (1) “full-loaded” IMT-2030, i.e. the relative energy saving could be obtained in different traffic load comparison to full-loaded case. One issue observed is that the relative energy saving depends on traffic load. It is difficult to tell the real ratio of relative energy saving out from that from reduced traffic load. It could be solved by (2) applying a baseline with specific parameter configuration set and keeping the same load as that of evaluated case, while calibration of baseline in a selected loaded case may be needed. The specific parameter configuration set could include e.g. system bandwidth, number of TxRU, DL power level, and etc. Another solution (3) is to introduce throughput (transmitted data over whole evaluation duration) as part of energy efficiency that can somehow mitigate the impact of different loads.

Proposal 3. Before getting into value discussion of Energy Efficiency, a proper baseline needs to be carefully considered.

The TPRs with square brackets indeed need further discussion. Some of them, such as security, resilience are not needed to be TPRs. Instead, such IMT-2030 capabilities can be reflected in other methods from ITU.

## Discussions on usage scenario mapping and potential value targets

It is clear to know that technical performance requirements to be defined in ITU is “minimum requirement” to evaluate whether a candidate technology can be a qualified 6G technology or not, rather than a complete set of requirements for the whole 6G system and all services. KPIs focused on IMT-2030 should represent the most basic elements to be supported in IMT-2030 technologies. Additional KPIs or service requirements can be further discussed as a part of 6G general requirements study.

Table 3 Mapping between candidate IMT-2030 KPI (without square brackets) and usage scenarios

| Num | Candidate IMT-2030 KPIs | Usage Scenarios | Possible values in consideration |
| --- | --- | --- | --- |
| 1 | Peak data rate | IC | 2.5~5x of IMT-2020 target |
| 2 | Peak spectral efficiency | IC | 2.5 of IMT-2020 target |
| 3 | User experienced data rate | IC Dense Urban | 2~3x of IMT-2020 target |
| 4 | 5th percentile user spectral efficiency | IC | 2~3x of IMT-2020 target |
| 5 | Average spectral efficiency | IC | 2~3x of IMT-2020 target |
| 6 | Area traffic capacity | IC InH | 2~3x of IMT-2020 target |
| 7 | User plane latency | IC, HRLLC | TBD |
| 8 | Control plane latency | IC, HRLLC | 10ms |
| 9 | Connection density | MC | 10x of IMT-2020 target |
| 10 | Energy efficiency (UE & Network) | IC | TBD (quantitative) Baseline to relative energy saving should be discussed first Limited impact from one side to the other should be reflected |
| 11 | Reliability | HRLLC | TBD |
| 12 | Mobility | IC | TBD |
| 13 | Mobility interruption time | IC, HRLLC | 0ms |
| 14 | Bandwidth | N.A. | TBD |
| 15 | Positioning | Indoor Factory in ISAC or HRLLC, or Indoor Hotspot - IC | <=10cm |
| 16 | Sensing(detection probability，localization accuracy and velocity accuracy) | Indoor Factory-SH & Urban Macro - AV | TBD |
| 17 | AI relative KPIs | AIAC | Qualitative |

- Spectral efficiency

- Peak spectral efficiency

According to the generic formula, modulation level, coding rate, spatial layers and overhead are key factors. Some of which are difficult to achieve much growth again though. To consider the possible improvement, e.g. modulation and number of streams, the target value is expected to reach 2.5x of IMT-2020 targets, i.e. 75bps/Hz for downlink and 37.5bps/Hz for uplink.

- Average and 5th percentile user spectral efficiency

The improvements may mostly come from larger antenna arrays and better channel rebuild. But considering cost and windward side in the practice, the number of antenna and TxRU cannot increase unlimited. In most current deployment in 5G, the popular number of TxRUs is 32 at gNB and mainly 4 at UE. The number of antenna elements does not beyond 256 at gNB. So we expect that there will be improvement in 6G, but within a reasonable range (e.g., up to 1024). 2~3x of IMT-2020 targets are proposed for both, e.g. 0.6bps-0.9bps 5th percentile user spectral efficiency at downlink Indoor Hotpot.

Also, evaluations for Indoor Hotspot, Dense Urban and Rural for full buffer as defined environments in 5G are sufficient. New carrier frequency, e.g. U6GHz, could be considered to match 6G frequency band.

- Data rate (User experienced data rate and Area traffic capacity)

KPIs of data rate category, including User experienced data rate and Area traffic capacity, mostly could be derived from the spectral efficiency and the supported bandwidth. In 5G, there was pretty high bandwidth applied to reach the IMT-2020 targets, e.g. 800MHz, in the case that the applied spectral efficiency in calculation was actually beyond its IMT-2020 target. It was questioned even in 5G how it could reach the target in practice, particularly for user experienced data rate, which seemed impossible at all. With increase of spectral efficiency, there will be higher target of data rate, but it is not that reasonable to reply on extremely high and non-existing bandwidth/carrier aggregation in practice. The target values of these KPIs are expected to be 2~3x of IMT-2020 targets. The similar consideration comes to peak data rate.

- Latency (Control plane latency and User plane latency)

Substantial decrease in control plane latency is expected on the simplification of the access process and the enhancement of the processing capability of base station and UE, reflecting a faster transition from battery efficiency state to an active communication state. The target value is expected to be 10ms.

Not sure yet the remarkable use case to apply lower value of user plane latency because end-end latency seems more important to immersive cases while the latency only from radio interface may not be the key.

- Connection density

The connection density is valuable and unique to be defined for Massive Communication. Considering the rise of numerous IoT devices and the expansion of use cases, the requirement for connection density may also increases. This KPI is expected to reach 10x of IMT-2020 target.

- Energy efficiency

The same relative energy saving definition applies to both network and device sides. Regarding the target value(s), those for the network and device can be considered separately. Also, it is necessary to consider the impact on energy efficiency from one side tothe other side, e.g., when the target value of network energy efficiency is reached, it is assumed that the impact on the device power consumption should be limited. For example, if the network energy efficiency increases by X%, the corresponding device energy efficiency should reduce no more than Y% (Y<X), and vice versa.

- Positioning (Positioning accuracy)

Positioning was 5G supplementary-service related requirement in 38.913[5]. It is an existing capability supported in each of previous generations and should be quantified in IMT-2030. Requirements to accuracy of positioning performance depend on the use cases, environments including geographic assumption and deployment. In 5G, for autonomous vehicle, positioning accuracy is 0.1m~0.5m (TS 22.261); for AGV positioning in TS 22.104, positioning accuracy is 0.3m~0.5m. In 6G, several use cases have been defined for high-accuracy positioning in TR 22.870, e.g., for immersive gaming and seamless immersive reality, the positioning accuracy is less than 10cm; for UE assisted bi-static sensing requires positioning information of UE. It is noted that in M.2160, 1-10cm as a capability is required.

It is also important to define evaluation environments and channel model together with KPI of positioning accuracy, to both horizontal and vertical plane. The evaluation scenarios for positioning should be aligned with industrial requirements and commercialization applications. In contrast to outdoor environments equipped with global navigation satellite systems, indoor scenarios face significant challenges in obtaining reliable satellite signals and exhibit higher environmental complexity. Consequently, prioritizing indoor scenario evaluations is critical. To address these needs, it is proposed to prioritize the evaluation of a representative indoor scenario, such as Indoor Factory in ISAC or HRLLC, or Indoor Hotspot in IC. The positioning accuracy KPI is expected to be <=10cm.

- Sensing

The usage scenarios for sensing should be carefully selected, taking into account the commercial interest and the evaluation effort. We propose to adopt one indoor scenario and one outdoor scenario in sensing evaluation as maximum for IMT-2030 TPR. For indoor, we think Indoor Factory-SH is a reasonable choice, since sensing AGV (automatic guided vehicle) and human worker in the factory is an attractive use case for vertical industry. Specifically, ‘-SH’ denotes ‘sparse clutter’, which is a simple, typical case for Indoor Factory. For outdoor, considering the popularity of UAV detection and tracking, Urban Macro-AV should be prioritized. This is also aligned with the study progress in 3GPP, which just adopted UMa-AV as the scenario for channel calibration for UAV sensing.

It is widely acknowledged that most of the sensing use cases are built on the basis of detection and tracking. Therefore, we believe detection probability, localization accuracy and velocity accuracy are suitable KPIs for sensing.

Proposal 4. To consider proposed values and mapping between candidate IMT-2030 KPI (without square brackets) and usage scenarios in Table 3.

## Configurations

- Antenna configuration at Network side

With the increasing performance requirements and the continuous upgrading of antenna products, the number of antennas in 6G will increase compared to 5G. For 4GHz, the maximum number of antennas per TRxP is 256. When the frequency is raised to U6GHz, the half wavelength of the antenna is reduced to nearly half of the previous value. That is, with the same antenna area, the number of supported antennas can be increased by four times. Due to the fact that the number of antennas does not improve performance as much as the number of TxRUs and increases simulation time, we propose that the number of antenna elements per TRxP is up to 1024 in 6G evaluation. Such antenna configuration can also be applied to sensing, positioning.

- Channel model and device distribution for positioning

For positioning, in the case that Indoor factory could be applied, its channel model can reuse InF model in 3GPP TR 38.901. It is highly recommended to adopt the positioning channel model and device distribution from 3GPP in ITU, if Indoor factory is applied as a deployment scenario.

- Channel modelling for sensing

Currently there is no channel modelling for sensing in ITU. Meanwhile, 3GPP is now working on the study of channel modelling for sensing, which is expected to be completed in June 2025 [11]. It is highly recommended to adopt the sensing channel model from 3GPP in ITU.

# Proposals

With above, there are proposals from CATT:

Proposal 1. Proposed TPR of AI is qualitative with definition in Table 1.

Proposal 2. Proposed TPRs of Sensing are quantitative with definitions in Table 2.

Proposal 3. Before getting into value discussion of Energy Efficiency, a proper baseline needs to be carefully considered.

Proposal 4. To consider proposed values and mapping between candidate IMT-2030 KPI (without square brackets) and usage scenarios in Table 3.

Proposal 5. It is highly recommended to adopt the sensing channel model when it is ready, positioning channel model and device distribution from 3GPP in ITU, when corresponding new deployment scenario(from ITU perspective) appears.

# References

- RP-233985 Additional considerations for 6G timeline, TSG Chairs

- RP-240823 High-level considerations for 6G timeline	, TSG Chairs

- RP-242128 Views on IMT-2030 KPI, CATT

- RT-240015 Liaison statement to external organizations - Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R Working Parting 5D

- 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 15)

- ITU-R M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond

- ITU-R M.2410 Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- RP-243327 Study on 6G Scenarios and Requirements

- RP-243276 RAN Chair’s Guidance for 6G Study: ITU Focused

- RP-243131 Consideration and proposals on KPIs for IMT-2030, CATT

- RP-242348, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR.