3GPP TSG RAN meeting #107							RP-250574

Incheon, Korea, March 12-14, 2025

Title:	Proposal on improved resilience to GNSS interference

Source: 	ST Engineering iDirect

Type:	discussion

Document for:	discussion

Agenda Item:	15.2.7 Additional RAN1-led topics for 5G-Advanced in Rel-20

Abstract

This proposal emphasizes the need to enhance resilience to GNSS unavailability (e.g., due to spoofing or jamming) for an NTN-capable UE, and provides a possible solution how to do perform time/frequency tracking in connected mode when the UE does not have a valid GNSS position.

# Introduction

This document proposes a simple mechanism to improve the resilience to GNSS interference for satellite access.

In the current specifications, it is mandated that the UE supporting NTN is GNSS-capable (TS 38.300 § 16.14.1), and that the UE shall only transmit when it has a valid GNSS position (TS 36.300 § 23.21.2.2).

Based on its GNSS position and satellite ephemeris data (that is received in SIB19), the UE can pre-compensate time and frequency offsets caused by the satellite movement (TS 38.300 § 16.14.2.2). If the UE does not have a valid GNSS position, it shall transition to the idle state (TS 36.300 § 23.21.2.2).

# Discussion

## Justification

As outlined in the introduction, an NTN-capable UE depends on GNSS for satellite access

at logon, for time/frequency offset pre-compensation based on the UE and satellite positions

in connected mode, for time and frequency tracking

Furthermore, GNSS is also used to get pointing information for the UE as well as for cell handovers in NGSO constellations.

The use of GNSS signals has the following drawbacks

GNSS signals are weak and are prone to spoofing and jamming [1][2]

GNSS acquisition causes longer start-up times and increases the power consumption [3]

Evidence of real GPS interference [4][5]

Live map of GPS interference https://gpsjam.org/

Figure 1. Screenshot of gpsjam.org that visualizes GPS interference on February 17, 2025 
(while we were greeting in Athens for the WG meeting)

Apart from above-mentioned current issues with GNSS, we believe that a 3GPP system should not depend on any non-3GPP system. The ultimate target is full GNSS independence.

## Objectives

As a first step towards GNSS independence, the objective is to improve resilience to intermittent GNSS unavailability, i.e., keep the UE connected to the gNB even when it is not able to acquire valid GNSS signals. For this, the objective is to remove dependence on GNSS for time/frequency tracking in connected mode.

Hypothesis: the NTN-capable UE has a GNSS receiver and is able to connect to the gNB, but the GNSS system is not available while the UE is in connected mode.

Note: these objectives are in line with contributions by other companies in the past [3][6][7] and with the discussion during RAN 101 in Bengalaru [8].

## Proposal

Timing advance is already communicated via the Timing Advance Command MAC control element (CE), e.g., in connected mode or via Msg2 during the RACH procedure.

A possible solution to improve GNSS resilience with minimal impact to the current specifications is to introduce a similar MAC CE to communicate frequency offset estimates from the gNB towards the UE [9]. The procedure could be as follows.

Perform closed-loop tracking of carrier frequency and symbol time offsets at the gNB

- The gNB communicates the estimated time and frequency offset to the UE

This requires the addition of one new MAC CE to provide frequency offset corrections to the UE, similar to existing TA corrections. Alternatively, two new MAC CEs could be introduced to communicate the time and frequency errors that can be used to calculate corrections at the UE.

Requested time units for the RAN working groups

- RAN1: 0.5 TU per meeting

- RAN2: 0.25 TU per meeting

- RAN4 RRM: 0.5 TU per meeting

- RAN4 RF: 0.25 TU per meeting

# Conclusion

In this discussion document, we propose to enhance resilience to GNSS interference, spoofing, and jamming. A possible solution is to add a MAC-layer message to communicate frequency offset estimates by the gNB towards the UE in a fast-tracking loop. This allows keeping NTN-capable UEs connected to the satellite, even when the UE does not have a valid GNSS position.

This can be a first step towards realizing full GNSS independence, e.g., by combining the proposed approach with a non-GNSS-based positioning system (such as 6G PNT). Furthermore, for a stationary UE for which the position can be pre-configured, the presented solution can be sufficient for GNSS independence.

# References

[1] M. L. Psiaki and T. E. Humphreys, “GNSS spoofing and detection,” Proceedings of the IEEE, vol. 104, no. 6, pp. 1258-1270, June 2016

[2] Matthias Schäfer, "Use of ADS-B to detect GNSS Interference," ICAO EUR/MID Radio Navigation Symposium, 2024

[3] RWS-230048, Consideration on RAN1/2/3 led NTN topics for Release 19, Section 2.3.1.1.1 “Justification GNSS Independent UE Operation”, Taipei, June 15-16, 2023 [noted]

[4] https://www.forbes.com/sites/erictegler/2023/12/05/gps-spoofing-in-the-middle-east-is-now-capturing-avionics/

[5] https://ing.dk/artikel/danish-gps-expert-unusually-strong-gps-jamming-over-baltic-sea

[6] RWS-210221 Sharp, Support of Non GNSS capable devices in NTN systems (Rel-18), June-July, 2021 [noted]

[7] RP-233296 EchoStar, Priorities for Rel-19 NTN-NR Evolution, RAN #102, December 11-15, 2023 [noted]

[8] RP-262310, Moderator's summary for REL-19 RAN2 topic NTN for NR, Section 2.5 “Enhanced GNSS Operation”, Bengalaru, September 11-15, 2023

[9] P. Delbeke and D. Duyck, “5G-NR GNSS independent time and frequency synchronization in NTN scenarios”, 28th Ka and Broadband Communications Conference, October 2023

END