3GPP TSG-RAN4 Meeting # 114	R4-2500924

# Athens, GR, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5278 | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_UE_pow_sav-Core) CR on R16 measurement relaxation for low mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R16, the measurement relaxation for IDLE/INACTIVE was introduced when the low mobility and/or not-at-cell edge criterion meet. However some confusion exists when capturing these features. 1st change: The logic between ‘and’ and ‘or’ is confused. 2nd change: The condition regarding the T331 in inter-frequency case is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1st change: Refine the ‘and’, ‘or’ logic. 2nd change: To add the condition regarding the T331 in inter-frequency to align with inter-RAT EUTRAN case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion exists for the measurement relax criterion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.10.2, 4.2.2.10.3, 4.2.2.11.2, 4.2.2.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

##### 4.2.2.10.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and

UE has fulfilled only the lowMobilityEvaluation [2] criterion.

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements are defined as follows:

-	Tdetect,NR_Inter_Relax as specified in Table 4.2.2.10.2-1.

-	Tmeasure,NR_Inter_Relax as specified in Table 4.2.2.10.2-1.

-	Tevaluate,NR_Inter_Relax as specified in Table 4.2.2.10.2-1.

-	The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax  * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax  * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax *Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax  * Tevaluate,NR_Inter.

-	When T331 is running,

-	The parameter Ncarrier_Relax is the total number of NR inter-frequency carriers not configured for idle mode CA/DC measurements.

-	The parameter Ncarrier_Non_relax is the total number of NR inter-frequency carriers configured for idle mode CA/DC measurements.

-	When T331 is not running,

-	The parameter Ncarrier_Relax is the total number of inter-frequency carriers configured for mobility measurements only and the number of inter-frequency carriers configured for both mobility measurement and idle mode CA/DC measurements.

-	The parameter Ncarrier_Non_relax =0.

When UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

-	if the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60

-	otherwise, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.10.2-1: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 | cycles) |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

##### 4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is only configured with cellEdgeEvaluation [2] criterion, and UE has fulfilled the cellEdgeEvaluation [2] criterion or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and

UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter_Relax as specified in Table 4.2.2.10.3-1.

-	Tmeasure,NR_Inter_Relax as specified in Table 4.2.2.10.3-1.

-	Tevaluate,NR_Inter_Relax as specified in Table 4.2.2.10.3-1.

-    The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax  * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax  * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax * Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax  * Tevaluate,NR_Inter.

-    When T331 is running,

-     The parameter Ncarrier_Relax is the total number of NR inter-frequency carriers not configured for idle mode CA/DC measurements.

-    The parameter Ncarrier_Non_relax is the total number of NR inter-frequency carriers configured for idle mode CA/DC measurements.

-    When T331 is not running,

-      The parameter Ncarrier_Relax is the total number of inter-frequency carriers configured for mobility measurements only and the number of inter-frequency carriers configured for both mobility measurement and idle mode CA/DC measurements.

-     The parameter Ncarrier_Non_relax =0.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7

Table 4.2.2.10.3-1: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

# <End of Change #1>

# <Start of Change #2>

##### 4.2.2.11.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and UE has fulfilled only the lowMobilityEvaluation [2] criterion.

The UE shall not relax measurements on inter-RAT E-UTRAN carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.5 apply for this clause except that:

-	Tdetect,EUTRAN_Relax as specified in Table 4.2.2.11.2-1.

-	Tmeasure,EUTRAN_Relax as specified in Table 4.2.2.11.2-1.

-	Tevaluate,EUTRAN_Relax as specified in Table 4.2.2.11.2-1.

-	The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [1] within NEUTRAN carrier_Relax * Tdetect,EUTRAN_Relax + NEUTRAN carrier_Non_relax  * Tdetect,EUTRAN. Cells which have been detected shall be measured at least every NEUTRAN carrier_Relax * Tmeasure,EUTRAN_Relax + NEUTRAN carrier_Non_relax  * Tmeasure,EUTRAN. The UE shall be able to evaluate that an already identified inter-RAT E-UTRAN cell has met reselection criterion defined in TS 38.304 [1] within NEUTRAN carrier_Relax * Tevaluate,EUTRAN_Relax + NEUTRAN carrier_Non_relax  * Tevaluate,EUTRAN.

-	When T331 is running,

-	The parameter NEUTRAN carrier_Relax is the total number of inter-RAT E-UTRAN carriers not configured for idle mode CA/DC measurements.

-	The parameter NEUTRAN carrier_Non_relax is the total number of inter-RAT E-UTRAN carriers configured for idle mode CA/DC measurements.

-	When T331 is not running,

-	The parameter NEUTRAN carrier_Relax is the total number of inter-RAT E-UTRAN carriers configured for mobility measurements only and the number of inter-RAT E-UTRAN carriers configured for both mobility measurement and idle mode CA/DC measurements.

-	The parameter NEUTRAN carrier_Non_relax =0.

When UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

-	if the UE is configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60

-	otherwise, the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.2-1: Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax, and Tevaluate,EUTRAN_Relax

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion. |  |  |  |

##### 4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is only configured with cellEdgeEvaluation [2] criterion and UE has fulfilled the cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and

UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The UE shall not relax measurements on inter-RAT E-UTRAN carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.5 apply for this clause except that:

-	Tdetect,EUTRAN_Relax as specified in Table 4.2.2.11.3-1.

-	Tmeasure,EUTRAN_Relax as specified in Table 4.2.2.11.3-1.

-	Tevaluate,EUTRAN_Relax as specified in Table 4.2.2.11.3-1.

-	The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [1] within NEUTRAN carrier_Relax * Tdetect,EUTRAN_Relax + NEUTRAN carrier_Non_relax  * Tdetect,EUTRAN. Cells which have been detected shall be measured at least every NEUTRAN carrier_Relax * Tmeasure,EUTRAN_Relax + NEUTRAN carrier_Non_relax  * Tmeasure,EUTRAN. The UE shall be able to evaluate that an already identified inter-RAT E-UTRAN cell has met reselection criterion defined in TS 38.304 [1] within NEUTRAN carrier_Relax * Tevaluate,EUTRAN_Relax + NEUTRAN carrier_Non_relax  * Tevaluate,EUTRAN.

-	When T331 is running,

-	The parameter NEUTRAN carrier_Relax is the total number of inter-RAT E-UTRAN carriers not configured for idle mode CA/DC measurements.

-	The parameter NEUTRAN carrier_Non_relax is the total number of inter-RAT E-UTRAN carriers configured for idle mode CA/DC measurements.

-	When T331 is not running,

-	he parameter NEUTRAN carrier_Relax is the total number of inter-RAT E-UTRAN carriers configured for mobility measurements only and the number of inter-RAT E-UTRAN carriers configured for both mobility measurement and idle mode CA/DC measurements.

-	The parameter NEUTRAN carrier_Non_relax =0.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-RAT E-UTRAN frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.3-1: Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax, and Tevaluate,EUTRAN_Relax

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |

# <End of Change #2>