3GPP TSG-RAN5 Meeting #106	R5-251281

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4803 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM TC 9.2.5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_Ph1-CT_n3GPPA-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Step 47 shall not check the same presence as in step 37A since UE is powered off and on in steps 38-39, the rejected NSSAI for the current PLMN and registration area combination shall be deleted after switching off UE as indicated in TP4. In Tables 9.2.5.1.2.3.3-11/12, the encoding of ST is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set 47 was updated to check removal of the rejected NSSAI for the current PLMN and registration area combination. ST values were updated in Tables 9.2.5.1.2.3.3-11/12. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect configuration is used in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-250088 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-250088 -> R5-251281 |  |  |  |  |  |  |  |  |

[Start of change]

9.2.5.1.2	Initial registration / 5GS services / NSSAI handling

9.2.5.1.2.1	Test Purpose (TP)

(1)

with { UE has sent a REGISTRATION REQUEST message including requested NSSAI }

ensure that {

when { UE receives REGISTRATION ACCEPT message with allowed NSSAI }

then { UE shall replace any stored allowed NSSAI for the current PLMN with new allowed NSSAI for the current PLMN }

}

(2)

with { UE has sent a REGISTRATION REQUEST message including Requested NSSAI }

ensure that {

when { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject cause “S-NSSAI not available in the current PLMN” }

then { UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and not attempt to use the Rejected NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed }

}

(3)

with { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject cause “S-NSSAI not available in the current PLMN” }

ensure that {

when { UE has been switched off, then switched on }

then { UE shall delete the stored Rejected NSAAI and shall send the NSSAI in Requested NSSAI IE of the REGISTRATION REQUEST message as per the configured and Allowed NSSAI for current PLMN }

}

(4)

with { UE has sent a REGISTRATION REQUEST message including Requested NSSAI }

ensure that {

when { UE receives REGISTRATION ACCEPT message with Rejected NSSAI with reject cause “S-NSSAI not available in the current registration area” }

then { UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination and not attempt to use the Rejected NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed }

}

9.2.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.5.1.2.2 and 5.5.1.2.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501 clause 5.5.1.2.2]

5.5.1.2.1	General

This procedure can be used by a UE for initial registration for 5GS services.

…

The UE shall include the requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wants to register and shall include the mapping of the requested NSSAI which is the mapping of each S-NSSAI of the requested NSSAI to the S-NSSAI(s) of the HPLMN, if available, in the REGISTRATION REQUEST message. If the UE has allowed NSSAI or configured NSSAI for the current PLMN, the requested NSSAI shall be either:

a)	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN;

b)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

c)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and those are neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

If the UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a default configured NSSAI, the UE shall:

a)	include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the default configured NSSAI; and

b)	include the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" in the REGISTRATION REQUEST message.

If the UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no default configured NSSAI, the UE shall not include a requested NSSAI in the REGISTRATION message.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN, if the S-NSSAI is neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for the current PLMN.

NOTE 3:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 4:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

[TS 24.501 clause 5.5.1.2.4]

The AMF shall include the allowed NSSAI for the current PLMN and shall include the mapping of each S-NSSAI of the allowed NSSAI to the S-NSSAI(s) of the HPLMN contained in the requested NSSAI from the UE if available, in the REGISTRATION ACCEPT message if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the requested NSSAI but rejected by the network associated with rejection cause(s).

The AMF may include a new configured NSSAI for the current PLMN in the REGISTRATION ACCEPT message if:

a)	the REGISTRATION REQUEST message did not include the requested NSSAI;

b)	the REGISTRATION REQUEST message included the requested NSSAI containing an S-NSSAI that is not valid in the serving PLMN; or

c)	the REGISTRATION REQUEST message included the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI".

If a new configured NSSAI for the current PLMN is included in the REGISTRATION ACCEPT message, the AMF shall also include the mapping of the configured NSSAI for the current PLMN the S-NSSAI(s) of the HPLMN if available in the REGISTRATION ACCEPT message. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The AMF shall include the Network slicing indication IE with the Network slicing subscription change indication set to "Network slicing subscription changed" in the REGISTRATION ACCEPT message if the UDM has indicated that the subscription data for network slicing has changed. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in the current PLMN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message or none of the requested NSSAI are present in the subscribed S-NSSAIs, and one or more subscribed S-NSSAIs (containing one or more S-NSSAIs each of which may be associated with a new S-NSSAI) marked as default are available, the AMF shall put the subscribed S-NSSAIs marked as default in the allowed NSSAI of the REGISTRATION ACCEPT message. The AMF shall determine a registration area such that all S-NSSAIs of the allowed NSSAI are available in the registration area.

9.2.5.1.2.3	Test description

9.2.5.1.2.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27

UE:

-	UE is previously registered on WLAN Cell 27 using default message contents according to TS 38.508-1 [4];

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0W-B) according to TS 38.508-1 [4].

9.2.5.1.2.3.2	Test procedure sequence

Table 9.2.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 2 to 6 as per the IKEv2 protocol as defined in 3GPP TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 2 | Check: Does UE transmit a REGISTRATION REQUEST message? | --> | REGISTRATION REQUEST | - | - |
| 3-6 | Steps 4 to 7 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 7 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Configured NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 8 | Step 10 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 9 | Switch off procedure in WLAN Ipsec_SA_Established specified in TS 38.508-1 subclause 4.9.6.5 is performed. | - | - | - | - |
| 10 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 11 to 15 as per the IKEv2 protocol as defined in 3GPP TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 11 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 1 | P |
| 12-15 | Steps 4 to 7 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 16 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Rejected NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 17 | Step 10 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 18 | Switch off procedure in WLAN Ipsec_SA_Established specified in TS 38.508-1 subclause 4.9.6.5 is performed. | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 19 to 23 as per the IKEv2 protocol as defined in 3GPP TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 19 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 2 | P |
| 20-23 | Steps 4 to 7 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 24 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 25 | The UE transmits a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 26 | Step 10 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 27 | Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause “S-NSSAI not available in the current PLMN” associated with current PLMN using AT/MMI? | - | - | 2 | P |
| 28 | Switch off procedure in WLAN Ipsec_SA_Established specified in TS 38.508-1 subclause 4.9.6.5 is performed. | - | - | - | - |
| 29 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 30 to 34 as per the IKEv2 protocol as defined in 3GPP TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 30 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 3 | P |
| 31-34 | Steps 4 to 7 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 35 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Rejected NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 36 | Step 10 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 37 | Check: Is S-NSSAI=2 removed from the Rejected NSSAI list associated with current PLMN? | - | - | 3 | P |
| 37A | Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause “S-NSSAI not available in the current registration area” associated with current PLMN and registration area combination using AT/MMI? | - | - | 4 | P |
| 38 | Switch off procedure in WLAN Ipsec_SA_Established specified in TS 38.508-1 subclause 4.9.6.5 is performed. | - | - | - | - |
| 39 | The UE is brought back to operation or the USIM is inserted | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 40 to 44 as per the IKEv2 protocol as defined in 3GPP TS 23.502 [33] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 40 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 4 | P |
| 41-44 | Steps 4 to 7 of the generic procedure for WLAN Ipsec_SA_Released specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 45 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 46 | The UE transmits a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 47 | Check: Is S-NSSAI=1 removed from the Rejected NSSAI list with cause “S-NSSAI not available in the current registration area” associated with current PLMN and registration area combination using AT/MMI? | - | - | 4 | P |

9.2.5.1.2.3.3	Specific message contents

Table 9.2.5.1.2.3.3-1: REGISTRATION REQUEST (step 2, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |
| Requested NSSAI |  | Note |  |
| S-NSSAI IEI |  | S-NSSAI value 1 | Note |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Note: S-NSSAI =1 will be always included from the allowed NSSAI list associated with PLMN of WLAN Cell 27 by the UE but may include other S-NSSAI from Configured NSSAI list associated with PLMN of NCG WLAN Cell 27 if configured in the UE. See TS 24.501 sub-clause 5.5.1.2.1 |  |  |  |

Table 9.2.5.1.2.3.3-2: REGISTRATION ACCEPT (step 7, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration result value | ‘010’B | Non 3GPP access |  |
| Allowed NSSAI |  |  |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Configured NSSAI |  |  |  |
| S-NSSAI IEI |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |

Table 9.2.5.1.2.3.3-3: PDU SESSION ESTABLISHMENT ACCEPT (step 8, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |
| SST | ‘0000 0010’B | SST value 2 |  |

Table 9.2.5.1.2.3.3-4: REGISTRATION REQUEST (step 11, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |
| Requested NSSAI |  | Note |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| S-NSSAI IEI |  | S-NSSAI value 1 | Note |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Note: S-NSSAI =2 will be always included by the UE from the allowed NSSAI list associated with PLMN of NCG WLAN Cell 27 but may include S-NSSAI =1 from Configured NSSAI list associated with PLMN of NCG WLAN Cell 27. See TS 24.501 sub-clause 5.5.1.2.1 |  |  |  |

Table 9.2.5.1.2.3.3-5: REGISTRATION ACCEPT (step 16, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration result value | ‘010’B | Non 3GPP access |  |
| Allowed NSSAI |  | Note |  |
| S-NSSAI IEI |  | S-NSSAI value 1 | Note |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Rejected NSSAI |  |  |  |
| Rejected S-NSSAI-1 |  | Rejected S-NSSAI value 1 |  |
| Cause value | ‘0000’B | S-NSSAI not available in the current PLMN |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Note: If UE has requested only S-NSSAI =2 in step 11 and S-NSSAI =2 is added in the Rejected NSSAI list by the SS then AMF/SS can include default subscribed S-NSSAIs in the allowed NSSAI of REGISTRATION ACCEPT message, see TS 24.501 sub-clause 5.5.1.2.4. |  |  |  |

Table 9.2.5.1.2.3.3-6: REGISTRATION REQUEST (step 19, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  |  |
| S-NSSAI IEI |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |

Table 9.2.5.1.2.3.3-7: REGISTRATION ACCEPT (step 24, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration result value | ‘010’B | Non 3GPP access |  |
| Allowed NSSAI |  |  |  |
| S-NSSAI IEI |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |

Table 9.2.5.1.2.3.3-8: REGISTRATION REQUEST (step 30, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |
| Requested NSSAI |  | Note |  |
| S-NSSAI IEI |  | S-NSSAI value 1 | Note |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Note: S-NSSAI =1 will be always included by the UE from the allowed NSSAI list associated with PLMN of NCG Cell C but may include S-NSSAI =2 from Configured NSSAI list associated with PLMN of NCG Cell C. See TS 24.501 sub-clause 5.5.1.2.1 |  |  |  |

Table 9.2.5.1.2.3.3-9: REGISTRATION ACCEPT (step 35, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration result value | ‘010’B | Non 3GPP access |  |
| TAI list |  |  |  |
| Type of list | ‘01’B | list of TACs belonging to one PLMN, with consecutive TAC values |  |
| Number of elements | ‘00001’B | 2 Elements |  |
| TAC | PLMN =MCC/MNC stored in EFIMSI TAC 1 = 2 | TAI2, TAI 3 |  |
| Allowed NSSAI |  |  |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Rejected NSSAI |  |  |  |
| Rejected S-NSSAI-1 |  | Rejected S-NSSAI value 1 |  |
| Cause value | ‘0001’B | S-NSSAI not available in the current registration area |  |
| SST | ‘00000001’B | 1 |  |
| SD | Not Present |  |  |
| Note: If UE has requested only S-NSSAI =1 in step 30 and S-NSSAI =1 is added in the Rejected NSSAI list by the SS then AMF/SS can include default subscribed S-NSSAIs in the allowed NSSAI of REGISTRATION ACCEPT message, see TS 24.501 sub-clause 5.5.1.2.4. |  |  |  |

Table 9.2.5.1.2.3.3-10: PDU SESSION ESTABLISHMENT ACCEPT (step 36, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |
| SST | ‘0000 0010’B | SST value 2 |  |

Table 9.2.5.1.2.3.3-11: REGISTRATION REQUEST (step 40, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | initial registration |  |
| Requested NSSAI |  | Note |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |

Table 9.2.5.1.2.3.3-12: REGISTRATION ACCEPT (step 45, Table 9.2.5.1.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration result value | ‘010’B | Non 3GPP access |  |
| Allowed NSSAI |  |  |  |
| S-NSSAI IEI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000010’B | SST |  |
| SST | ‘00000010’B | 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |

[End of change]