3GPP TSG-RAN Meeting #108		RP-250900

Prague, Czech Republic, 9th – 13th June 2025

Title: 	5G-A Rel-20 UE RF and spectrum related topics

Agenda Item:	15.4.2

Source: 	Nokia

Document for:	Discussion

# 1				Introduction

In this contribution we express our views for Rel-20 5G-A UE RF non-spectrum and spectrum topics.

# 2	Discussion

## 2.1	Non-spectrum UE RF

### 2.1.1	HPUE

RAN4 5G-A package should include only topics that are immediately implementable and beneficial for the ecosystem. One quite obvious way to do this would be specifying high power classes for using existing power amplifier designs, i.e. PC2 and PC3 for FWA. Combining existing PAs brings the additional benefit of making UL-MIMO inherently implementable which would significantly increase the data rate of FWA links which are mainly data pipes. In order to maximize the benefits of this work, UL-MIMO could be considered mandatory for 4 Tx “PC1bis” FWA devices (where PC1bis corresponds to devices with 4 x 26dBm = 32dBm)

We recommend that HPUE should be the only objective for UE RF work in Rel-20, in order to keep the 5G-A package size reasonable.

Below is our proposal for the detailed HPUE objectives:

- High power PC1.5/PC1 UE for TN single carrier in NR FR1

  - PC1bis* with 4Tx with 4-layer UL-MIMO for FWA.

  - PC1bis* with 8Tx with 8-layer UL-MIMO for FWA

    - Redo coexistence studies for FWA UE to see if ACLR can be relaxed.

  - PC1.5 FDD with 2Tx handheld and FWA.

*) 32 dBm

Observation 1: HPUE should be the only objective for UE RF work in Rel-20, in order to keep the 5G-A package size reasonable.

### 2.1.2		Power enhancement with relaxed UE Tx EVM and advanced BS receiver

From the BS receiver perspective, it is questionable what gains could be achieved in combination with power enhancements, especially in combination with a relaxed UE Tx EVM. The effective receive SNR will be dominated by the new Tx EVM, i.e., the achievable effective receive SNR will be reduced and maximum TPUT/MCS/performance along with it. There is no signal processing solution to regenerating a randomly reduced signal quality at the transmission point.

It may be thinkable to compensate non-linearities in the transmitter via non-linear signal processing, pre/post-distortion, etc. However, the relaxed Tx EVM and its random effects on receive signal quality make such efforts much less likely to bring any gains and would require a comprehensive study to confirm otherwise, which is infeasible within the available time for 5G-A in Rel-20.

Observation 2: The study of unconfirmed gains from newly proposed theoretic non-linear post equalization receiver implementations is not feasible in the available time for 5G-A in Rel-20.

### 2.1.3		MSD optimization

We see MSD optimization very beneficial; it has been an integral part of RAN4 HPUE MSD simplification discussions in Rel19. What is still open, assuming that current RAN4 agreements for Rel-19 hold, is some HPUE UL CA cases and default power class MSD automation. However, it is our understanding that at least the default power class automation is a big effort and there are more productive ways of using the available TUs. This topic needs to be addressed in 6G and can be part of the 6G SI.

Observation 3: MSD optimization for default power class can also be considered as part of 6G.

### 2.1.4	UL Tx switching enhancement

UL Tx switching was first introduced in 3GPP Rel-16 as a new feature to enhance inter-band EN-DC, inter-band UL CA, and SUL operations. Since then, multiple enhancements have been proposed and agreed with the latest being 2-band 3Tx UL switching. Even there exist multiple specified possibilities for UL Tx switching there have based on our observations not been widely adaptation and use of these features.

Observation 4: UL Tx switching is already supported by the current specification.

Both the current supported switching cases and the observed proposals all come with additional complications to the schedular which needs to take into account switching gabs and/or plan for transitions in UL. This may be an explanation for the reluctance of widely using the feature.

Given the fact that we are in the end of this generation we believe that further enhancing this feature does not provide sufficient gain to justify a Rel-20 WI. This said, considering introducing more freedom for the UL scheduling can be studied for the next generation.

Observation 5: Further UL Tx switching enhancements should not be considered in Rel-20.

## 2.2	Spectrum Basket WIs

Rel-20 basket WIs should be power class agnostic. Rel-19 HPUE baskets should be merged into the Rel-20 proposed “normal” baskets as shown in the table below. All band-related feature baskets are merged into NR_bands_xFeature_20. New operating bands can be considered based on operator requests.

| Rel-19 Baskets: | Rel-20 Basket(s) |
| --- | --- |
| DC_R19_xBLTE_yBNR | HPUE objective is added into DC_R20_xBLTE_yBNR |
| HPUE_DC_LTE_NR_R19 |  |
| NR_CADC_SUL_R19 | HPUE objective is added into NR_CADC_SUL_R20 |
| HPUE_NR_CADC_SUL_R19 |  |
| LTE_CA_R19_xBDL_yBUL | LTE_CA_R20_xBDL_yBUL |
| HPUE_NR_FR1_bands_R19 | HPUE_NR_FR1_bands_R20 |
| HPUE_LTE_bands_R19 | Not needed in R20 |
| NR_bands_CA_ENDC_R19_BWs | NR_bands_CA_ENDC_R20_BWs |
| NR_bands_xFeature_R19 | All band related feature baskets are merged into NR_bands_xFeature_20 |
| DL_intrpt_combos_TxSW_R19 |  |
| LTE_NR_R19_Simult_RxTx |  |

Table 1: Rel-20 Baskets

Observation 6: Rel-20 basket WIs should be power class agnostic.

Observation 7: All band-related feature baskets are merged into one WI.

# 3	Conclusion

We propose that the UE RF work in Rel-20 should focus only on HPUE, with the following objectives:

- High power PC1.5/PC1 UE for TN single carrier in NR FR1

  - PC1bis* with 4Tx with 4-layer UL-MIMO for FWA.

  - PC1bis* with 8Tx with 8-layer UL-MIMO for FWA

    - Redo coexistence studies for FWA UE to see if ACLR can be relaxed.

  - PC1.5 FDD with 2Tx handheld and FWA.

- *) 32 dBm

Rel-20 basket WIs should be power class agnostic and all band-related feature baskets should be merged into one WI. The proposed relation between Rel-19 and Rel-20 baskets is shown in Table 1.