3GPP TSG-RAN5 Meeting #107	R5-251955

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1511 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add default content for RRCEarlyData NB-IoT messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The RRC messages RRCEarlyDataRequest-NB and RRCEarlyDataComplete-NB are used by several test cases in TS 36.523-1, however they have no default message content defined in TS 36.508. This is against RAN5 PRD13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Default content has been defined for the RRC messages RRCEarlyDataRequest-NB and RRCEarlyDataComplete-NB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of specification will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.1.6	NB-IoT Default RRC message and information elements contents

This clause contains the default values of common RRC messages and information elements, which unless indicated otherwise in specific clauses of TS 36.521-1 [21], TS 36.521-3 [34], TS 36.523-1 [18] and other clauses in this specification. All the messages and information elements are listed in alphabetical order.

#### 8.1.6.1	NB-IoT Contents of RRC messages

#### <TEXT SKIPPED>

#### –	RRCConnectionSetupComplete-NB

Table 8.1.6.1-15: RRCConnectionSetupComplete-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 |  |  |
| s-TMSI-r13 | Not checked |  |  |
| registeredMME | Not checked |  |  |
| dedicatedInfoNAS | Present but contents not checked |  |  |
| attachWithoutPDN-Connectivity-r13 | Not checked |  |  |
| up-CIoT-EPS-Optimisation-r13 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCEarlyDataComplete-NB

Table 8.1.6.1-15aa: RRCEarlyDataComplete-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCEarlyDataComplete-NB-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcEarlyDataComplete-r15 SEQUENCE { |  |  |  |
| dedicatedInfoNAS-r15 | Not checked |  |  |
| extendedWaitTime-r15 | Not checked |  |  |
| redirectedCarrierInfo-r15 | Not checked |  |  |
| redirectedCarrierInfoExt-r15 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCEarlyDataRequest-NB

Table 8.1.6.1-15ab: RRCEarlyDataRequest-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCEarlyDataRequest-NB-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcEarlyDataRequest-r15 SEQUENCE { |  |  |  |
| s-TMSI | Any allowed value |  |  |
| establishmentCause-r15 | Present but contents not checked |  |  |
| cqi-NPDCCH-r15 | Not checked |  |  |
| dedicatedInfoNAS-r15 | Present but contents not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SCPTMConfiguration-NB

Table 8.1.6.1-15a: SCPTMConfiguration-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCPTMConfiguration-NB-r14 ::= SEQUENCE { |  |  |  |
| sc-mtch-InfoList-r14 SEQUENCE (SIZE (0.. maxSC-MTCH-NB-r14)) OF SEQUENCE { | 1 entry |  |  |
| sc-mtch-CarrierConfig-r14 CHOICE { |  |  |  |
| dl-CarrierIndex-r14 | 0 | INTEGER (0.. maxNonAnchorCarriers-NB-r14) |  |
| } |  |  |  |
| mbmsSessionInfo-r14 SEQUENCE { |  |  |  |
| tmgi-r13 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE { |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000001’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| sessionId-r13 | Not present |  |  |
| } |  |  |  |
| g-RNTI-r14 | ‘0100’H |  |  |
| sc-mtch-SchedulingInfo-r14 | Not present |  |  |
| sc-mtch-neighbourCell-r14 | Not present |  |  |
| npdcch-NPDSCH-MaxTBS-SC-MTCH-r14 | n680 |  |  |
| npdcch-NumRepetitions-SC-MTCH-r14 | r16 |  |  |
| npdcch-StartSF-SC-MTCH-r14 | v4 |  |  |
| npdcch-Offset-SC-MTCH-r14 | zero |  |  |
| } |  |  |  |
| scptm-NeighbourCellList-r14 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |