3GPP TSG-RAN5 Meeting #107	R5-253042

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3517 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the test frequencies for NR FR1 bands configuring a BWP without CORESET0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test,NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For band n38/n39/n40/n48/n50/n41/n77/n78/n79 in Table 6.2.3.1-8, the calculation is based on SCS 30Khz, SCS column is 30Khz, but they wrongly refer to clause 4.3.1.1.1.X 15Khz.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct SCS of the referred table in band n38/n39/n40/n48/n50/n41/n77/n78/n79 to 30Khz in Table 6.2.3.1-8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | n38/n39/n40/n48/n50/n41/n77/n78/n79 in Table 6.2.3.1-8 refer to wrong table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 6.2.3.1-8: Test frequencies for NR FR1 bands configuring a BWP without CORESET#0

| NR Band | SCS [kHz] Note2 | CBW [MHz] Note3 | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | point A [MHz] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] Note 4 | absoluteFrequencySSB [ARFCN] Note 5 |  | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 | absoluteFrequencySSB for NCD-SSB [ARFCN] | Rbstart of BWP | LRB of BWP | locationAndBandwidth of BWP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.5 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 177312 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.5 for bandwidth=20 MHz and SCS=15 kHz |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n8 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.8 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 188512 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.8 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n12 | 15 | 15 | 79 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.12 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 148362 | 54 | 25 | 6654 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.12 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 25 | 6654 |
| n13 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.13 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 151668 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.13 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n14 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.14 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 153068 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.14 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n20 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.20 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 161712 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.20 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n24 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.24 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 306668 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.24 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n26 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.26 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 175312 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.26 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n29 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n30 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.30 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 471468 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.30 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n70 | 15 | 15 | 79 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.70 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 401562 | 54 | 25 | 6654 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.70 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 25 | 6654 |
| n71 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.71 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 126912 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.71 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n76 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n91 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n92 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.92 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 289912 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.92 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n93 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n100 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n1 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.1 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 431500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.1 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n2 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.2 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 393456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.2 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n3 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.3 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 370500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.3 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n7 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.7 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 533500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.7 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n25 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.25 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 393456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.25 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n28 | 15 | 30 | 160 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.28 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 157120 | 108 | 52 | 14133 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.28 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 108 | 52 | 14133 |
| n65 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.65 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 425548 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.65 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n66 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.66 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 429456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.66 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n74 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.74 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 298548 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.74 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n75 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n94 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.94 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 289948 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.94 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n109 | 15 | 30 | 160 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.109 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 288050 | 108 | 52 | 14133 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.109 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 108 | 52 | 14133 |
| n34 | 15 | 15 | 79 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.34 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 404562 | 54 | 25 | 6654 |
| n51 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n53 | 15 | 10 | 52 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.53 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 498168 | 24 | 25 | 6624 |
| n54 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n101 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n38 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.38 for bandwidth=40 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 520952 | 80 | 24 | 6405 |
| n39 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.39 for bandwidth=40 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 382952 | 80 | 24 | 6405 |
| n40 | 30 | 80 | 217 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.40 for bandwidth=80 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 475020 | 192 | 24 | 6517 |
| n48 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.48 for bandwidth=40 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 638984 | 80 | 24 | 6405 |
| n50 | 30 | 60 | 162 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.50 for bandwidth=60 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 297512 | 138 | 24 | 6463 |
| n41 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.41 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 518310 | 222 | 51 | 13972 |
| n77 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.77 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 626370 | 222 | 51 | 13972 |
| n78 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30kHz. |  |  |  |  |  |  |  | 626370 | 222 | 51 | 13972 |
| n79 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.79 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  | 699704 | 222 | 51 | 13972 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Subcarrier spacing is configured according to Table 6.2.3.1-1 to 6.2.3.1-5. Note 3: Channel bandwidth is configured according to highest mandatory UL and DL channel bandwidth in clause 4.3.1. Note 4: SS block SCS for both CD-SSB and NCD-SSB. Note 5: AbsoluteFrequencySSB for CD-SSB Note 6: n29,n75,n76 are SDL band Note 7: The maximum carrierBandwidth for n91,n93,n100,n51 and n101 with signalling test SCS in 6.2.3.1-1 to 6.2.3.1-5 are no more than 25RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>