3GPP TSG-RAN5 Meeting #107	R5-251893

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1146 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 MU - PC7 MU and TT update for ACS and IBB tests in 38.521-2 Annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Certain progress has been made for PC7 MU and TT analysis in discussion R5-251890. Impacted test cases should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated MU in ACS and IBB tests for PC7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete for PC7. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 Max Device size ≤ 30 cm ±5.36 dB (FR2a, FR2b, NTC testing) ±6.34 dB (FR2c NTC testing) ±5.61 dB (FR2a, FR2b, ETC testing) ±6.48 (FR2c ETC testing) PC1 Max Device size ≤ 30 cm ±5.58 dB (FR2a, FR2b, NTC testing) ±5.83 dB (FR2a, FR2b, ETC testing) PC5 Max Device size ≤ 30 cm ±5.58 dB (FR2a, NTC testing) ±5.83 dB (FR2a, ETC testing) PC6 Max Device size ≤ 30 cm ±5.56 dB (FR2a, NTC testing) ±5.81 dB (FR2a, ETC testing) PC7 Max Device size ≤ 30 cm ±5.36 dB (FR2a, NTC testing) ±5.61 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.19-1 in TR 38.903) |
| 7.3.4 EIS spherical coverage | PC3 ±5.07 dB (Max Device size ≤ 30 cm, FR2a, FR2b) ±6.04 dB (Max Device size ≤ 30 cm, FR2c) PC1 ±5.07 dB (Max Device size ≤ 30 cm, FR2a, FR2b) PC5 ±5.07 dB (Max Device size ≤ 30 cm, FR2a) PC6 FFS PC7 Max Device size ≤ 30 cm ±5.07 dB (Max Device size ≤ 30 cm, FR2a) | MTSU = 1.00 x MU (from Table B.19-2 in TR 38.903) |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.4 Reference sensitivity power level for CA (5DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.5 Reference sensitivity power level for CA (6DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.6 Reference sensitivity power level for CA (7DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.7 Reference sensitivity power level for CA (8DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.3.1 EIS spherical coverage for CA (2DL CA) | TBD |  |
| 7.3A.3.2 EIS spherical coverage for CA (3DL CA) | TBD |  |
| 7.3A.3.3 EIS spherical coverage for CA (4DL CA) | TBD |  |
| 7.3A.3.4 EIS spherical coverage for CA (5DL CA) | TBD |  |
| 7.3A.3.5 EIS spherical coverage for CA (6DL CA) | TBD |  |
| 7.3A.3.6 EIS spherical coverage for CA (7DL CA) | TBD |  |
| 7.3A.3.7 EIS spherical coverage for CA (8DL CA) | TBD |  |
| 7.4 Maximum input level | TBD |  |
| 7.4A.1 Maximum input level for CA (2DL CA) | TBD |  |
| 7.4A.2 Maximum input level for CA (3DL CA) | TBD |  |
| 7.4A.3 Maximum input level for CA (4DL CA) | TBD |  |
| 7.4A.4 Maximum input level for CA (5DL CA) | TBD |  |
| 7.4A.5 Maximum input level for CA (6DL CA) | TBD |  |
| 7.4A.6 Maximum input level for CA (7DL CA) | TBD |  |
| 7.4A.7 Maximum input level for CA ((DL CA) | TBD |  |
| 7.5 Adjacent channel selectivity | PC3 ±8.08 dB (Max Device size ≤ 30 cm, FR2a, FR2b) ±9.46 dB (Max Device size ≤ 30 cm, FR2c) PC1 ±8.31 dB (Max Device size ≤ 30 cm, FR2a, FR2b) PC5 ±8.31 dB (Max Device size ≤ 30 cm, FR2a) PC6 ±8.28 dB (Max Device size ≤ 30 cm, FR2a) PC7 ±8.08 dB (Max Device size ≤ 30 cm, FR2a) | MTSU = 1.00 x MU (from Table B.21-1 in TR 38.903) |
| 7.5A.1 Adjacent channel selectivity for CA (2UL CA) | TBD |  |
| 7.5A.2 Adjacent channel selectivity for CA (3UL CA) | TBD |  |
| 7.5A.3 Adjacent channel selectivity for CA (4UL CA) | TBD |  |
| 7.5A.4 Adjacent channel selectivity for CA (5UL CA) | TBD |  |
| 7.5A.5 Adjacent channel selectivity for CA (6UL CA) | TBD |  |
| 7.5A.6 Adjacent channel selectivity for CA (7UL CA) | TBD |  |
| 7.5A.7 Adjacent channel selectivity for CA (8UL CA) | TBD |  |
| 7.6.2 In-band blocking | Same as 7.5 |  |

## <<< END OF CHANGES >>>