3GPP TSG-RAN5 Meeting #107	R5-252471

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4990 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR MBS test case 14.2.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Title for NR MBS 14.2.3.4 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the title for NR MBS 14.2.3.4 to “MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Lossless handover/ PDCP status report / 18 bit SN” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect title |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.3.4	MBS Multicast / PDCP/ PDCP HFN and SN maintenance  / Lossless handover/ PDCP status report / 18 bit SN

14.2.3.4.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and RLC-AM entity for PTP transmission and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < initialRX-DELIV }

then { UE discards the PDCP DATA PDU with RCVD_COUNT }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and RLC-AM entity for PTP transmission and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = initialRX-DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

(3)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and RLC-AM entity for PTP transmission and PDCP configured for 18 bit SN }

ensure that {

when { UE is requested to make a lossless handover with pdcp data recovery }

then {  UE creates a PDCP status report to SS }

}

(4)

with { UE is requested to make a lossless handover with pdcp data recovery and UE is configured with DL only RLC-UM entity for PTM transmission and RLC-AM entity for PTP transmission and PDCP configured for 18 bit SN }

ensure that {

when { UE receives the retransmitted PDCP DATA PDU in RLC-AM entity for PTP transmission which failed in RLC-UM entity for PTM transmission before handover}

then {  UE delivers PDCP Data PDU to upper layers }

}

(5)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and RLC-AM entity for PTP transmission and PDCP configured for 18 bit SN }

ensure that {

when { UE is requested to make a lossless handover with pdcp re-establishment }

then {  UE creates a PDCP status report to SS }

}

(6)

with { UE is requested to make a lossless handover with pdcp re-establishment and UE is configured with DL only RLC-UM entity for PTM transmission and RLC-AM entity for PTP transmission and PDCP configured for 18 bit SN }

ensure that {

when { UE receives the retransmitted PDCP DATA PDU in RLC-AM entity for PTP transmission which failed in RLC-UM entity for PTM transmission before handover}

then {  UE delivers PDCP Data PDU to upper layers }

}

<End of modified section 1>