3GPP TSG RAN Meeting #108	RP-251133
Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.4.3

Source:	Ericsson

Title:	Discussion on RRM and demod proposals for 5G-Advanced Release 20

Document for:	Discussion

# 1	Introduction

During RAN#107, first discussions were held on the RAN4 5G-A package. Discussions are expected to continue at RAN#108 prior to package approval at RAN#109. This contribution is provided for information with our views in each of the areas identified at RAN#107.

# 2	RRM

In the area of RRM generally there are very large number of proposals. Therefore, our aim is to focus on those areas which are urgent and have substantial practical significance and cannot be covered in work items led by other working groups. Based on the arguments we have the following two proposals for RRM.

- Further enhanced RedCap

- Rel-19 leftover

## 2.1 Further enhanced RedCap

Firstly, RAN4 introduced requirements for relaxed Radio Link Monitoring (RLM) and Beam Failure Detection (BFD) in release 17 for regular NR UEs as part of the UE power saving WI [2]. This feature allows the UE to achieve power saving gain by measuring less often when fulfilling some criteria, see Figure 1.

The criteria introduced are related to radio conditions of the UE with respect to the measured cell and mobility state of the UE. However, these requirements are currently not applicable for RedCap devices. Power saving is a relevant feature for RedCap UEs as they have reduced capability and cost. Therefore, we propose to extend this CONNECTED mode power saving feature for both 1 Rx and 2 Rx RedCap.

It was noted from the last plenary discussions that many companies have expressed support for this particular enhancement for RedCap. Thus it was highlighted as candidate topic with common interest among the companies in RAN4#107, see the summary document in [1].

Add RLM/BFD relaxation for Redcap devices in Rel-20 RRM.

RAN4 defined requirements to support L1/L2 triggered mobility for NR UEs in Rel-18 [2]. This feature enables low-latency handover while reducing the interruption time. But this feature is currently not supported for RedCap. Taking into consideration that RedCap comprises both 1Rx and 2Rx which in turn can support different use cases, we propose to develop requirements to support L1/L2 triggered mobility for RedCap by using the corresponding non-RedCap UE requirements as reference. From a UE complexity point of view, 2Rx RedCap is similar to non-RedCap UE and therefore it is possible reuse most of the existing requirements for 2Rx RedCap.

L1/L2 triggered mobility can be added for Redcap with low effort

## 2.2 Other topics

In addition to the RedCap related proposals in previous section, we are also interested in developing the RRM requirements to support on-demand SSB (OD-SSB) based SCell activation requirements for NR UEs. It is noted that only single SCell activation procedure is considered in ongoing release 19 NES work item. Given the deployment scenarios and commercial interest, extending this OD-SSB based SCell activation requirements for multiple SCell activation procedure can bring good power saving gain while it also reduces the SCell activation delays in multiple Scells. Therefore, in the interest of time, RAN4 could also consider this work.

It was also observed from last plenary discussions that there are various proposals (e.g. A-TRS based delay reduction) to speed up SCell activation procedure. While we share the view that reducing the SCell activation delays is important, RAN4 should use the time to improve the delays for type of SCells activation procedures which are already used in the field and/or likely to be used in the field, especially given that the RAN4 TUs are to be shared between 5G advanced related topics and 6G, not many RAN4 led 5G advanced topics are to be agreed. Therefore, we propose to focus on improvements which are urgent and have substantial practical significance.

## 2.3 Proposed objectives for RRM

For 1 RX and 2 RX (e)RedCap capabilities introduced in Rel-17 and Rel-18:

- Specify relaxed RLM/BFD evaluation period requirements based on the Rel-17 relaxed RLM/BFD requirements for non-RedCap UEs.

  - Relaxation is in time-domain and relaxation criteria (i.e. good serving cell- and low mobility criteria) from Rel-17 relaxed RLM/BFD work for non-RedCap UEs shall be reused

  - Relaxation condition (i.e. short DRX periodicity) from Rel-17 relaxed RLM/BFD requirements shall be reused.

# 3	Demodulation

During RAN#107, the following areas were identified as potential candidates for demodulation work in Rel-20:

Demodulation evolution

- SCM based requirements and/or further study of SCM

  - Depending on Rel-19 SI progress, and check the progress by September.

    - [At least for SU-MIMO requirement is expected in Rel-20].

- R-ML receiver for 8Rx SU-MIMO scenario

  - Focus on demodulation requirements rather than CSI reporting

  - FFS on whether the new UE capability is needed.

- CRI Reporting Requirements

- BS MMSE-IRC

  - For async TDD scenario and/or SBFD, and/or hetnet scenario with co-channel interference

- 6-layer performance requirements for handheld UE

- UE IC receiver

The spatial channel model has been studied during Release 19. The reasoning behind introducing a spatial channel model is that the current TDL based model used for the performance requirements does not properly capture spatial characteristics of real channels (considering directions of arrival). This leads to unrealistic MIMO requirements that do not properly test the performance difference between layers, and poor testing of PMI and interference mitigation as part of MU-MIMO.

Looking towards 6G, and also in the late stages of 5G, AI/ML is becoming of increasing importance. TDL modelling, that does not properly represent the spatial characteristics of the real world, is not sufficient for verifying the performance of AI models trained on real world data.

The reason that the CDL model was not introduced for 5G was that there was insufficient time to study and introduce it during the closing stages of release 15. To avoid a similar situation for 6G, which would be even more critical, since advanced high order MIMO, exploitation of the spatial domain and AI are integral parts of 6G, it is important to develop a suitable model for demodulation requirements.

For 5G, the spatial channel model is useful and meaningful, since also 5G includes SU- and MU- MIMO.

The remaining proposals for demodulation evolution all relate to more advanced receivers based on MIMO. It makes no sense to develop MIMO requirements without a proper channel model. Considering Rel-19 study item progress so far, we propose that RAN4 starts the normative work to introduce SU-MIMO requirements using the results of spatial channel model study in Rel-20 while it continues to study use of spatial channel model for MU-MIMO.

Aim to introduce SU-MIMO requirements using the results of the spatial channel model study in Rel-20.

Further discuss how to handle MU-MIMO in Rel-20.

## 3.1 Proposed objectives for Demod

- Specify SU-MIMO demodulation requirements:

  - Introduce PDSCH requirement for SU-MIMO with 8 layers

  - Introduce Rel-16 Type II PMI reporting requirements

  - Introduce Rel-18 non-AI/ML predicted PMI reporting requirements

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	L1/L2 triggered mobility can be added for Redcap with low effort

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add RLM/BFD relaxation for Redcap devices in Rel-20 RRM.

Proposal 2	Aim to introduce SU-MIMO requirements using the results of the spatial channel model study in Rel-20.

Proposal 3	Further discuss how to handle MU-MIMO in Rel-20.

# 4	References

[1]	RP-250817, “Moderator's summary for RAN4 led REL-20 topics”, RAN4 Chair (Huawei)

[2]	RP-233970, “Revised WID on Further NR Mobility Enhancements”, Rapporteur (MediaTek Inc., Apple)