3GPP TSG RAN Meeting #108					RP-251171

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Ericsson

Title:	Internal 3GPP requirements - NTN

Document for:	Approval

# 1	Introduction

Satellites are key to provide wireless communication services wherever they are needed even in areas not covered by terrestrial networks. Since digital services are key to enable education, healthcare and reduce isolation for populations, and since terrestrial infrastructures are vulnerable to war and natural disasters, non-terrestrial network infrastructure offers an opportunity to maintain connectivity almost everywhere.

# 2	Discussion

6G should be able to cover virtually all people on Earth, and virtually all areas, including oceans. With the seamless integration of 3GPP-compliant satellites into terrestrial networks, it shall be possible to deliver global coverage for basic mobile broadband, FWA, and IoT services.

5G NTN relies on GNSS for access procedures. The challenge with GNSS is that it is vulnerable to malfunction, interference and attacks (i.e., jamming and spoofing). Moreover, currently the 3GPP NTN technologies are fragmented depending on the use case (i.e., NR NTN and IoT NTN, which comprises NB-IoT and LTE-M).

A key differentiator for the 6G solution for NTN is to provide ubiquitous connectivity without depending on GNSS. More specifically, GNSS-free operation will result in improved resilience for critical services (e.g., public safety) and reduced complexity/battery consumption for demanding use cases (e.g., massive IoT). 6G should support NTN/TN integration from day 1 to provide efficient complimentary coverage of rural and remote areas currently underserved. Also, 6G will comprise a single RAT, covering various use cases including massive IoT and eMBB.

Resilience aspects related to NTN are discussed in our contribution RP-251168.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Functional requirements for a 6G Network with satellite access

### 5.x.x Seamless operation under mobility

The 6G RAN architecture shall support the seamless operation under mobility within and between terrestrial and non-terrestrial components of the network.

### 5.x.x GNSS-free operations

The 6G system shall support GNSS-free operation for the non-terrestrial network.

### 5.x.x Harmonized radio design

The 6G Radio design for the TN and NTN RAT should be harmonized, and the design should be able to cover various use cases including massive IoT.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.