3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250140

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3627 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NR5GC MAC testcase 7.1.1.12.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To re-verify NR5GC test case 7.1.1.12.4.1 to the IWD_24wk50 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.12.4.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | R5-250519 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR R5-250519 is agreed in RAN5#106 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

3 Branches executed	8

4	Execution Log Files	8

4.1	MediaTek MT6983 UE	8

4.2	HiSilicon Balong 5000 UE	8

5	References	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.1.12.4.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D24wk50

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983, HiSilicon Balong 5000

Verification Status:	      PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_4_NR_Common |
| --- | --- |
| Reason for change | The function f_NR_GetSCGConfig_SCellAdd returns an RUNTIME error due to the incorrect parameter p_CellGroupId is passed |
| Summary of change | The function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1 is modified to use parameter tsc_NR_SCellIndex_1 to overcome the RUNTIME error. |
| TTCN module | MAC_PowerSaving_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//DRX adaptation / SCell dormancy indication <<SKIPPED CODE>> //@siclog Steps 1-2 siclog@ //SS transmits an RRCReconfiguration message. (Note 1) //The UE transmits the RRCReconfigurationComplete message v_CellGroupConfig := f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(p_PCell, p_SCell, tsc_NR_CellGroupId_MCG); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_4_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//DRX adaptation / SCell dormancy indication <<SKIPPED CODE>> //@siclog Steps 1-2 siclog@ //SS transmits an RRCReconfiguration message. (Note 1) //The UE transmits the RRCReconfigurationComplete message v_CellGroupConfig := f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(p_PCell, p_SCell, tsc_NR_SCellIndex_1); //WA# v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); <<SKIPPED CODE>>

## Change 2

| Function name | f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1 |
| --- | --- |
| Reason for change | - The function f_NR_GetSCGConfig_SCellAdd returns an RUNTIME error due to the incorrect parameter p_CellGroupId is passed - Since the DRX-Config within MAC-CellGroupConfig is being setup, the other IEs must also be initialised else RUNTIME errors occur - Current TTCN implementation replaces search space ID 1 with search space ID 7 rather than adding the Search Space ID 7 to the Search Space Array list, this impacts the local end configuration which had been setup with 4 search spaces and is expecting search space id 1 at index 1 in the search space array |
| Summary of change | - The function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1 is modified to use parameter tsc_NR_SCellIndex_1 to overcome the RUNTIME error. - The IE mac_CellGroupConfig is initialised using the template cs_NR_MAC_CellGroupConfig - The Search Space ID 7 is added to the end of the SearchSpace Array list |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, CellGroupId p_CellGroupId ) runs on NR_BASE_PTC return template (value) CellGroupConfig {//DRX adaptation / SCell dormancy indication <<SKIPPED CODE>> var template (omit) BWP_DownlinkDedicated.pdsch_Config v_BWP_DownlinkDedicated_PDSCH_Config; //Table 7.1.1.12.4.1.3.3-2 v_CellGroupConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, p_CellGroupId); //configure DRX if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_CellGroupConfig.mac_CellGroupConfig); //compiler warning may be ignored <<SKIPPED CODE>> //Configure initial DL-BWP in SS @sic R5s240686 Ch2.3 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_PCell, tsc_NR_BWP_Id); //@sic R5s240686 Ch2.3 sic@ v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_AllOmit; v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(p_PCell, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s240686 Ch2.3sic@ <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, SCellIndex p_SCellIndex) //WA# runs on NR_BASE_PTC return template (value) CellGroupConfig {//DRX adaptation / SCell dormancy indication <<SKIPPED CODE>> var template (omit) BWP_DownlinkDedicated.pdsch_Config v_BWP_DownlinkDedicated_PDSCH_Config; //WA# var integer v_SearchSpaceIndex_DCI_2_6; v_CellGroupConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, p_SCellIndex); //WA# v_CellGroupConfig.mac_CellGroupConfig := cs_NR_MAC_CellGroupConfig;//WA# //configure DRX if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_CellGroupConfig.mac_CellGroupConfig); //compiler warning may be ignored <<SKIPPED CODE>> //Configure initial DL-BWP in SS @sic R5s240686 Ch2.3 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_PCell, tsc_NR_BWP_Id); //@sic R5s240686 Ch2.3 sic@ v_SearchSpaceIndex_DCI_2_6 := lengthof(v_DownlinkBWP.Pdcch.SearchSpaceArray);//WA# v_DownlinkBWP.Pdcch.SearchSpaceArray[v_SearchSpaceIndex_DCI_2_6]:= cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(v_SearchSpace); //WA# f_NR_CellInfo_SetDownlinkBWP(p_PCell, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s240686 Ch2.3sic@ <<SKIPPED CODE>>

## Change 3

| Function name | f_TC_7_1_1_12_4_NR_Steps3_9 |
| --- | --- |
| Reason for change | The DRB transmissions at Step 5 and Step 8 should happen on BWP#0 with data routed via the PDCCH of the SCell which does not necessitate the switching of the BWP. |
| Summary of change | Removed the configuration for the BWP switch. |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / SCell dormancy indication <<SKIPPED CODE>> //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-225297 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Step 6 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 7 siclog@ //The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) //Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_5Items('1'B, '1'B))); //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Steps 9 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }//f_TC_7_1_1_12_4_NR_Steps3_9

After Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / SCell dormancy indication <<SKIPPED CODE>> //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. // *REMOVED* f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-225297 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Step 6 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 7 siclog@ //The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) //Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_5Items('1'B, '1'B))); //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); // *REMOVED* f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Steps 9 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }//f_TC_7_1_1_12_4_NR_Steps3_9

# 3 Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “CA_n78c” using NEA1/NIA1 algorithms.

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_7_1_1_12_4_1_NR5GC_FR1_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## HiSilicon Balong 5000 UE

The HiSilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_7_1_1_12_4_1_NR5GC_FR1_HiSi.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s250071: Supporting information for Re-verification of NR5GC MAC testcase 7.1.1.12.4.1 |
| --- | --- |