3GPP TSG RAN Meeting #108							RP-251197

Prague, CZ, June 9-13, 2025

Agenda item:     15.1.2

Source: 	Tejas Networks, Reliance Jio, China Unicom , CEWiT, Lekha Wireless , IISc Bangalore , IIT Madras ,IIT Kanpur, IIT Bhilai

Title: 	                  Views on Rel-20 5G-Advanced MIMO Enhancements

Document for:      Discussion and Decision

Introduction

In this contribution, we provide our views on a set of potential MIMO Phase 6 enhancements for the Rel-20 work item. A key aspect we have identified involves DMRS enhancements, incorporating ports and port allocation combinations. Another important aspect is the reduction of CSI-RS overhead for more than 32 CSI-RS ports. We discuss each of the identified items in detail in the subsequent sections.

DMRS Enhancements

This section proposes objectives and justifications for enhancing Demodulation Reference Signal (DMRS) in 5G New Radio (NR), with a primary focus on addressing the increasing demands of large-scale multiple-input multiple-output (MIMO) systems. Fixed Wireless Access (FWA) has emerged as a critical use case within the 5G NR landscape, providing high-speed internet connectivity in areas where traditional broadband infrastructure is limited and enabling an additional monetization option for operators. However, these deployments introduce new challenges, including the need for higher-density cell configurations, improved channel estimation in scenarios involving high Modulation and Coding Scheme (MCS) indices, and significant delay spread. The proposal emphasizes DMRS port allocation strategies, including asymmetric and symmetric combinations across Code-Division Multiplexing (CDM) groups—to improve throughput and performance, particularly for FWA with outdoor Customer Premises Equipment (CPE).

## Justification

### Application

Fixed Wireless Access (FWA) has evolved as an extremely useful 5G NR use case, offering substantial advantages for residential and SME connectivity solutions. By leveraging 5G capabilities, FWA delivers high-speed internet without extensive wired infrastructure, which is particularly challenging in rural and underserved regions. Release 17 introduced 1024-Quadrature Amplitude Modulation (QAM) in the downlink to boost data rates and spectral efficiency, primarily targeting FWA devices. However, optimizing FWA deployment requires careful consideration of coverage, capacity, and economic feasibility, especially given the strain on backhaul networks from increased capital expenditure and operational expenditure.

Non-line-of-sight (NLoS) FWA deployments using massive MIMO (mMIMO) base stations (gNB) must support higher MCS levels (256QAM and beyond) in high delay spread environments. This necessitates improvements in throughput for high-MCS, high delay spread scenarios. A key area for optimization lies in re-evaluating DMRS port allocation principles across CDM groups. Current specifications for uplink Physical Uplink Shared Channel (PUSCH) and downlink Physical Downlink Shared Channel (PDSCH) in TS 38.212 limit port allocations to symmetric distributions within CDM groups, which restricts performance gains achievable through alternative allocation strategies.

Across Releases 17,18 and 19, significant enhancements toward joint transmission and multi-TRP objectives were achieved. These include a unified TCI Framework, improved CSI Reporting for NJCT, tight synchronization across TRPs for CJT (coherent joint transmission), and advanced codebooks. For Release 20 we could target the port allocation principle that incorporates DMRS port combinations, which provide an asymmetric advantage across ports and benefit TRPs with uneven performance across layers.

### Scheme

Figure 1  Proposed principle for DMRS port allocation

As shown in Figure 1, our studies demonstrate that adopting a new principle for DMRS port allocation enhances Mean Squared Error (MSE) performance in channel estimation. In this approach, ports for layer assignment are chosen across CDM groups, thereby introducing new sets of asymmetric and symmetric port allocation combinations spanning multiple CDM groups. This improvement occurs when user-assigned layers partially occupy CDM groups, allowing receivers to bypass de-spreading steps by treating Frequency Domain (FD) references as distinct reference signals rather than Orthogonal Cover Code (OCC)-multiplexed reference signals. Transmitter-side CDM spreading remains unchanged. Details of the design principle are covered in appendix Section 5.1.1.

### Link Level Simulation

The following Figure 2 and Figure 3 show improvement in throughput and channel estimation MSE based on enhanced DMRS port allocation.

Figure 2 Comparison between proposed DMRS port allocation and legacy port allocation for 3 port case

Figure 3 Comparison between proposed DMRS port allocation and legacy port allocation for 4 port case

#### Observation

Rank-3 configurations using enhanced port allocation, [0,2,4] (Type 2, Length 2), show 40% higher throughput compared to legacy port allocations, [0,1,2] (Type 1, Length 1).

Rank-4 configurations using enhanced port combination, [0,2,4,6] (Type 2, Length 2), demonstrate ~15-17% throughput gains over legacy port allocations, [0,2,4,6] (Type 1, Length 2).

- Combined with the growing emphasis on FWA performance, these findings justify exploring additional DMRS port allocation principles and defining new port combinations for enhancement

### System Level Simulations

Our system-level simulations with only FWA deployment for Proposed port allocation (for example Type-1 Len-2) demonstrate the performance gain of up to 4-6% compared to the baseline with Type-1 Len-1 DMRS type-1.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| BS antenna | 32T32R |
| FWA/UE Antenna | 6T6R/4T4R |
| Scenario/ISD | Uma, 1000m |
| Modulation | Up to 256 QAM (CQI Table-2) , Up to 1024QAM(CQI Table-4) |
| gNB Tx power | 49 dBm |
| UE/FWA Tx power | 23 dBm |
| gNB antenna height | 25 m |
| FWA/UE Antenna height | 5m/1.5m (variable for indoor) |
| gNB receiver noise figure | 5dB |
| FWA/UE receiver noise figure | 6/9dB |
| Numerology & Bandwidth | 30kHz SCS, 20 MHz |
| UE receiver | MMSE-IRC |
| Network Layout | - 8 FWA UEs per sector (in a total of 21 sectors (7 cells X 3 sectors per cell)) -168 Devices |
| UE distribution | Fixed outdoor |
| DMRS Configuration | Baseline: DMRS Type-1 Len-1 Proposed Enhanced DMRS port: Switch DMRS Type-1 Len-1 and Type-1 Len2 - Rank-1,2 -> DMRS Type-1 Len-1 - Rank-3,4 -> Switch to Type-1 Len-2 for benefited UE’s and De-spread bypassed |
| CSI feedback delay | 5ms |

Figure 4 CDF of UE DL Throughput for FWA deployment (CQI Table-2)

Figure 5 Overall throughput gains in % for DL for FWA devices (for CQI Table-2)

Figure 6 CDF of UE DL Throughput for FWA deployment (for CQI Table-4)

Figure 7 Overall throughput gains in % for DL for FWA devices (for CQI Table-4)

Observations

About 10% UE Experiencing a gain of ~10% for FWA devices with CQI Table-2 & about 30% of the UE experienced a gain of ~6% for FWA devices with CQI Table-2.

Overall cell throughput gains of about ~4% for FWA devices with CQI Table-2.

About 40% of the UE experienced a gain of ~8% for FWA devices with CQI Table-4.

Overall cell throughput gains of about ~6% for FWA devices with CQI Table-4.

For 4T4R base station configurations offer limited MSE improvement in channel estimation when employing advanced LMMSE algorithms with the proposed port allocation schemes. While some gains are observed at very high SNRs under high delay spread conditions, such scenarios may have limited practical relevance.

In contrast, configurations with a higher number of base station antennas (e.g., >8T8R) would provide noticeable MSE improvements at more practical SNR levels resulting in enhanced throughput for higher MCS making them better suited for enhancing channel estimation performance in realistic deployment scenarios.

### TU Estimate

Limiting the Port combinations to only Rank-3 and Rank-4 can significantly reduce the TU requirements to reach the agreements. Based on the following TU, we feel it is reasonable to include this WI for 5G-Advanced Rel-20 specifications.

|  |  | Activities | TU Estimate |
| --- | --- | --- | --- |
| 1 | RAN 1 | Simulations and Validation of Rank-3/Rank-4 benefits | 0.25TU for 1Q |
| 2 | RAN-1,2,3 | Agreements for the Port Table - Agreements on the Antenna port entries for Rank-3/4 for DL & UL - Specification changes for co-scheduled UE’S - UE Capability Signaling | 0.4 TU for 2Q |
| 3 | RAN-4 | Performance part | 0.25 TU |

### Views on UE and Specification impact

Following are the specification impacts for the proposed objective.

- New port allocation combinations are proposed for inclusion in the following tables of TS 38.212

  - Downlink PDSCH Channel: Tables 7.3.1.2.2-1 to 7.3.1.2.2-10A

  - Uplink PUSCH Channel: Tables 7.3.1.1.2-6 to 7.3.1.1.2-23D and Tables 7.3.1.1.2-38 to 7.3.1.1.2-69

- A new UE capability parameter is proposed for addition in TS 38.306 to indicate whether a device can utilize partially allocated CDM groups by bypassing the de-spreading step.

- Minor refinements to co-scheduling procedures in TS 38.214 are proposed, based on the new capability parameter.

- RAN-4 Performance Part

UE Impact:

As mentioned in Appendix Section 2.1, to improve performance, UE/CPE/FWA devices need to enable frequency/time de-spreading by treating FD/TD references as distinct, even though they are multiplexed over OCC on the transmitter side. This means devices must support bypassing the de-spreading process, which is not supported in legacy and current UEs. However, this can be managed using a UE capability parameter, so the gNB applies it only to compatible UEs. Given the time needed for 6G standardization and deployment, and the need for capacity enhancement for FWA especially in India, we recommend including DMRS port enhancement in the 5G-Advanced Rel-20 specifications for new devices.

### Summary of Justifications

- Performance enhancements in high MCS, high delay spread to cater to NLoS FWA, CPE use cases

Enhancement of  mTRP deployment to manage uneven layer performance from issues like time misalignment.

Improvement in the performance for deployments using higher SCS (larger impact of delay spread).

~40% for Rank-3, ~15-17% for Rank-4 in LLS Simulations in the example

- ~4%-6% Increase in NW throughput.

- Can be managed within a reasonable TU timeline.

UE impact can be managed through capability parameters to limit to Rel-20 FWA devices.

- This provides a common approach to both UL and DL capacity enhancement.

## Proposed DMRS Enhancement Objective

- On enhancing DL & UL capacity, specify the following enhancement:

  - Specify additional Demodulation Reference Signal (DMRS) antenna port configurations with particular emphasis on Fixed Wireless Access (FWA) deployments involving BS antenna arrays exceeding 8T8R:

- Investigate and define alternative DMRS antenna port design principles to enable more efficient port allocation strategies. Update the antenna port mapping tables in TS 38.212 by specifying new port combinations for SU-MIMO Rank-3 and Rank-4.

- Introduce enhancements to the co-scheduling mechanisms defined in TS 38.214 to support improved MIMO performance due to updated port mapping tables.

- Define additional UE capability signalling to maintain backward compatibility, ensuring minimal impact for legacy and non-supporting devices.

CSI-RS Overhead Reduction

This section presents the objectives and justifications for proposing CSI-RS resource overhead reduction as a key enhancement in Rel-20 5G-Advanced MIMO Phase 6 enhancements.

## Justification and Background

### Motivation

For DL channel sounding, NR Rel-19 agreed to enhance the CSI framework to support 128 CSI ports. To realize this enhancement, a maximum of four 32-port CSI-RS resources are aggregated in a single resource set and sounded as a single reference signal of up to 128-ports. In general, K M-port resources could be configured in a single resource set to sound  PCSI-RS=N CSI-RS ports, where N= K*M, such that K=N/M is the number of resources in a resource set.

The Rel-19 agreed 128-port CSI enhancements for FR1 may be deployed soon and could improve the spectrum efficiency of NR. However, several implementation challenges must be addressed:

Increased Resource Element (RE) Overhead: Supporting larger port counts requires extending the CSI-RS pattern in time, frequency, or both. A gNB will allocate more REs for transmitting reference signals, thereby increasing CSI-RS overhead. This leads to significant degradation in DL throughput, influenced by factors such as the number of aggregated CSI-RS ports (K resources), the frequency domain density (0.5 or 1), and the periodicity of CSI-RS occasions.

Energy and Complexity Impact: Regularly sounding full 128 ports at the desired periodicity increases energy consumption at the network transmitter and computational complexity of CSI processing at the UE receiver during CSI occasions.

While CSI-RS resource overhead may be manageable in rural deployments, it becomes critical in dense urban environments, smart city infrastructure, and industrial IoT networks where signal quality, interference levels, and propagation characteristics are dynamic, necessitating frequent CSI occasions for accurate and reliable network quality.

To address the challenges inherent in 128 port systems and to mitigate the associated degradation in network load and capacity, CSI-RS overhead reduction in the port (resource) domain presents a viable solution. An effective approach to achieve this is a flexible CSI-RS transmission scheme that involves transmitting full-port CSI-RS (covering all resources) occasionally, while transmitting partial-port CSI-RS (covering a subset of resources) more frequently.

The rationale behind this approach is to capture and report the two-dimensional (2D) spatial variations at a lower frequency, while enabling more frequent refinement of spatial characteristics along a single dimension, particularly where spatial variability is high, depending on the deployment scenario. This method leverages spatial flatness in one dimension to reduce overhead in the spatial domain, all without requiring modifications to the UE’s receiver algorithms.

We believe that using static-deployment-aware channel sounding is immensely helpful in 128-port massive MIMO narrow-beam systems. Because of the increased channel variability in such systems, it is otherwise challenging to reduce their rate of full-port CSI occasions. This simple incremental enhancement benefits infrastructure and UE vendors. It makes the roll out of 128 port implementations easier and proliferates their deployments in 5G-Advanced. The proposal addresses an urgent commercial need that favors real deployment, and we believe the proposal is a better fit in 5G-Advanced than in 6G.

### CSI-RS Port (Resource) Domain OH (Overhead) Reduction

Scheme Description

To achieve CSI-RS overhead reduction in port (resource) domain, the BS performs the following:

- Occasional sounding of full port CSI-RS with a long periodicity and

- Frequent sounding of reduced CSI-RS ports with a short periodicity,

The sounding of a subset of ports is such that it still aids full port CSI acquisition by leveraging spatial flatness in any one dimension over the chosen long period.

The flexible sounding of full-port CSI-RS and reduced-port CSI-RS is briefly explained as follows;

- All K M-port CSI-RS resources configured for aggregating full N ports will have a coarse period/lower rate of CSI occasions.

A subset of L M-port CSI-RS resources from among the aggregated full ports (where L < K) will have a relatively shorter period/higher rate of CSI occasions.

Figure 8: An illustration of the full-port and partial-port sounding in the coarser and finer periods, respectively.

The key parameters in our proposal are:

- N: Total number of CSI-RS ports

- M: Number of ports per CSI-RS resource

- K: Number of M-port resources for full sounding (K = N/M)

- L: Number of M-port resources for partial sounding (L < K)

Example Configuration Table:

| Total Ports (N) | Ports per Resource (M) | Full Resources (K) | Partial Resources (L) | Overhead Reduction |
| --- | --- | --- | --- | --- |
| 128 | 32 | 4 | 1 | 75% in fine periods |
| 128 | 32 | 4 | 2 | 50% in fine periods |
| 64 | 32 | 2 | 1 | 50% in fine periods |

As an example, we demonstrate the measurement and reporting techniques in scenarios requiring enhanced beamforming resolution in the horizontal domain (azimuth plane), by considering a dual-polarized large antenna panel with 128 ports (N1=8, N2=4).

Figure 9: An example CSI-RS mapping and CSI measurement reporting that exploit spatial flatness in any one dimension to seamlessly bring in spatial domain overhead reduction without the need for new UE receiver algorithms.

Figure 9 demonstrates a practical implementation exploiting spatial flatness in one dimension. In this example:

- The horizontal domain (H-domain/azimuth plane) requires higher beamforming resolution and more frequent updates.

- The vertical domain (V-domain/elevation plane) exhibits spatial flatness and can be updated less frequently.

Full CSI-RS ports are sounded in the coarse periods, and the UE reports complete CSI including parameters for both dimensions (i11, i12). In the fine periods, only a subset of ports corresponding to the H-domain are sounded, and the UE reports only parameters for that dimension (updated i11).

The BS receiver then combines:

- Updated i11 parameters from fine CSI occasions.

- Previously received i12 parameters (from the most recent coarse CSI occasion).

This allows the BS to maintain full CSI knowledge while reducing overhead.

Benefits

- Network Energy Saving: In the fine periods/occasions with reduced CSI-RS ports, only a subset of antennas is active.

- UE Complexity Reduction: In the fine periods, UE measures channel for reduced ports only, without needing to infer the entire channel, potentially reducing computation by up to 75% during these periods.

- Increased Data Throughput: More data can be packed into the REs freed up in fine periods where only a subset of CSI-RS resources is sounded.

### System Level Simulations

Our system-level simulations demonstrate that the beam variations in the domain requiring higher beamforming resolution are well captured using the proposed method, while achieving throughput gains.

Furthermore, due to the reduced CSI-RS resource elements (REs) in the fine occasions where only a subset of resources is sounded, more data can be packed into the REs, thereby increasing the data throughput. Figure 10 shows a 5.3% throughput gain with the proposed flexible sounding approach compared to the baseline with only full-port sounding.

Figure 10: Throughput gain with the proposed flexible sounding of full and reduced CSI-RS ports.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Scenario/ISD | UMa |
| Modulation | Up to 256 QAM |
| gNB Tx power | 44 dBm |
| UE Tx power | 23 dBm |
| gNB antenna height | 25 m |
| gNB receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Numerology & Bandwidth | 15kHz SCS, 20 MHz |
| UE receiver | MMSE-IRC |
| Network Layout | 10 UEs per sector (in a total of 21 sectors (7 cells x 3 sectors per cell)) |
| UE distribution | 80% Indoor (3Km/h) 20% outdoor (30km/h) |
| CSI feedback delay | 5ms |

### TU Estimate

In our view, the TUs required for the inclusion of CSI-RS port (resource) domain overhead reduction are very small. There is a proposal to use frequency domain CSI-RS overhead reduction techniques which are also straightforward, like the proposed CSI-RS overhead reduction in the port (resource) domain. This means that by excluding the UE receiver enhancements required to recover the missing ports in spatial domain overhead reduction, the CSI-RS overhead reduction in both frequency and port (resource) domains can be completed with 0.25 TUs.

### Specific Specification Impact

Current specifications that include up to 128-port CSI-RS support using aggregated resources offer nominal challenges to realize the flexible sounding approach. The following specific changes may be needed:

- TS 38.214: Modify CSI-RS resource configuration to support different periodicities for subsets of resources within a resource set.

- TS 38.331: Enhance RRC signaling to configure different periodicities for CSI-RS resources within a resource set.

These changes can be realized through minor modifications to the CSI-RS resource and report configurations and settings, specifically:

- Altering the periodicity rules of resources.

- Minor enhancements to report quantity types.

Given its minimal specification effort, it is reasonable to support this enhancement as part of Rel 20 5G-Advanced standardization.

### Implementation Impact

BS Implementation: The BS receiver buffers the i1 parameters (for the dimension that is spatially flat) received in a coarse CSI occasion until the next coarse occasion, applying them during the fine occasions. This requires relatively simple changes to existing BS implementations.

UE Implementation: The UE does not require advanced receiver processing to recover the absent ports, as it does not infer the entire channel. The UE can operate with reduced computational complexity (potentially equivalent to 32-port legacy processing) in the fine periods where only a subset of ports (resources) is sounded and measured.

## Proposed CSI-RS Overhead Reduction Objective

To accelerate 5G-Advanced deployments with up to 128-port CSI-RS, we propose enhancing the CSI framework to support occasional full-port CSI-RS sounding and frequent sounding of a subset of CSI-RS ports. This enhancement would enable full-port CSI acquisition at the BS across all periods while reducing overall CSI-RS RE overhead, UE complexity, and network energy consumption without requiring hardware changes.

Potential WID Objectives:

Specify CSI-RS overhead reduction for up to 128 CSI-RS ports in frequency and port (resource) domains, without presenting UE receiver enhancements for CSI acquisition.

- Modify NR CSI framework to enable partial frequency CSI-RS sounding and measurement to support frequency domain overhead reduction.

- Modify NR CSI framework to enable occasional sounding of full CSI-RS ports (resources) and frequent sounding of a subset of CSI-RS ports (resources) to support port domain overhead reduction.

- Enhance the CSI-RS resource and report configurations and settings to enable CSI-RS overhead reduction.

Proposed Objectives for Rel-20 5G-Advanced MIMO Phase-6

For Rel-20 5G-Advanced MIMO Phase-6 enhancements, we propose the following enhancements:

- On enhancing DL & UL capacity, specify the following enhancement:

  - Specify additional Demodulation Reference Signal (DMRS) antenna port configurations with particular emphasis on Fixed Wireless Access (FWA) deployments involving BS antenna arrays exceeding 8T8R:

- Investigate and define alternative DMRS antenna port design principles to enable more efficient port allocation strategies. Update the antenna port mapping tables in TS 38.212 by specifying new port combinations for SU-MIMO Rank-3 and Rank-4.

- Introduce enhancements to the co-scheduling mechanisms defined in TS 38.214 to support improved MIMO performance due to updated port mapping tables.

- Define additional UE capability signalling to maintain backward compatibility, ensuring minimal impact for legacy and non-supporting devices.

- On CSI-RS overhead reduction, specify the following enhancement:

Specify CSI-RS overhead reduction for up to 128 CSI-RS ports in frequency and port (resource) domains, without presenting UE receiver enhancements for CSI acquisition.

Modify NR CSI framework to enable partial frequency CSI-RS sounding and measurement to support frequency domain overhead reduction.

- Modify NR CSI framework to enable occasional sounding of full CSI-RS ports (resources) and frequent sounding of a subset of CSI-RS ports (resources) to support port domain overhead reduction.

- Enhance the CSI-RS resource and report configurations and settings to enable CSI-RS overhead reduction.

Appendix

## DMRS Port Allocation Proposal

### DMRS Port Allocation Principle

Adopt a new principle for the DMRS port allocation, where the ports for layer assignment are chosen across CDM groups such that at least one layer is allocated to each group before additional layers are assigned within the same group. This approach provides a Mean Square Error (MSE) advantage to layers in Channel Estimates for partially allocated CDM groups.

Figure 11 Proposed strategy for port allocation

As shown in the above Figure 11 , the orange arrows illustrate the sequence of layers mapped to the CDM ports. Here the layers are first mapped across CDM groups (indicated by an arrow directed across CDM grp) and once at least one port from each CDM group is assigned then the layers are mapped within the CDM group (indicated by the horizontal arrows).

### Proposed DMRS Port Combinations

Based on the port allocation principle described in Section 5.1.1, we propose additional rows for the Antenna (DMRS) port allocation tables in TS 38.212. The following Table 1 shows the proposed DMRS port combinations.

To leverage the performance benefit, the receiver must bypass the de-spreading step in Channel Estimation (CE) for partially allocated CDM groups by treating references on corresponding frequency and time domain resources as distinct rather than OCC.

Modification is required only on the receiver side (Software upgrade), and no changes are required on the transmitter.

Table 1 Proposed port allocation combinations

| Number of DMRS CDM group(s) without data | Proposed DMRS/Antenna port combination | Number of Front Loaded DMRS Symbols | Tables Impacted (TS 38.212) | DL/UL |
| --- | --- | --- | --- | --- |
| 2 | [0,2,4] | 2 | Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,2,4] | 1 | Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1 | DL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1 | DL |
| 3 | [0,2,4] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,1,2,4] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 2 | [0,1,2,4] | 2 | Table 7.3.1.2.2-8: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 3 | [0,2,4] | 1 | Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 3 | [0,2,4] | 2 | Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 2 | [0,2,4] | 2 | Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2, rank = 3 | UL |
| 3 | [0,2,4] | 1 | Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1, rank =3 | UL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.1.2-19: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1, rank =4 | UL |
| 3 | [0,2,4] | 2 | Table 7.3.1.1.2-22: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank=3 | UL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank=4 | UL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.1.2-57: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1, rank = 4 | UL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.1.2-65: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2, rank = 4 | UL |