3GPP TSG RAN Meeting #108	R4-251284

# Prague, Czech Republic, June 9-13, 2025

Source: 	KDDI

Title: 	UE simulation assumption for A-MPR

Agenda Item:	15.4.2

Document for:	Discussion

- 1. Introduction

In RAN#104 meeting, a new WID [1] Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 is approved. The WID includes considerations for existing PC3 requirements as follows.

- 4.1 Objective of SI or Core part WI or Testing part WI - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

In this contribution, we discuss the UE simulation assumptions and the prerequisites for specification of PC2 and wider bandwidth in Band n28 using a full-band duplex filter.

- 2. Discussion

2.1 Simulation assumptions and results of n28 PC3 operation in NS_17 with a full-band duplexer

The legacy assumptions for MPR/A-MPR simulations are based on following factors.

•	Fixed Bias

•	NR Waveform

•	Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

•	Carrier Leakage: 28dBc

•	Image: 28dBc

•	CIM3: 60dBc

•	CIM5: 70dBc

According to legacy assumptions, PC3 A-MPR was proposed in [2], and also PA back-off to meet the NS_17 emission requirements was proposed in [3] in the previous RAN4 meetings.

Table 3 in [2]: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | >= 5.4 | A2 |

Table 4 in [2]: A-MPR for NS_17 for PC3

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 2.0 | ≤ 2.5 |
|  | QPSK | ≤ 2.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 3.0 | ≤ 4.0 |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 4.0 | ≤ 5.0 |
|  | 256 QAM |  |  |

PA back-off to meet the NS_17 emission requirements in [3]: Observation

Observation: Measurements indicate the following PA back-off to meet the NS_17 emission requirements: - For PC3: 3.5dB; - For PC2 single Tx: 2.5dB; - For PC2 dual-tx: 4.5dB.

Based on the legacy assumptions for A-MPR simulations, the companies’ simulation results show a necessity to introduce A-MPR of n28 PC3 operation in NS_17 with a full-band duplexer.

Observation 1: Based on the legacy assumptions for A-MPR simulations, the companies’ simulation results show a necessity to introduce A-MPR of n28 PC3 operation in NS_17 with a full-band duplexer.

2.2 Real UE implementation of n28 PC3 in NS_17 with a full-band duplexer

On the other hand, in the previous RAN4 meeting, one company pointed out that from real UE implementation perspective, some UE products have already implemented full-band b/n28 duplex in Japan around three years ago, so far no issue identified in the field nor in the conformance test assuming existing PC3 regulatory requirements (i.e. A-MPR=0) for DTV protection, and the margin is quite enough particularly for LTE [4].

Observation 2: One company point out that from real UE implementation perspective, some UE products have already implemented full-band b/n28 duplex in Japan around three years ago, so far no issue identified in the field nor in the conformance test assuming existing PC3 regulatory requirements (i.e. A-MPR=0) for DTV protection, and the margin is quite enough particularly for LTE.

2.3 Contradiction between RAN4 simulation results and real UE implementation

The two observations shown in the previous section means that there will be a contradiction between RAN4 simulation results and real UE implementation for A-MPR of n28 PC3 operation in NS_17 with a full-band duplexer.

Observation 3: There is an issue on a contradiction between RAN4 simulation results and real UE implementation for A-MPR of n28 PC3 operation in NS_17 with a full-band duplexer.

In the RAN plenary#104 meeting, this issue was discussed, but also there were no common consensus and no conclusions and RAN decided no changes to the existing requirements for PC3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing PC3 networks in the new Rel-19 n28 WI[1].
 Additionally, in the RAN4#112, one company pointed out that the current assumptions for A-MPR simulations and specification are not representative of state-of-the-art performance and updated assumptions for A-MPR simulations that enable compliance with existing RAN4 requirements for PC3 without A-MPR and an A-MPR specification for PC2 not removing the virtues of PC2 should therefore be used [5].
 Furthermore, in the RAN4#114bis, one company proposed to consider the following new assumptions for the simulation assumptions on n1 PHS protection update. Because the company pointed out that current above simulation assumptions have been applied for the last 10 years [6].

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -34dBc

- Image: -34dBc

- CIM3: -60dBc

- CIM5: -70dBc

Observation 4: There is a proposal on new simulation assumption for n1 PHS protection update with new Carrier Leakage value and IQ Image value in RAN4#114bis.

We think it would be better to study the issue on a contradiction between RAN4 simulation assumptions/results and real UE implementation as a general issue for some bands not focusing on n1 and n28 outside of the new Rel-19 n28 WI in Rel-19 timeframe. Even if the issue can’t be fully solved in Rel-19 timeframe, RAN and RAN4 should continue related discussions and solve the issue toward 6G era, in order to avoid to repeat same discussions in 6G.

Observation 5: It would be better to study the issue on a contradiction between RAN4 simulation assumptions/results and real UE implementation as a general issue for some bands not focusing on n1 and n28 outside of the Rel-19 n28 WI as Rel-20 5G-Advanced.

- 3. Conclusion

Observation 1: Based on the legacy assumptions for A-MPR simulations, the companies’ simulation results show a necessity to introduce A-MPR of n28 PC3 operation in NS_17 with a full-band duplexer.

Observation 2: One company point out that from real UE implementation perspective, some UE products have already implemented full-band b/n28 duplex in Japan around three years ago, so far no issue identified in the field nor in the conformance test assuming existing PC3 regulatory requirements (i.e. A-MPR=0) for DTV protection, and the margin is quite enough particularly for LTE.

Observation 3: There is an issue on a contradiction between RAN4 simulation results and real UE implementation for A-MPR of n28 PC3 operation in NS_17 with a full-band duplexer.

Observation 4: There is a proposal on new simulation assumption for n1 PHS protection update with new Carrier Leakage value and IQ Image value in RAN4#114bis.

Observation 5: It would be better to study the issue on a contradiction between RAN4 simulation assumptions/results and real UE implementation as a general issue for some bands not focusing on n1 and n28 outside of the Rel-19 n28 WI as Rel-20 5G-Advanced.

- 4. Reference

[1] RP-2401660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN, RAN#104

[2] R4-2400170, On n28 with full band duplexer, Apple, RAN4#110

[3] R4-2407163, PC2 n28 NS_17, Skyworks Solutions, Inc., RAN4#111

[4] R4-2407624, Views on b/n28 full-band duplex, Samsung, RAN4#111

[5] R4-2411671, 40 MHz channel bandwidth with PC2 in Band n28, Ericsson, RAN4#112

[6] R4-2504569, A-MPR regions for NS_05 and NS_05U and optionality in earlier releases by modified MPR behaviour, Ericsson, RAN4#114bis