3GPP TSG RAN #108	RP-251310

Prague, Czech Republic, 9th – 13th June, 2025

Agenda item:		16.1

Source:	MediaTek Inc.

Title:	Proposed inputs to 6G RAN Study Item TR 38.914

Document for:		Discussion and Decision

# Introduction

This contribution proposed requirements and corresponding text proposals for the 6G Scenarios & Requirements SI TR.

# Discussion

## Spectral efficiency

The 6G RAN Scenarios and Requirements SID in [1] states an objective for “enhanced spectral efficiency”.

Spectral efficiency targets for IMT2030 will cover existing and new bands. MNO feedback suggests that focus on Average and 5%ile spectral efficiency is more important than improved peak spectral efficiency. This should be highlighted by 3GPP.

We believe that the covered ITU-R scenarios covered would be broad enough to cover improvements in all scenarios. However, we are open to also consider 3GPP evaluation for relevant bands in Suburban scenario, as requested by some MNOs.

Spectral efficiency will clearly depend on number of antennas at device and Base Station. Typical FWA/CPE may have different targets to typical handheld devices, and some graceful degradation in performance would be expected for devices with lower Tx/Rx antenna capability e.g. smaller form factor devices.

Proposal 1: The following TP is proposed:

“6G shall target spectral efficiency improvements in all targeted existing and new spectrum, with particular focus on improving the average and 5%ile performance in accordance with IMT-2030 targets and deployment scenarios [TBD: and additionally Suburban scenario]. ‘Typically expected’ BS and UE antenna capabilities for the corresponding frequency band and deployment scenario shall be evaluated, and at least handheld and FWA/CPE device types should be evaluated.”

## Capacity

We believe that coverage from IMT-2030 TPRs for Spectral Efficiency, Area Traffic Capacity and Composite TPR for Immersive Communication should be sufficient to ensure focus on both generic and latency-bound capacity requirements, where latency-bound capacity will be important for Immersive Communication traffic profiles.

Proposal 2: Reference IMT-2030 TPRs and target values for Spectral Efficiency, Area Traffic Capacity for generic capacity requirements, and Composite TPR for Immersive Communication for latency-bound system Capacity requirements. 3GPP should evaluate performance for handheld, CPE/FWA and Wearable device types.

## Coverage

6G SID requires “Enhanced overall coverage, focus on cell-edge performance and UL coverage” and “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”.

5%-ile spectral efficiency and “User data rate” IMT-2030 TPR should help to characterise the level of improvement achievable in cell edge performance. However, more detailed deployment assumptions on the 7GHz deployment reusing the 5G mid band site grid may be required.

In relation to “enhanced overall coverage”, we believe an important enabler for 6G would be in improving baseline coverage availability, i.e. enable coverage which legacy generations with the same site grid are not able to. This should be applicable to extended TN coverage and complemented by a strong NTN baseline coverage performance.

Extended TN coverage is targeted for different bands, but more extreme on baseline coverage layer may be targeted on the coverage layer.

Proposal 3: Agree the following TP: “6G coverage requirements shall focus on the following:

- 6G operating spectrum at ~7GHz shall target comparable coverage as 5G mid-band (~3.5GHz) assuming reuse of the same site grid. Target cell edge data rate in uplink and downlink is FFS.

- Target terrestrial coverage extension for basic connectivity for all device types beyond that achievable by 5G terrestrial networks. The level of extension should be at least [10dB] beyond that of 5G (non-LPWA), for TBD QoS and for a given device antenna configuration. Requirements may differ for baseline coverage bands and higher bands.

- 6G NTN as a seamless complement to TN coverage with baseline coverage performance improvement vs rel-19 NR NTN targeted.

- 3GPP should evaluate performance at least for handheld, CPE/FWA.”

## Latency and Reliability

Reducing minimum latency and increasing highest reliability vs IMT-2020 is neither necessary nor required. More important in our view is sufficiently stringent latency and reliability for practical traffic profiles and with practical assumptions. An important practical area is also to ensure the latency tail for the last few percent of packets can be constrained.

Therefore, in addition to the IMT-2030 targets for Latency, Reliability, and Composite TPR, we believe that 3GPP should target improvement of the latency tail for a given user ([100%] of packets being delivered within a given latency) based on link level analysis.

Proposal 4: Agree the following TP: “6G latency and Reliability shall focus on the following areas:

- Target lowest latency same as IMT-2020.

- Target highest reliability same as IMT-2020.

- Practical latency and reliability aspects are a more important focus area for eMBB and Immersive Communication:

  - Composite latency and reliability for Immersive Communication shall target TBD performance.

    - The need for composite latency and reliability for other usage scenarios is FFS.

  - Improving the latency tail is important for latency-bound services.

  - HARQ, scheduling delays and support (e.g. SR/BSR) for uplink scheduling under typical assumptions (duplex mode constraints, dynamic scheduling) shall be accounted for.

- 3GPP should evaluate performance for handheld, CPE/FWA and Wearable device types.”

## 2.5	Sustainability

The following is important in our view for Sustainability:

- Network and device energy efficiency in zero traffic load, and partial traffic load (low, medium, high)

- Consideration of full buffer and FTP traffic profiles.

- Power models from 3GPP TR38.864 should be used to evaluate network energy efficiency.

- Power models for device energy efficiency should be reused from TR38.840.

- User experience should not be noticeably degraded when enabling network energy efficiency.

Proposal 5: Agree the following TP:

“A joint approach to network and device energy efficiency improvement in zero traffic load, and partial traffic load (low, medium, high) shall be targeted, covering realistic traffic models, and without noticeable degradation to end user experience.

- Energy efficiency improvement for Network and Device for a given system loading level shall be calculated from the following formula:

  - 100 * ( EnergyConsumption_Baseline - EnergyConsumption_6G ) / EnergyConsumption_Baseline

- The following design targets are recommended for [Dense Urban] deployment scenario, with power model assumptions according to TR38.864 and TR38.840 for Base Station and UE respectively:

  - Partial load EE improvement – BS: [35%]; UE: [45%]

  - Zero load EE improvement – BS: [50%]; UE: [70%]

  - EnergyConsumption_Baseline shall assume operation of the following:

    - Rel-18 with NES for BS

    - Rel-17 with UE Power Saving for UE”

## Mobility

It is important to improve “practical” mobility support in 6G, including beam-based mobility. Interruption requirements <[10] milliseconds should be targeted. But also ensuring data rates can be optimized immediately after the interruption.

Zero millisecond interruption is not a necessary design target in our view. If zero millisecond interruption requirement is introduced as an IMT-2030 TPR, it shall only be accounted for in special scenarios such as multi-TRP operation or inter-site spectrum aggregation.

5G NR has specified different mobility concepts: L3 mobility, LTM, CHO, and there is also intra-cell MIMO Beam Management. 6G should aim for a harmonized and simplified mobility framework, including harmonization of LTM and intra-cell beam management concepts.

Proposal 6: Agree the following TP:

6G mobility should aim to harmonize mobility framework/procedures compared to NR design, including LTM and intra-cell beam management, with practical interruption time target of < [10] ms.

## 2.7	Bandwidth, Spectrum and Duplexing

Regarding channel bandwidth, it is proposed to document that 200MHz single channel bandwidth shall be specified for bands in 7GHz range, and potentially for bands above 2GHz, as the following was added in the SID: “Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz”,

The smallest channel bandwidth should cover at least 3MHz deployments in sub-1GHz bands.

The need for more flexibility in UE channel bandwidths between 3-13MHz shall be considered.

SID says “Improved spectrum utilization and operations taking into account diverse spectrum allocations” is an objective. 3GPP should target enhanced aggregation of spectrum in terms of faster activation/deactivation of carriers at network and device, as well as improved operation across carriers.

Duplexing capability shall cover FDD and HD-FDD, TDD, and SBFD.

Proposal 7: Agree the following TP:

- “6G shall target improved spectrum utilization and operations taking into account diverse spectrum allocations. Study how to efficiently support channel bandwidths ranging from [3MHz] in low FR1 bands to 200MHz in bands above 2GHz / around 7GHz. The study should consider the need for additional flexibility vs 5G NR in the range from 3 – [13] MHz bandwidth, considering also complexity trade-offs.

- Study enhanced aggregation of spectrum, considering activation/deactivation of spectrum blocks at network and device and improved scalability and flexibility in combining spectrum.

- In terms of duplexing, the study shall target support FDD, TDD, SBFD and HD-FDD operation.”

## Migration, coexistence and interworking with legacy RATs

Multi-RAT spectrum sharing (MRSS) between 5G and 6G will be key for enabling 6G in existing bands, but sharing with 4G shall also be targeted, especially taking into account legacy LPWA deployments.

Proposal 8: Agree the following TP:

“The design of dynamic spectrum sharing between 5G and 6G shall minimize overheads within the spectrum block. For sharing between 4G and 6G, only (semi-)static sharing is required.”

## TN/NTN harmonization

In 5G NR, TN and NTN technologies were mainly developed in parallel with limited scope for fully realizing synergies and common functionalities. For 6G, harmonization and commonality of functionality should be considered right from the start, to minimize the complexity of supporting 6G radio for both TN and NTN.

Proposal 9: Agree the following TP:

“6G shall aim for a harmonised design for NTN and TN and their mutual integration. The study shall strive for common and extensible designs for TN and NTN deployment to minimise the complexity for the TN RAN/UE to support NTN.”

## Sensing

A holistic study of sensing use cases and modes of operation is required to ensure a futureproof system design. Furthermore, when sensing and communication operate on the same carrier, both operations shall mutually coexist optimally. Therefore, awareness is needed at both NW and UE regarding sensing activities.

Proposal 10: Agree the following TP:

“The 6G system should efficiently support sensing, and the study should cover at least the following aspects:

- Use cases: The following use cases should be taken into account:

  - Detection/tracking of UAV/car/pedestrian/AGV

  - Human health and motion monitoring

  - Environment reconstruction

  - Intrusion detection (smart home, highway/railway)

  - Sensing to assist communication e.g. Beam Management, CSI reporting, Positioning, Power Saving

- Sensing modes: TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, UE monostatic, and UE-UE bistatic should all be studied. No “sidelink” expected for UE-UE sensing.

- Coexistence aspects: Sensing activities within 6G radio resources should be known to both network and UE.”

# Conclusion

It is proposed that the Proposals 1-10 in this document are agreed and the corresponding TPs incorporated into the 3GPP TR 38.914.

# References

[1]		RP-250810, “Revised SID: Study on 6G Scenarios and requirements”; CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.