3GPP TSG RAN Meeting #108								RP-251318

Prague, Czech Republic, June 9-13, 2025

Agenda item:			15.2.5

Source:	Intelsat, Gilat, Viasat, Inmarsat, JSAT, Kratos, Tejas Networks, MITRE, Lockheed Martin

Title:	Multi-Orbit Solution for Non-Terrestrial Networks,

Document for:	Discussion and Decisions

# Introduction

In this contribution we like to introduce the concept of Multi-Orbit Connectivity solution for Non-Terrestrial Networks (NTN).

Since 3GPP Release 17 Non-Terrestrial Networks (NTN) have been established as supplementary network solutions for UEs and NB-IoT devices and together with the conventional terrestrial networks they offer a ubiquitous connectivity solution over land and over water.

In our view this ubiquitous coverage can be further extended.

In our view, the next evolution in the NTN connectivity solution would be the multi-orbit solution, whereby a UE or a VSAT can connect to various satellites in various orbits, as illustrated in the following Figure 1.  Figure 1 illustrates GEO orbit and LEO orbit for simplicity but potentially MEO orbit is also possible.

Figure 1 Multi-Orbit Connectivity

By leveraging the strengths of satellites at various orbital altitudes, this solution can potentially create a robust and versatile solution that enhances overall performance, enabling high-speed internet access, improved coverage, enhanced communication, and resilient services.

With the growing expansion of the communication services and ever-growing needs for data connectivity, multi-orbit solution is becoming essential to remain competitive. In addition to the technological enhancements, the multi-orbit solution has commercial advantages for operators who operate NGSO and GSO constellations.

In this concept, delay sensitive applications can be routed to LEO constellations, while non-delay sensitive applications can be routed to GEO; essentially load balancing the application data traffic using multi-orbit constellations. Following this approach, connections from GEO to LEO, MEO to LEO etc. can be prioritized based on their service requirements.

Additionally, with this multi-orbit approach, legacy satellites can be used for fall-back resiliency, which may also include non-3GPP satellite systems. Multi-orbit solution also offers services resilience as even if one satellite fails, the network can switch services through another satellite.

# Multi-Orbit Solution Overview

## Mobility Solution

Figure 1 illustrates the concept of the multi-orbit solution for Non-Terrestrial Networks.

In this concept the VSAT (or the UE) can connect to LEO or GEO constellations at any one time

From the VSAT’s perspective, it only ‘sees’ various terrestrial and satellite cells on the ground, illuminated ether by the mobile base-stations or by satellites above it. See following Figure 2

Figure 2 Satellite and terrestrial cell seen by the VSAT.

Irrespective of the type of cell, TN or NTN, the multi-orbit solution is an enhancement of the cell selection and re-selection procedure that we are familiar with.

As with the conventional mobility solution available to date, there would be two distinct multi-orbit mobility solutions:

- Mobility in the idle mode (RRC_Idle or RRC_Inactive)

- Mobility in the connected mode (RRC_connected)

See following Figure 3

Figure 3 Mobility in Idle and Connected Mode

## Service Anchor

In the mobility solutions that have been developed in Rel17 and Rel18, whereby the UE is handed out and handed back in, the service anchor has been the terrestrial network, and all the signalling was anchored in the terrestrial 5G core.

With the multi-orbit solution, this service anchor would be in the satellite’s 5G Core Network whereby the VSAT is handed between GEO, LEO or a terrestrial cell. See following Figure 4

Figure 4 Service Anchors

Mobility Scenarios

With the multi-orbit approach, we envisage additional mobility scenarios such as

- Scenario 1 Terrestrial Cell to/from LEO Cell

- Scenario 2 Terrestrial Cell to/from GEO Cell

- Scenario 3 LEO Cell to/from GEO Cell

- Scenario 4 GEO Cell t/from GEO Cell

- Scenario 5 LEO Cell to/from LEO Cell

See following Figure 5. Scenarios 3-5 would require further analysis.

Although not shown MEO cells could also be part of the Mobility Scenarios

Other handover scenarios could be possible.

Figure 5 Various Handover Scenarios

## Mobility Triggers

For the VSAT to be handed from one cell to another certain network condition must occur for the VSAT to transition to another cell these triggers are as follows:

### Signal level based

The VSAT handover from weak to strong cell based on RSRP/RSRQ values. Hand-out and hand-in solution was completed in Rel17 and Rel18 respectively.

### Location Based

The VSAT handover is triggered by VSAT’s location on the ground. This solution is GPS based, and it is currently being developed in Rel19 by RAN2.

See following Figure 6

Figure 6 Handover Based on Signal Strength and Location Based

### Other Mobility Triggers

- Time Based: The handover trigger is based on the length of time a VSAT has remained in a particular cell.

- Load Balancing: The VSAT is pushed to another less-congested cell due to cell congestion.

- Service Based: The VSAT is pushed to another cell due to service requirements.

- User Priority Based

  - Mission-critical users have higher priority.

  - Emergency service users have higher priority over regular users.

  - Gold customers have priority over regular users,

- Regulatory Triggers

  - VSAT is triggered to handover to a new cell due to Regional Regulatory Requirements

- Elevation Based Triggers

  - NGSO satellite elevation above the ground triggers handover to another cell

## Handover Type

### Terrestrial to Non-Terrestrial Cell

- Baseline Handover (BHO)

- Conditional Handover (CHO)

### GSO to NGSO Cell Handover

- GEO to LEO Cell and vice versa via the 5G Core network

- GEO to LEO cell and vice versa through inter-satellite links

### NGSO to NGSO Handover

- Handover between to LEO Cells or LEO to MEO cells

### Intra-Satellite Handover

- Intra-satellite handover between smaller GEO Cells; local gNodeB switching.

## RAN Network Architecture

RAN Transparent and Regenerative architectures offer several solutions for handover: depending on the implementation and throughput capacity available on the feeder links, few options are available:

- Intra-gNodeB handover through Xn interface with Regenerative Architecture

- Intra gNodeB handover with CU-DU split with Xn-C or NG handover, with Split regenerative architecture

- Intra gNodeB handover with Xn-C or NG handover, with Transparent Architecture

- See following Figure 7

Figure 7 Several Network Architectures

# Inter-Network Handover

To provide mobility between Terrestrial and Non-Terrestrial networks, the conventional handover would be through 5G Core networks. In the example shown in the following Figure 8 illustrates a typical handover between GEO, LEO and Terrestrial network through three separate core networks.

With the 5G Core implementation this mobility between various 5G Cores is achieved through the N32 Interface. See following Figure 8

Figure 8 Inter-Network Mobility Solution Through Various 5G Core Networks

## Number of Neighbouring Cells

In the conventional TN-NTN handover, the VSAT would see up to 8 satellite neighbouring cells, assuming that the service anchor is the terrestrial network.

In the multi-orbit concept, additional neighbouring cells would be visible to the VSAT and the active neighbouring cells available to the VSAT could be up to 16 cells. These would be a combination of GEO, LEO and Terrestrial cells. Moreover, modern GEO Satellites, to make more efficient use of the radio resources, are designed to provide smaller GEO cells on the ground; therefore, the VSAT would see more denser concentration of GEO Cells in its surrounding. See following Figure 9:

Figure 9 Larger Number of Neighbouring Cells Around the VSAT

To measure larger number of neighbouring cells, longer measurement time would be required in the VSAT.

## New System Information Block (SIB)

For the REl17 and Rel18 mobility and handover solution a new SIB 19 was devised where it provides the satellite network information to the UE. Furthermore, SIB25 was developed to provide coverage information to assist neighbour cell measurements for the UEs in a satellite cell.

For the multi-orbit solution, where the service anchor is the satellite network, a new type of SIB would be required between the GEO and LEO networks to exchange system information of both networks. This new SIB would also signal the neighbouring terrestrial network’s information to the VSAT. See following Figure 10.

Figure 10 New NTN-NTN SIB for Multi-Orbit Solution

# Areas of Further Study

For the realisation of the multi-orbit the following areas would require further analysis

- Larger number of neighbouring cell list

- Extended Measurement time to detect larger number of neighbouring cells.

- Signal strength-based handover between GEO and LEO cells.

- Location based handover between GEO and LEO cells.

- New SIB for NTN-NTN handover information

- Simultaneous LEO and GEO connectivity

- Signalling enhancements to minimize handover interruption time and enable RACH-less handover

- Othe non-RAN related issues such as Lawful Intercept in various Regulatory Regions

Explore security related dependencies with SA3 and other working groups

- Matching capacities and service SLAs when many VSATs/UEs handover to a higher orbit due to NGSO gaps or failures

# Conclusion

From our analyses, multi-orbit solution would be an extension of the cell selection and cell re-selection procedures.

To date, mobility and handover solution have been provided between TN and NTN networks, and the solution is transferable to the multi-orbit solution. However, to realise the complete Multi-Orbit Solution further work, and analysis would be required in 3GPP.

We like to propose a work item on the overall multi-orbit solution in the 5G Advanced Rel20 phase.