3GPP TSG RAN WG1 #121	R1-2504818

St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0064 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on transmission within a shared COT with multiple channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (NTT DOCOMO, INC.), OPPO, CATT, CICTCI, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the beginning of clause 4.5.3 of TS 37.213, it is described that an initiated channel occupancy using multi-channel access procedure can be provided from the initiating UE to another UE, to perform transmission on a single channel from the shared set of channels, as below: --- When a UE initiates a channel occupancy using the channel access procedures described in clause 4.5.1 or clause 4.5.6.3 on a channel(s) to transmit SL transmission(s) including PSCCH/PSSCH(s), the UE can provide a channel occupancy sharing information in SL control information … --- On the other hand, the shared UE’s behavior is described only when single channel access procedure is applied to initiate the channel occupancy, as below. --- If a UE shares a channel occupancy initiated by another UE using the channel access procedures described in clause 4.5.1 on a channel to transmit SL transmission(s), the UE may transmit a SL transmission that follows the SL transmission by the UE that has initiated the channel occupancy after a transmission gap as follows: --- Shared UE’s behavior for the case where a multi-channel occupancy initiated by using multi-channel access procedures as described in clause 4.5.6.3 is shared is missing and thus spec update is necessary to cover this case for U2U COT sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A clause indication is added for the case with multi-channel access to the paragraph. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When a multi-channel occupancy is shared, the COT sharing UE’s behavior is missing in clause 4.5.3 and thus the channel occupany cannot be used by any other UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.5.3	SL channel access procedures in a shared channel occupancy

When a UE initiates a channel occupancy using the channel access procedures described in clause 4.5.1 or clause 4.5.6.3 on a channel(s) to transmit SL transmission(s) including PSCCH/PSSCH(s), the UE can provide a channel occupancy sharing information in SL control information that includes at least the Layer 1 source and destination IDs, the corresponding channel access priority class, the remaining channel occupancy duration, and the frequency domain information for the applicable RB set(s) of the channel occupancy. The channel occupancy sharing information can also include additional IDs and associated cast type. The additional IDs includes one pair of Layer 1 source and destination IDs for all cast types, where the source ID is set to the source ID of the UE initiating channel occupancy for unicast and to the reserved bits for groupcast and broadcast. The channel occupancy sharing information transmitted in slot n indicates the remaining channel occupancy duration in a number of physical slot(s) K. If  K=0, the initiated channel occupancy by the UE shall not be shared for SL transmission(s) by other UE(s). Otherwise, the initiated channel occupancy by the UE can be shared for SL transmission(s) by other UE(s) within a duration starting Tproc,0SL from the end of slot n and ending at slot n+K.

<Unchanged parts omitted>

If a UE shares a channel occupancy initiated by another UE using the channel access procedures described in clause 4.5.1 or clause 4.5.6.3 on a channel to transmit SL transmission(s), the UE may transmit a SL transmission that follows the SL transmission by the UE that has initiated the channel occupancy after a transmission gap as follows:

-	If the transmission gap is at least 25μs, the UE can transmit the SL transmission on the channel after performing Type 2A channel access procedures as described in clause 4.5.2.1.

-	If the transmission gap is 16μs, the UE can transmit the SL transmission on the channel after performing Type 2B channel access procedures as described in clause 4.5.2.2.

-	If the transmission gap is up to 16μs, the UE can transmit the SL transmission on the channel after performing Type 2C channel access as described in clause 4.5.2.3.

<Unchanged parts omitted>